This document is classified as Clear in accordance with the Panel Information Policy. Recipients can distribute this information to the world, there is no limit on disclosure. Information may be shared without restriction subject to copyright.

DP279 ‘June 2025 SEC Release & DUIS Housekeeping Changes’

Annex A

Legal text – version 1.0

About this document

This document contains the redlined changes to the SEC that would be required to deliver this Modification Proposal.

SEC Appendix A – Definitions and Interpretations

These changes have been redlined against SEC Appendix A version 46.0.

## Amend Section A as follows:

SEC Appendix D – SMKI Registration Authority Policies and Procedures

These changes have been redlined against SEC Appendix D version 9.0.

## Amend Section 4.1.4 as follows:

4.1.4 	Director or Company Secretary ceasing to be eligible to act on behalf of a Party, RDP,

Manufacturer or SECCo

Where Director or Company Secretary ceases to be eligible to act on behalf of a Party, RDP, Manufacturer or SECCo in respect of the procedures set out in the SMKI RAPP:

a) 	the Director or Company Secretary themselves, or a Senior Responsible Officer (SRO) whose

identity has previously been verified by the DCC, shall advise the Service Desk of the change;

b) 	the DCC shall confirm such information from the relevant Nominee Details Form, in order to

provide confidence that the request is from a Director, Company Secretary or SRO; and

c) 	if b) is successful, the DCC shall update the DCC’s records of authorised individuals for the

Party, RDP, or SECCo and shall no longer consider the individual to be able to act on behalf

of that Party, RDP or SECCo.

SEC Section D – Modification Process

These changes have been redlined against SEC Section D version 11.0.

## Amend Section D2.6 as follows:

D2.6 	Unless it is an Authority-initiated Modification, a Modification Proposal that proposes variations to this Code that satisfy one or more of the following criteria shall have the status of an Authority Determined Modification:

- the variations are likely to have a material effect on existing or future Energy Consumers;

- the variations are likely to have a material effect on competition in the Supply of Energy or Commercial Activities connected with the Supply of Energy;

- the variations are likely to have a material effect on the environment, on access to or privacy of Data, on security of the Supply of Energy, and/or on the security of Systems and/or Smart Metering Systems;

- the variations are likely to have a material effect on the arrangements set out in Section C (Governance) or this Section D; and/or

(e) 	the variations are likely to unduly discriminate in their effects between one Party (or class of Parties) and another Party (or class of Parties).

## Amend Section D2.7 as follows:

Self-Governance Modification

D2.7 	A Modification Proposal that is not an Authority-initiated Modification, an Authority Determined Modification or a Fast Track Modification shall have the status of a Self-Governance Modification.

SEC Appendix R – Common Test Scenarios Document

These changes have been redlined against SEC Appendix R version 6.0.

## Amend Section 8.1.11 as follows:

8.1.11. Other User (OU) Role

SEC Appendix Q – IKI Certificate Policy

These changes have been redlined against SEC Appendix Q version 8.0.

## Amend Section 5.3.1 (B) as follows:

5.3.1 Qualification, Experience and Clearance Requirements

(A) The ICA shall ensure that all ICA Personnel must:

(i) be appointed to their roles in writing;

(ii) be bound by contract to the terms and conditions relevant to their roles;

(iii) have received appropriate training with respect to their duties;

(iv) be bound by contract not to disclose any confidential, sensitive, personal or security-related Data except to the extent necessary for the performance of their duties or for the purposes of complying with any requirement of law; and

(v) in so far as can reasonably be ascertained by the ICA, not have been previously relieved of any past assignment (whether for the ICA or any other person) on the grounds of negligence or any other failure to perform a duty.

(B) The DCC shall ensure that all ICA Personnel have, as a minimum, passed a Security Check to BS7858 standard, or an equivalent standard which updates or replaces it, before commencing their roles in line with SEC Section G 4.6

Appendix AD ‘DCC User Interface Specification’

These changes have been redlined against Appendix AD version 5.3.

## Amend the following rows in the table in Section 3.1 as follows:

| Service Request Name | Service Reference | Service Reference Variant | Critical | Modes of Operation |  |  |  | Eligible User Roles | Available in relation to SMETS1 Devices |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | On Demand | Future Dated Response Pattern | DCC Scheduled | Non-Device Request |  |  |
| Read Export Profile Data | 4.8 | 4.8.3 | No | Yes | DSP | Yes | No | ES ED MDR OU | Yes |
| Communications Hub Status Update. – Fault Return | 8.14 | 8.14.3 | No | No | No | No | Yes | IS GS RSA | No |
| Communications Hub Status Update – No Fault Return | 8.14 | 8.14.4 | No | No | No | No | Yes | IS GS RSA | No |

Table 18 : Service Request Matrix

## Amend Section 3.8.43.2 as follows:

      - 3.8.43.2	Specific Data Items for this Request

CreateSchedule Definition

| Data Item | Description / Valid Set | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ScheduleFrequency | The frequency of which the required service reference is executed Valid set - Daily - Weekly (The specified Service Request will be scheduled once a week, on the Schedule Start Date day of the week.) - Monthly (The specified Service Request will be scheduled once a month, on the Schedule Start Date day of the month, where possible. For those months where the Schedule Start Date day of the month doesn’t exist, the Service Request will be scheduled on the last day of that month.) - Quarterly (The specified Service Request will be scheduled once every three months, with Scheduled Start Date as for Monthly.) - Half-Yearly (The specified Service Request will be scheduled once every six months, with Scheduled Start Date as for Monthly.) - Yearly (The specified Service Request will be scheduled once every 12 months, with Scheduled Start Date as for Monthly.) | sr:ScheduleFrequency (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| ScheduleStartDate | The UTC date that the scheduled repeating request is required to commence from - Valid date in the future | xs:date | Yes | None | UTC Date |
| ScheduleEndDate | The UTC date that the scheduled repeating request is required to cease, or if not present then the repeating schedule shall remain in force until deleted by the User or by the DCC Systems, e.g. because of Device Decommission - Valid date in the future >= ScheduleStartDate | xs:date | User Role IS, GS, ES, ED, GT, MDR: No User Role OU: Yes | None | UTC Date |
| ScheduleExecutionStartTime | The UTC start time after which a scheduled Command (invoked by the schedule) may be run - Valid Time | xs:time | No | 00:01:00 | UTC Time |
| KAPublicSecurityCredential | The Key Agreement Public Security Credentials, associated with the User submitting the Request that will be relied upon for Sensitive data Responses. Only applicable to those Scheduled Service Requests that can be submitted by User Roles for which the Device doesn’t hold Security Credentials. | sr:Certificate (xs:base64Binary) | User Role IS, ES, ED: N/A User Role GT (where Response includes sensitive data and Device Type = Gas Smart Meter): Yes User Role MDR, OU (where Response includes sensitive data): Yes | None | N/A |
| DSPScheduledServiceReference | Reference of the Service Request to be Scheduled. Valid Set: see clause 3.1 , where DCC Scheduled column in table is set to “Yes” | sr:DSPScheduledServiceReference (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| DSPScheduledServiceReferenceVariant | Reference Variant of the Service Request to be Scheduled. Valid Set: see clause 3.1 , where DCC Scheduled column in table is set to “Yes” | sr:DSPScheduledServiceReferenceVariant (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| DeviceID | This is the Device ID to which the DCC Schedule is targeted. | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| Choice of Service Request XML Element to be Scheduled | Name and Request Data Items corresponding to DSPScheduledServiceReferenceVariant to be Scheduled, choice of: |  | Yes | None | N/A |
|  | DSPRetrieveImportDailyReadLog DSPRetrieveExportDailyReadLog DSPReadActiveImportProfileData DSPReadReactiveImportProfileData DSPReadExportProfileData DSPReadNetworkData DSPReadPrepaymentDailyReadLog DSPRetrieveDailyConsumptionLog | sr:ReadLogPeriodOffset |  |  |  |
|  | DSPReadInstantaneousExportRegisters DSPReadMaximumDemandImportRegisters DSPReadMaximumDemandExportRegisters DSPReadLoadLimitData DSPReadActivePowerImport | sr:DSPReadData |  |  |  |
|  | DSPRecordNetworkDataGAS | sr:RecordNetworkDataGAS |  |  |  |

Table 157 : CreateSchedule (sr:DSPSchedule) data items

## Amend Section 1.4.7.14 as follows:

#### 1.4.7.14Activate Firmware SRV 11.3

This clause is a supplement to clause 3.8.125.

SRV 11.3 shall be supported for SMETS1 CHFs, SMETS1 PPMIDs as well as SMETS1 Smart Meters.

In clause 3.8.125.4, where the target Device type is SMETS1 CHF, then the additional DCC processing related to Firmware Version and Device Status updates shall also apply to the associated GPF.

Where the Service Request relates to a SMETS1 CHF or SMETS1 PPMID and the Business Originator ID does not relate to the Responsible Supplier for the SMETS1 ESME on the same home area network then the DCC shall return a Response containing an error code of E4 indicating failure.

Where the DCC produces a SMETS1 Response indicating successful activation of SMETS1 CHF or SMETS1 PPMID firmware, if there is, according to the Smart Metering Inventory,  a GSME on the same home area network as that Device then the DCC shall issue a DCC Alert with code N57 to inform the Responsible Gas Supplier of the activation.

## Amend Section 3.8.27.2 as follows:

      - 3.8.27.2		Specific Data Items for this Request

The data items applicable depend on whether the Service Request is executed on an On Demand or DCC Scheduled basis.

For execution of this Service Request as an On Demand Service, the RetrieveImportDailyReadLog XML element defines this Service Request.

RetrieveImportDailyReadLog (On Demand) Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ExecutionDateTime | A User shall only add this Data Item to the Service Request where they require the Service Request to be executed at a future date and time. The UTC date and time the User requires the command to be executed on the Device Date-time in the future that is either <= current date + 30 days or the date = ‘3000-12-31T00:00:00Z’ | xs:dateTime | No | None | UTC Date-Time |
| ReadLogPeriod | The Start and End Date-Times for which the data is required | sr:ReadLogPeriod (see clause 3.10.1.14) | Yes | None | N/A |
| KAPublicSecurityCredentials | The Key Agreement Public Security Credentials (of the requesting party) to be used where the request is from an Unknown Remote Party (e.g. Old Responsible Supplier) with respect to the BusinessTargetID specified within the Service Request. | sr:Certificate (xs:base64Binary) | Responsible Supplier: N/A Old Responsible Supplier: Yes MDR: Yes Mandatory for User Roles IS/GS that do not have their credentials on the Device when sending the Service Request e.g. Old Suppliers | None | N/A |

Table 136 : RetrieveImportDailyReadLog (sr:ReadLogFutureDatableAndURPCredentials) data items

## Amend Section 3.8.111.3 to the following:

#### 3.8.111.3 		Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for ‘Other Device’ Existence validation.

| Response Code | Response Code Description |
| --- | --- |
| E080801 | This Response Code is not used. The DCC Systems Smart Metering Inventory is no longer checked for the join status of the ‘Other Device’ |

#### 3.8.112.3 		Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for ‘Other Device’ Existence validation.

| Response Code | Response Code Description |
| --- | --- |
| E080801 | This Response Code is not used. The DCC Systems Smart Metering Inventory is no longer checked for the join status of the ‘Other Device’ |
| E080821 | The User Role is not authorised to Unjoin this Device Type |
| E080822 | The User is not authorised to Unjoin these Gas Smart Meter / Gas Proxy Function |

#### 3.8.123.3 	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’

| Response Code | Response Code Description |
| --- | --- |
| E110101 | The Firmware Version is not approved. The Firmware version ID does not align with an entry on the Central Products List. |
| W110101 | The Update Firmware Warning contains between one and five lists of Device IDs for which the validation failed for each DeviceID: - InvalidDeviceIDList . The DeviceID does not exist or the User is not the Meter or HCALCS Import Supplier - NotCommissionedDeviceIDList. The Meter status is not ‘Commissioned’ or ‘Suspended’ or the Device is not a Meter or a HCALCS - NotApplicableFirmwareDeviceIDList. The Firmware Version is not applicable to the Device - OtherRequestsInProgressList. The Device already has another Firmware update request in progress.InvalidGBCSVersionList. The CHF associated with the HCALCS Device has a GBCS Version prior to v4.1,or the HCALCS Device currently has a GBCS Version that is prior to v4.1. |
| E110102 | The firmware is not marked as active in the Central Products List and Smart Metering Inventory. |
| E110103 | The DCC Systems calculated Hash value over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request differs from that held in the CPL for the specified FirmwareVersion. |
| E110105 | The firmware image is not constructed as per the GBCS definition, i.e. the FirmwareImage does not contain both an OTA Header and a Firmware Image concatenated together , or where the Target Device is a SMETS2+ Device, the Length of the base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image” exceeds 1024000. |

      - 3.8.126.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’

| Response Code | Response Code Description |
| --- | --- |
| E110401 | The Firmware Version is not approved. The Firmware version ID does not align with an entry on the Central Products List. |
| W110401 | The Update Firmware Warning contains between one and five lists of Device IDs for which the validation failed for each DeviceID: - InvalidDeviceIDList . The DeviceID does not exist or the User is not the Import Supplier or Gas Supplier for a Meter connected to the same HAN as the PPMID. - NotCommissionedDeviceIDList. The PPMID status is not ‘Commissioned’ or ‘Suspended’ or the Device is not a PPMID - NotApplicableFirmwareDeviceIDList. The Firmware Version is not applicable to the PPMID - OtherRequestsInProgressList. The PPMID already has another Firmware update request in progress. - InvalidGBCSVersionList. The CHF associated with the PPMID Device has a GBCS Version prior to v4.1. |
| E110402 | The firmware is not marked as active in the Central Products List and Smart Metering Inventory. |
| E110403 | The DCC Systems calculated Hash value over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request differs from that held in the CPL for the specified FirmwareVersion. |
| E110405 | The firmware image is not constructed as per the GBCS definition, i.e. the FirmwareImage does not contain both an OTA Header and a Firmware Image concatenated together, or the Length of the base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image” exceeds 1024000. |

## Amend the following Sections:

      - 1.4.11.2Countersigned SMETS1 Response and Alert Format

The diagram below illustrates the structure of a Countersigned SMETS1 Response or Countersigned SMETS1 Alert.

Figure 1 : Overall structure of a Countersigned SMETS1 Response or Alert

The SMETS1 Response Message shall contain the following Common Objects as further defined with the DUIS XML Schema:

| Data Item | Description | Type | Mandatory | Valid Values |
| --- | --- | --- | --- | --- |
| ServiceReference | Identifier that signals the particular Service Reference to DCC (and is driven from the User’s selection of Request) | sr:ServiceReference (See 3.10.1.5) | Yes | As per the Request |
| ServiceReferenceVariant | Identifier that signals the particular Service Reference Variant to DCC (and is driven from the User’s selection of Request) | sr:ServiceReferenceVariant (See 3.10.1.6) | Yes | As per the Request |
| DSPScheduleID | Schedule ID generated by the DCC Systems Valid Set: >= 0 and <= 1000000000000 | sr:scheduleID (Restriction of xs:nonNegativeInteger) | Present for DCC Scheduled requests | See description |
| ThrottledAlertSequenceId | An optional data item that identifies that this Alert Code is currently subject to throttling by the DCC Data Systems. If this attribute is included in the Alert then it indicates the sequence number for this Alert message since Alert throttling began. | xs:unsignedInt | No | As per Table 43 |
| ThrottledAlertCount | An optional data item used to indicate the number of Alerts that have been consolidated by DCC Data Systems since the last Alert was forwarded to the User. | xs:unsignedInt | No | As per Table 43 |
| SMETS1SignedResponse | Message created and signed by the S1SP. It contains a SMETS1 Response or a SMETS1 Alert | sr:SMETS1SignedResponse (see clause 1.4.11.3) | Yes | See description |

Table 4 : SMETS1ResponseMessage Format

      - 3.6.2.2	Device Alert Body Format

The DCC shall, where a Device raises an Alert, construct a message to the User conforming to the devicealert message format. This message combines the GBCSPayload received from the Device with the Alert Code extracted from the GBCSPayload.

| Data Item | Description | Type | Mandatory | Valid Values |
| --- | --- | --- | --- | --- |
| AlertCode | Code indicating the alert or reason for the alert to be generated GBCS includes ‘0x’ at the start of such codes. This definition uses a hexBinary representation for valid values. | xs:hexBinary | Yes | See GB Companion Specification for base list and apply hexBinary representation of these GBCS defined values |
| ThrottledAlertSequenceId | An optional data item that identifies that this Alert Code is currently subject to throttling by the DCC Data Systems. If this attribute is included in the Alert then it indicates the sequence number for this Alert message since Alert throttling began. | xs:unsignedInt | No | As per Table 43 |
| ThrottledAlertCount | An optional data item used to indicate the number of Alerts that have been consolidated by DCC Data Systems since the last Alert was forwarded to the User. | xs:unsignedInt | No | As per Table 43 |
| GBCS Payload | See GB Companion Specification for Details of the format of the GBCS Alert | xs:base64Binary | Yes | See GB Companion Specification for message construction. |

Table 38 : Device Alert Body

      - 3.6.3.2	DCC Alert Body Format

The DCC shall, where the DCC Systems raises a DCC Alert, construct a message to the User with the Body element conforming to the DCCAlertMessage format. This message is generated by the DCC Systems as a result of a trigger event.

| Data Item | Description | Type | Mandatory | Valid Values |
| --- | --- | --- | --- | --- |
| DCCAlertCode | Code indicating the alert or reason for the Alert to be generated by DCC | Restriction of xs:string (Enumeration) | Yes | See clause 3.6.3.4 |
| DCCAlert | This is body specific content dependent on the DCCAlertCode being sent. See clause 3.9 for body specific format. | sr:DCCAlert See clause 3.9 | Yes | See clause 3.9 |
| ThrottledAlertSeqeunceID | An optional data item that identifies that this Alert Code is currently subject to throttling by the DCC Data Systems. If this attribute is included in the Alert then it indicates the sequence number for this Alert message since Alert throttling began. | xs:unsignedInt | No | As per Table 43 |
| ThrottledAlertCount | An optional data item used to indicate the number of Alerts that have been consolidated by DCC Data Systems since the last Alert was forwarded to the User. | xs:unsignedInt | No | As per Table 43 |

Table 40 : DCC Alert Body

      - 3.8.124.4		Additional DCC System Processing

Upon receipt of a Response to this Service Request containing a Firmware Version value:

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model, but is different from that stored in the SMI, the DCC Systems shall update the Firmware Version in the SMI to the value returned by the Device. Note that updating the Firmware Version may also update the Device’s GBCS Version in the SMI.

  - If the target Device is CHF, the associated GPF Firmware Version shall also be updated.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Current” and the Read Firmware Version Service Request was not submitted by the Responsible Supplier, DCC Alert N49 shall be sent to the Responsible Supplier.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Removed”, the SMI Firmware Version shall be updated, and the Device Status shall  be set to ‘Suspended’ (where the target Device is CHF, the associated GPF Device Status shall also be set to ‘Suspended’). In this case DCC Alert N50 shall be sent to the Responsible Supplier . In addition, DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter to notify the “Suspension” of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alerts N40 and N41 will be sent to the Users responsible for those Service Requests.

  - Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current”, the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device does not match an entry on the  CPL for that Device Model, but is different from that stored in the SMI, DCC Alert N51 will be sent to the Responsible Supplier  and the SMI Firmware Version will  be updated with the Firmware Version returned by the Device. The Device’s GBCS Version in the SMI will not be updated since there is no CPL entry on which to base an update.

  - As there is no entry on the CPL, the Device Status of the Device shall be set to ‘Suspended’ and an additional DCC Alert N28 shall be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

  - If the target Device is CHF, the associated GPF Firmware Version shall also be updated and the GPF Device Status shall also be set to ‘Suspended’.

  - All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alerts N40 and N41 will be sent to the Users responsible for those Service Requests.

if the Target Device Type is GPF and the GSME Firmware Version returned by the GPF is different from that stored in the SMI, DCC Alert N52 will be sent to the Responsible Supplier as a warning and the SMI Firmware Version will not be updated.

      - 3.8.125.4	Additional DCC System Processing

The DCC Systems shall monitor all Responses received to this Service Request.

Where the DCC identifies any Response where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent to the Responsible Supplier and to the  Electricity Distributor or Gas Transporter.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter to notify the “Suspension” of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alert N40 and N41 will be sent to the Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model) and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, DCC Alert N51 will be sent to the Responsible Supplier  and the Smart Metering Inventory Firmware Version will  be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Users responsible for those Service Requests.

      - 3.8.126.5	Additional DCC System Processing

The DCC Systems shall calculate a Hash over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request and ensure that the Hash calculated matches that held in the CPL for the specified FirmwareVersion. Where the DCC identifies any mismatch in Hash values, a Response Code of E110403 shall be returned to the User.

The DCC Systems shall monitor all Device Alerts received from the PPMID indicating the activation outcome (Alert Code 0x8F8B) received as a consequence of sending this Service Request.

Where the DCC identifies any Device Alert where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent only to the Responsible Supplier.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent only to the Responsible Supplier to notify the “Suspension” of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model), and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device,  DCC Alert N51 will be sent to the Responsible Supplier  and the Smart Metering Inventory Firmware Version will be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent only to the Responsible Supplier. All DSP Future Dated Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N41 will be sent to the Users responsible for those Service Requests.

      - 3.9.15.3S1SP Alert Codes in DCC Alerts

This section specifies the meaning associated with the SMETS1 Service Provider codes which can be sent to DCC Users in DCC Alerts which are Countersigned S1SPAlerts.

S1SP Alerts may contain an S1SP Alert Code as defined here or a Response Code as defined in 3.5.10.

An S1SP Alert Code consists of a four letter prefix followed by a number. S1SP Alert Code types are:

- SMETS1 UTRN. Prefix ‘S1UT’;

- SMETS1 Validation Error. Prefix ‘S1VE’;

- SMETS1 Communication Error. Prefix ‘S1CE’; or

- other condition also indicated by prefix ‘S1’

The DCC shall maintain and publish to all Users the list of S1SP Alert descriptions and, for each such description the associated S1SP Alert Code.

The DCC shall publish such information regarding an S1SP Alert at least [10] working days before any such S1SP Alert is created in relation to a SMETS1 Service Request operated through the DCC.

## Amend the below Section to the following:

      - 3.8.124.4		Additional DCC System Processing

Upon receipt of a Response to this Service Request containing a Firmware Version value:

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model, but is different from that stored in the SMI, the DCC Systems shall update the Firmware Version in the SMI to the value returned by the Device. Note that updating the Firmware Version may also update the Device’s GBCS Version in the SMI.

  - If the target Device is CHF, the associated GPF Firmware Version shall also be updated.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Current” and the Read Firmware Version Service Request was not submitted by the Responsible Supplier, DCC Alert N49 shall be sent to the Responsible Supplier.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Removed”, the SMI Firmware Version shall be updated, and the Device Status shall  be set to ‘Suspended’ (where the target Device is CHF, the associated GPF Device Status shall also be set to ‘Suspended’). In this case DCC Alert N50 shall be sent to the Responsible Supplier . In addition, DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter to notify the suspension of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

  - Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current”, the DCC Systems shall update the Device Status to the status it held immediately prior to its suspension and DCC Alert N29 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device does not match an entry on the  CPL for that Device Model, but is different from that stored in the SMI, DCC Alert N51 will be sent to the Responsible Supplier  and the SMI Firmware Version will  be updated with the Firmware Version returned by the Device. The Device’s GBCS Version in the SMI will not be updated since there is no CPL entry on which to base an update.

  - As there is no entry on the CPL, the Device Status of the Device shall be set to ‘Suspended’ and an additional DCC Alert N28 shall be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

  - If the target Device is CHF, the associated GPF Firmware Version shall also be updated and the GPF Device Status shall also be set to ‘Suspended’.

  - All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

if the Target Device Type is GPF and the GSME Firmware Version returned by the GPF is different from that stored in the SMI, DCC Alert N52 will be sent to the Responsible Supplier as a warning and the SMI Firmware Version will not be updated.

      - 3.8.125.4	Additional DCC System Processing

The DCC Systems shall monitor all Responses received to this Service Request.

Where the DCC identifies any Response where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its suspension and DCC Alert N29 will be sent to the Responsible Supplier and to the  Electricity Distributor or Gas Transporter.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter to notify the suspension of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alert N40 and N41 will be sent to the Service Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model) and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, DCC Alert N51 will be sent to the Responsible Supplier  and the Smart Metering Inventory Firmware Version will  be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

      - 3.8.126.5	Additional DCC System Processing

The DCC Systems shall calculate a Hash over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request and ensure that the Hash calculated matches that held in the CPL for the specified FirmwareVersion. Where the DCC identifies any mismatch in Hash values, a Response Code of E110403 shall be returned to the User.

The DCC Systems shall monitor all Device Alerts received from the PPMID indicating the activation outcome (Alert Code 0x8F8B) received as a consequence of sending this Service Request.

Where the DCC identifies any Device Alert where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its suspension and DCC Alert N29 will be sent only to the Responsible Supplier.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent only to the Responsible Supplier to notify the suspension of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model), and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device,  DCC Alert N51 will be sent to the Responsible Supplier  and the Smart Metering Inventory Firmware Version will be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent only to the Responsible Supplier. All DSP Future Dated Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N41 will be sent to the Service Users responsible for those Service Requests.

## Amend the below Section to the following:

      - 3.8.123.4		Specific Data Items in the Response

This Service Request synchronous response is defined in the XSD DSPUpdateFirmwareWarning XML element, which contains the list(s) of Device IDs that failed DCC Authorisation / Validation.

The DCC shall return the following data items which are specific to this Service Request for Successful Responses

DSPUpdateFirmwareWarning Definition:

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| InvalidDeviceIDList | Comma separated list of Device IDs for which the User ID is not the Import Supplier or the DeviceIds don’t exist. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotCommissionedDeviceID List | Comma separated list of Device IDs which aren’t in a status of “Commissioned” or ‘Suspended’ or the Device is not a Meter or a HCALCS. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotApplicableFirmware DeviceIDList | Comma separated list of Device IDs for which the Firmware is not applicable. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| OtherRequestsInProgress List | Comma separated list of Device IDs for which another Firmware Update request is in progress. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| InvalidGBCSVersionList | Comma separated list of Device IDs for which either; - the HCALCS DeviceID for which the GBCS Version of the associated CHF is prior to v4.1 - HCALCS Device IDs for which the current GBCS Version is prior to v4.1 | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |

Table 255 : DSPUpdateFirmwareWarning (sr:DSPUpdateFirmwareWarning) data items

      - 3.8.126.4	Specific Data Items in the Response

This Service Request synchronous response is defined in the XSD DSPUpdateFirmwareWarning XML element, which contains the list(s) of Device IDs that failed DCC Authorisation / Validation.

The DCC shall return the following data items which are specific to this Service Request for Successful Responses

DSPUpdatePPMIDFirmwareWarning Definition:

| Data Item | Description / Allowable values |  | Type |  | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
| InvalidDeviceIDList | Comma separated list of Device IDs for which the User ID is not the Import Supplier or Gas Supplier or the DeviceIds don’t exist. Containing a minimum of 1 Device ID, each as defined by sr:EUI |  | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”)) |  | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotCommissionedDevice IDList | Comma separated list of Device IDs which aren’t in a status of “Commissioned” or ‘Suspended’ or the Device is not a PPMID. Containing a minimum of 1 Device ID, each as defined by sr:EUI |  | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) |  | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotApplicableFirmware DeviceIDList | Comma separated list of Device IDs for which the Firmware is not applicable. Containing a minimum of 1 Device ID, each as defined by sr:EUI |  | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) |  | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| OtherRequestInProgressList | Comma separated list of Device IDs for which another Firmware Update request is in progress. Containing a minimum of 1 Device ID, each as defined by sr:EUI |  | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) |  | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| InvalidGBCSVersionList |  | Comma separated list of Device IDs for which the CHF associated with PPMID device has a GBCS version prior to v4.1 |  | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |

Table 257.2 : DSPUpdatePPMIDFirmwareWarning (sr:DSPUpdatePPMIDFirmwareWarning) data items

## Amend Section 1.4.7.13 to the following:

      - 1.4.7.13Update Firmware SRV 11.1

This clause is a supplement to clause 3.8.123.

SRV 11.1 shall be supported for SMETS1 CHFs and SMETS1 PPMIDs as well as Smart Meters, and the processing in clause 3.8.123.5 shall apply.

Following the receipt by DCC of a successfully authenticated Update Firmware Service Request to distribute firmware to a SMETS1 CHF or SMETS1 PPMID, if there is, according to the Smart Metering Inventory, a GSME on the same home area network as that Device  then the DCC shall issue DCC Alert with code N57 to inform the Gas Supplier of the Service Request.

## Amend Table 18 Service Request Matrix to the following:

The following table (the “Service Request Matrix”) sets out these attributes (i.e. the ones referred to above) for each Service Request. The description of Eligible User Roles is only a summary, as further qualifications concerning Eligible User Roles are set out in the DCC User Interface Services Schedule.

| Service Request Name | Service Reference | Service Reference Variant | Critical | Modes of Operation |  |  |  | Eligible User Roles | Available in relation to SMETS1 Devices |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | On Demand | Future Dated Response Pattern | DCC Scheduled | Non-Device Request |  |  |
| Read Device Configuration (Event and Alert Behaviours) | 6.2 | 6.2.10 | No | Yes | No | No | No | IS GS ED | No |
| Set CHF Sub GHz Configuration | 6.28 | 6.28 | No | Yes | No | No | No | IS GS | No |
| Request CHF Sub GHz Channel Scan | 6.29 | 6.29 | No | Yes | No | No | No | IS GS | No |
| Read CHF Sub GHz Configuration | 6.30 | 6.30 | No | Yes | No | No | No | IS GS RSA | No |
| Read CHF Sub GHz Channel | 6.31 | 6.31 | No | Yes | No | No | No | IS GS RSA | No |
| Read CHF Sub GHz Channel Log | 6.32 | 6.32 | No | Yes | No | No | No | IS GS RSA | No |
| Read Inventory (Current and Future Suppliers may use this Service Request) | 8.2 | 8.2 | No | No | No | No | Yes | IS ES GS RSA ED GT MDR OU | Yes |

Table 18 : Service Request Matrix

## Amend Section 3.6.3.4 Table 41 N51 to the following:

3.6.3.4 	DCC Alert Codes

| DCC Alert Code | Alert Name | Event | Trigger | DCC Alert Recipient | Device Applicability |
| --- | --- | --- | --- | --- | --- |
| N51 | Invalid Firmware Version | Device’s Firmware Version is unknown (not in the CPL) Device’s Firmware Version updated in the Smart Metering Inventory and Device Status set to ‘Suspended’ | Upon successful completion of Service Request 11.2 Read Firmware Version where the target Device is an ESME, GSME, HCALCS, PPMID or, CHF and the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR Upon completion of Service Request 11.3 Activate Firmware where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR Future Dated Firmware Activation Alert (Alert Code 0x8F66 or 0x8F67 and Message Code 0x00CA) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR PPMID Firmware Activation Alert (Alert Code 0x8F8B) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL | IS GS | All |

Table 41 : DCC Alert Codes

## Amend Section 3.8.111.3 to the following:

#### 3.8.111.3	 Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for ‘Other Device’ Existence validation.

| Response Code | Response Code Description |
| --- | --- |
| E080801 | This Response Code is not used. The DCC Systems Smart Meter Inventory is no longer checked for the join status of the ‘Other Device’ |

## Amend Section 3.8.112.3 to the following:

#### 3.8.112.3 	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for ‘Other Device’ Existence validation.

| Response Code | Response Code Description |
| --- | --- |
| E080801 | This Response Code is not used. The DCC Systems Smart Meter Inventory is no longer checked for the join status of the ‘Other Device’ |
| E080821 | The User Role is not authorised to Unjoin this Device Type |
| E080822 | The User is not authorised to Unjoin these Gas Smart Meter / Gas Proxy Function |

## Amend Section 3.8.124.3 to the following:

      - 3.8.124.3		Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for Execution Date Time validation.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’.

## Amend Section 3.8.125 as follows:

    - 3.8.125	Activate Firmware

      - 3.8.125.1		Service Description

| Service Request Name | - ActivateFirmware |  |  |
| --- | --- | --- | --- |
| Service Reference | - 11.3 |  |  |
| Service Reference Variant | - 11.3 |  |  |
| Eligible Users | Import Supplier (IS) Gas Supplier (GS) |  |  |
| Security Classification | Critical |  |  |
| BusinessTargetID - Device Type applicable to this request | Electricity Smart Meter (ESME) Gas Smart Meter (GSME) HAN Connected Auxiliary Load Control Switch (HCALCS) |  |  |
| Can be future dated? | Device |  |  |
| On Demand? | Yes |  |  |
| Capable of being DCC Scheduled? | No |  |  |
| Command Variants applicable to this Request (Only one populated | For Service Request 4 – Transform For Signed Pre-Commands, choice of: 5 - Send (Critical) 6 - Return for local delivery (Critical) 7 - Send and Return for local delivery (Critical) |  |  |
| Common Header Data Items | See clause 3.4.1.1 |  |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause3.5 for more details on processing patterns - Acknowledgement - Response to Transform Request - PreCommand Format - Service Response from Device – GBCSPayload - Service Response (from Device) - FutureDatedDeviceAlertMessage - Response to a Command for Local Delivery Request – LocalCommand Format Also see Response Section below for details specific to this request |  |  |
| Response Codes possible from this Service Request | See clause 3.5.10 for Common Response Codes |  |  |
| GBCS Cross Reference | Electricity (ESME) | Gas | Electricity (HCALCS) |
| GBCS version earlier than v4.1 MessageCode | 0x0012 | 0x0012 | N/A |
| GBCS version earlier than v4.1 Use Case | CS06 | CS06 | N/A |
| GBCS v4.1 or later MessageCode | 0x0012 | 0x0012 | 0x0012 |
| GBCS v4.1 or later Use Case | CS06 | CS06 | CS06 |

## Amend Section 3.8.125.4 to the following:

      - 3.8.125.4	Additional DCC System Processing

The DCC Systems shall monitor all Responses received to this Service Request.

Where the DCC identifies any Response where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent to the Responsible Supplier and to the  Electricity Distributor or Gas Transporter.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of  “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter to notify the “Suspension” of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device(s) and DCC Alert N40 and N41 will be sent to the Service Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model) and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, DCC Alert N51 will be sent to the Responsible Supplier  and the Smart Metering Inventory Firmware Version will  be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

## Amend Section 3.8.126.5 to the following:

      - 3.8.126.5	Additional DCC System Processing

The DCC Systems shall calculate a Hash over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request and ensure that the Hash calculated matches that held in the CPL for the specified FirmwareVersion. Where the DCC identifies any mismatch in Hash values, a Response Code of E110403 shall be returned to the User.

The DCC Systems shall monitor all Device Alerts received from the PPMID indicating the activation outcome (Alert Code 0x8F8B) received as a consequence of sending this Service Request.

Where the DCC identifies any Device Alert where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent only to the Responsible Supplier.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, and the Device Status will be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier. In addition, DCC Alert N28 will be sent only to the Responsible Supplier to notify the “Suspension” of the Device. All DSP Future Dated and Scheduled Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N40 and N41 will be sent to the Service Users responsible for those Service Requests.

If the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model), and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device,  DCC Alert N51 will be sent to the Responsible Supplier and the Smart Metering Inventory Firmware Version will be updated with the Firmware Version returned by the Device. As there is no entry on the CPL, the Device Status of the Device will be set to ‘Suspended’ and an additional DCC Alert N28 will be sent only to the Responsible Supplier. All DSP Future Dated Service Requests shall be deleted for the ‘Suspended’ Device and DCC Alerts N41 will be sent to the Service Users responsible for those Service Requests.

## Amend Section 3.8.127.3 to the following:

      - 3.8.127.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Request, the general Authorisation Checks as defined below shall not be carried out.

- a Response Code of E4 as defined in clause 3.2.4 “Verify that the User, in the User Role defined in the Service Request is a Eligible User for the Device” or

- a Response Code of E5 as defined in clause 3.2.4 “Verify that the Service Request or Signed Pre-Command is applicable to the Device status”

## Amend Section 3.8.127.4 to the following

      - 3.8.127.4	Specific Data Items in the Response

This Service Response is defined in the XSD ResponseMessage DSPWANMatrix element, which contains the SM WAN connectivity details.

Request WAN Matrix Service Request Response Data Items

DSPWANMatrix Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| Request | Input details of the Request, i.e. UPRN or Partial Address | sr:RequestWANMatrix | Yes | None | N/A |
| DSPWANMatrixResponse | Array of responses for each CSP Region | Sr:DSPWANMatrixResponse | Yes | None | N/A |

Table 260 : DSPWANMatrix (sr:DSPWANMatrix) data items

## Amend Section 3.8.127.4 Table 260a to the following:

DSPWANMatrixResponse Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| CSPRegion | The CSP Region the address is associated with Valid set: - North - Central - South - 4G Central - 4G South - 4G North | sr:CSPRegion (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| CSPRegionResponseCode | The outcome for a single CSP Region Valid set: • I0 • E120101 • E120102 • E120103 | sr:ResponseCode | Yes | None | N/A |
| CoverageAvailability | Coverage Availability Valid set: - true. (Yes) - false. (No) | xs:boolean | CSPRegionResponseCode = I0: Yes Otherwise: N/A | None | N/A |
| AnticipatedCoverageDate | If Coverage Availability is set to false, the anticipated date when Coverage will be available. If no Coverage is planned then this date will be set to ‘3000-12-31T00:00:00Z’ | xs:date | Coverage Availability = false: Yes Otherwise: N/A | None | N/A |
| WANTechnology | The WAN technology to be used for this location The allowable values shall be in line with Equipment names as defined and maintained within Annex E of the CH INSTALLATION AND MAINTENANCE SUPPORT MATERIALS | sr:WANTechnologyType (Restriction of xs:string (maxLength = 30)) | CSPRegionResponseCode = I0: Yes Otherwise: N/A | None | N/A |
| ConnectivityLikelihood | The likely Connectivity strength Valid set: - High - Medium - Low | Restriction of xs:string (Enumeration) | CSPRegionResponseCode = I0: Yes Otherwise: N/A | None | N/A |
| AuxiliaryEquipmentRequired | Free text with details of any required auxiliary equipment, if any | Restriction of xs:string (maxLength = 50) | No | None | N/A |
| AdditionalInformation | Free text providing additional information | Restriction of xs:string (maxLength = 250) | No | None | N/A |

#### Table 260a : DSPWANMatrixResponse (sr:DSPWANMatrixResponse) data items

## Amend Table 262 to the following:

  - DCC Alert Messages

    - Specific Data Items in the DCC Alert Message

Each Alert Code being reported as a DCC Alert shall conform to the DCC Alert format as defined in 3.6.3 DCC Alerts - DCCAlertMessage Format. The DCC shall ensure that the Body of each DCC Alert (DCCAlert XML element) conforms to one of the DCC Alert formats as defined in the table below:

DCCAlert Definition

| DCC Alert Format / Data Item | Description / Allowable values | Type | Mandatory for Alert Codes | Default | Units |
| --- | --- | --- | --- | --- | --- |
| PowerOutageEvent | The trigger event indicates that a device power has failed | sr:PowerOutageEvent See 3.9.2 | AD1 | None | N/A |
| DeviceStatusChangeEvent | The trigger event indicates that a Device’s SMI Status has changed | sr:DeviceStatusChangeEvent See 3.9.3 | N1, N2, N8, N9, N16, N28 and N29, N44, N45 | None | N/A |
| DSPScheduleRemoval | The trigger event indicates that a DCC Schedule is to be deleted | sr:DSPScheduleRemoval See 3.9.4 | N4, N5, N6, N17, N37 and N40 | None | N/A |
| CommandFailure | The trigger event indicates that a Command has failed | sr:CommandFailure See 3.9.5 | N3, N7, N10, N11, N12, N13, N14, N15, N33, N34, N35, N36, N38, N41 and N53 | None | N/A |
| FirmwareDistributionFailure | The trigger event indicates that a Firmware Distribution Command to the CSP has failed, at least for some of the Devices | sr:FirmwareDistributionFailure See 3.9.6 | N18, N19, N20, N21, N22 and N23 | None | N/A |
| UpdateHANDeviceLogResult | The trigger event indicates if a Command to Update a Communications Hub Whitelist Update. (addition ONLY) has succeeded or no Alert has been received by the DCC. | sr:UpdateHANDeviceLogResult See 3.9.7 | N24 and N25 | None | N/A |
| ChangeOfSupplier | The trigger event indicates if an Update Security Credentials (CoS) has succeeded or has failed the CoS Party access control | sr:ChangeOfSupplier See 3.9.8 | N26 and N27 | None | N/A |
| DeviceLogRestored | The trigger event indicates that the CHF or GPF Device Log has been restored | sr:DeviceLogRestored (See clause 3.9.9) | N30, N31 | None | N/A |
| PPMIDAlert | The trigger event indicates an Alert has been generated by the PPMID Device | sr:PPMIDAlert (See clause 3.9.10) | N39 | None | N/A |
| SecurityCredentialsUpdated | The trigger event indicates receipt of a success Response from Update Security Credentials where the Remote Party whose certificate has been placed on the Device is not the sender of the Service Request | sr:SecurityCredentialsUpdated (see clause 3.9.11) | N42 | None | N/A |
| PPMID Removal | The trigger event is receipt of a successful Response from Update HAN Device Log (Removal) where the removed Device is a PPMID that was joined to both an ESME and the GSME | sr:PPMIDRemoval (See clause 3.9.12) | N43 | None | N/A |
| QuarantinedRequest | The trigger event indicates that the Service Request has been quarantined, because an Anomaly Detection volume threshold or attribute limit has been breached | sr:QuarantinedRequest (See clause 3.9.17) | N46, N47, N48 | None | N/A |
| FirmwareVersionMismatch | N49. The trigger event indicates there is a mismatch between the Device’s Firmware Version in SMI and that returned by the Read Firmware Version Service Request and that the version returned by the Device matches an entry on the CPL with a status of “Current” N50. The trigger event indicates there is a mismatch between the Device’s Firmware Version in SMI and that returned by the Read Firmware Version Service Request, the Activate Firmware Service Request or the Future Dated Firmware Activation Alert and that the version returned by the Device matches an entry on the CPL with a status of “Removed” N51. The trigger event indicates there is a mismatch between the Device’s Firmware Version in SMI and that returned by the Read Firmware Version Service Request, the Activate Firmware Service Request or the Future Dated Firmware Activation Alert and the version returned by the Device doesn’t match an entry on the CPL N52. The trigger event indicates there is a mismatch between the GSME’s Firmware Version in SMI and that returned by the Read Firmware Version Service Request where the target Device is GPF | sr:FirmwareVersionMismatch (See clause 3.9.13) | N49, N50, N51, N52. | None | N/A |
| DualBandCHAlert | The trigger event indicates an Alert has been generated by the Dual Band CHF Device | sr:DualBandCHAlert (See clause 3.9.14) | N54 | None | N/A |
| S1SPAlertDSP | Used for conveying an S1SP Alert N55: The trigger event indicates that a SMETS1 Service Provider reports a Service Request validation error or other notification. N56: The trigger event is the provision of a prepayment top-up UTRN in response to a Service Request where SRV is 2.2 | sr:S1SPAlertDSP (See clause 3.9.15) | N55, N56 | None | N/A |
| SMETS1CHFirmwareNotification | See Clauses 1.4.7.13 and 1.4.7.14. | sr: SMETS1CHFirmwareNotification (See clause 3.9.18) | N57 | None | N/A |
| ALCSHCALCSConfigurationChange | The trigger event indicates the ESME’s ALCS/HCALCS/APC configuration has changed | sr:ALCSHCALCSConfigurationChange (See clause 3.9.19) | N58 | None | N/A |
| FirmwareUpgradeRequested | This Alert is used to share the list of Devices that have been approved by CSPs for firmware update. | sr:FirmwareUpgradeRequested (See clause 3.9.20) | N59 | None | N/A |
| CSPFirmwareDeliveryStatus | The trigger event indicates a notification has been generated by the CSP in relation to the transfer of a firmware image to a Comms Hub. | sr:CSPFirmwareDeliveryStatus (See clause 3.9.21) | N60, N61 | None | N/A |
| CommsHubAlert | The trigger event indicates an Alert has been generated by the Comms Hub | sr:CommsHubAlert (See clause 3.9.22) | N62 | None | N/A |
| ECoSAlert | The trigger event indicates that the ECoS Party has generated an ECoS Alert. | sr:ECoSAlert (See clause 3.9.23) | N63 | None | N/A |
| CoSCertificateAlert | The trigger event indicates that a Device has been installed with a CoSCertificate in its CoS Trust Anchor Cell that does not allign with an active CoS Party. | sr:CosCertificateAlert (See clause 3.9.24) | N65 | None | N/A |
| CommsHubFirmwareActivation | The trigger event indicates that a new version of Firmware has been activated on a SMETS2+ Comms Hub. | sr:FirmwareVersionActivation (See clause 3.9.25) | N64 | None | N/A |
| DUISVersionMismatch | The trigger event indicates that the DCC Alert or Service Response to be sent to the User is not compatible with their DUIS XSD version | sr:DUISVersionMismatch (See clause 3.9.16) | N999 | None | N/A |

Table 262 : DCCAlert (sr:DCCAlert) data items

## Amend Section 3.8.119.3 to the following:

      - 3.8.119.3		Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Request, the general Authorisation Checks as defined below shall not be carried out.

- A Response Code of E4 as defined in clause 3.2.4 “Verify that the User, in the User Role defined in the Service Request is a Eligible User for the Device”

- a Response Code of E5 as defined in clause 3.2.4 “Verify that the Service Request or Signed Pre-Command is applicable to the Device status”

| Response Code | Response Code Description |
| --- | --- |
| E081401 | The Device Type of the Device being notified is not CHF |
| E081402 | The install date & time supplied is a future date |
| W081401 | The CHF Device status is not ‘InstalledNotCommissioned’, which is the only valid status compatible with this Service Request. Note: Status of the Device for the CHF and the associated GPF will be updated to InstalledNotCommissioned if it is still in the Pending State. |

## Amend Section 1.3 Defined term to the following:

  - 1.3	Defined Terms

| Initial Retry Period | means the period of time as defined in clause 2.10.1 (Retry Processing), commencing from the issuance of the first command for delivery to Devices and concluding at the initiation of the Back-off period. |
| --- | --- |

## Amend Section 1.4.11.5 to the following:

      - 1.4.11.5SMETS1 Alerts

Clause 3.6.2 shall not apply in relation to SMETS1 Devices.

The DCC shall not generate the following Alerts:

- N13 for On Demand Service Request if a S1SP N55 Alert is already generated indicating failure of that On Demand Request

- N13 for Future Dated (Device) Service Request if a S1SP N55 Alert is already generated indicating failure of that Future Dated (Device) Request

- N11 for Future Dated (DSP) Service Request if a S1SP N55 Alert is already generated indicating failure of that Future Dated (DSP) Request

- N11 for DCC Scheduled Service Request is a S1SP N55 alert is already generated indicating failure of that DCC Scheduled Request

## Amend Section 3.2.4 to the following:

    - 3.2.4		Authorisation

The DCC shall verify that the User has permission to send the Service Request or Pre-Command as per the following steps and where authorisation checks are failed the following Response Code shall be added by the DCC to the Service Response that is sent to the sending User;

| Authorisation Check | Process | Response Code |
| --- | --- | --- |
| Validate the User Role | The sending organisation (User) as determined from the Business Originator ID and their associated User Role are checked to confirm it is a valid SEC party / User Role combination | E1 |
| Verify that the User Role is allowed to use the Service Request or Signed Pre-Command | This is a User Role based check for the mapping between Service Requests and User Roles (see clause 3.1 - Service Request Matrix) i.e. that the User Role is that of a User within an Eligible User Role for that Request. | E2 |
| Verify the status of the User | This is a status based check to find out if the User is suspended (not allowed to run that Service Request or Signed Pre-Command) at the time when the Service Request or Signed Pre-Command is received | E3 |
| Verify that the User, in the User Role defined in the Service Request is an Eligible User for the Device | This check is based on the Registration Data associated with the Device via MPxN lookup. Check that the User is an Eligible User in respect for that Device for the period that the Service Request pertains to. The checks for eligibility are as follows : - Confirm (using the Registration Data) that the User ID used to send the Request is that of a User that is an Eligible User for the Request. - Authorisation is performed using the Device specified in the BusinessTargetID except for Non-Device Service Requests, where the BusinessTargetID is specified in the Service Request itself. Note that this check is not applied for Critical Service Requests or Critical Signed Pre-Commands or for a limited number of specific Service Requests as documented in the Service Request Processing Document and stated explicitly within each Service Request definition in clause 3.8. Requests from a User that had ceased to be a registered Party more than 24 months ago will be rejected by the DCC Systems. | E4 |
| Verify that the Service Request or Signed Pre-Command is applicable to the Device status | This is a check to confirm that the target Device has a status within the Smart Metering Inventory that enables the User to send it the particular Service Request or Signed Pre-Command This check is not applicable to Service Requests 8.2 (Read Inventory) and 12.1 (Request WAN Matrix) or to Critical Service Requests or Signed Pre-Commands. With the exception that it is applied for Signed Pre-Commands when the Device Status is ‘Recovery’. Devices can only be communicated with in response to a Request if they are in a status of ‘Commissioned’, ‘InstalledNotCommissioned’, ‘Whitelisted’, ‘Pending’ or ‘Recovered’ in the Smart Metering Inventory. The DCC shall, where the Device has a Smart Metering Inventory (SMI) Status of ‘Suspended’ prevent any Non-Critical Service Requests from being processed with the exception of, Service Requests 11.1 (Update Firmware), 11.2 (Read Firmware Version), 11.4 (Update PPMID Firmware) and 6.23 (Update Security Credentials (CoS)). The DCC shall, where a Device has a Smart Metering Inventory (SMI) Status of ‘Recovery’ prevent any Service Requests relating to that Device from being processed with the exception of Non-Device Service Requests (subject to their specific validation).Note that where a Device has an SMI Status of ‘Recovered’ the Device’s SMI Status immediately prior to it having the SMI Status of ‘Recovery’ shall be used in validation. | E5 |
| Verify that the Service Request or Signed Pre-Command is available for Local Command Services | This is a check to confirm that a Service Request or Signed Pre-Command is available to Users for local delivery to a Device using Local Command Services including additional reference to the requesting User Role and SMI Status combination. A Service Request or Signed Pre-Command is not available to Users for local delivery using Local Command Services where the Service Request or Signed Pre-Command is one of the following; - A Service Reference Variant of 8.1.1 – Commission Device - A Future Dated Service as defined by clause 2.6.3 In addition, a Service Request or Signed Pre-Command can only be delivered locally in the following combinations of requesting User Role and SMI Status of the target Device: - Where the User Role of the sender is either IS, ES or GS, the target Device within the request must have an SMI Status of either “Pending”, “Whitelisted”, “InstalledNotCommissioned” or “Commissioned”. - Where the User Role of the sender is either ED, GT, RSA or OU, the target Device within the request must have an SMI Status of either “InstalledNotCommissioned” or “Commissioned”. Note that where a Device has an SMI Status of ‘Recovered’ the Device’s SMI Status immediately prior to it having the SMI Status of ‘Recovery’ shall be used in validation. | E17 |
| Verify that the Device exists | This is a check to confirm that the target Device within the Service Request or Signed Pre-Command exists Note that this check is only applicable to Service Requests and Signed Pre-Commands that are addressed to a specified Device. For Non-Device Service Requests this Response Code (E19) shall be returned if the BusinessTargetID is not the DCC Access Control Broker ID. | E19 |

Table 22 : Authorisation checks

## Amend Section 1.4.7.5 to the following:

      - 1.4.7.5	Top Up Device SRV 2.2

This clause is a supplement to clause 3.8.9.

The clause 3.8.9 definition of the UTRN data item shall, for SMETS1 Service Requests, not apply.

For SMETS1 Service Requests with a Command Variant value of 1, the UTRN data item shall contain a SMETS1 UTRN.

For SMETS1 Service Requests with a Command Variant value of 2 or 3, the UTRN data item shall contain a 20 digit string (each digit taking a value of 0-9) representing value in pence, using leading zeros as necessary to give a 20 digit length.

Where the Service Request has a Command Variant value of 2 or 3, the DCC shall send a synchronous response detailing whether the Service Request has passed initial validation.

Where the Service Request has a Command Variant value of 2, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’.

Where the Service Request has a Command Variant value of 2 or 3 and is successfully validated by the DCC, the DCC shall generate a SMETS1 UTRN.

Where the Service Request has a Command Variant value of 1 or 3, the DCC shall then send the SMETS1 UTRN contained within or resulting from the Service Request to the Device.