This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

MP164 ‘November 2021 SEC Release supporting changes’

Annex A

Legal text – version 0.1

About this document

This document contains the redlined changes to the SEC that would be required to deliver this Modification Proposal.

Where the following changes are already impacted by SECMP0007 ‘Firmware updates to IHDs and PPMIDs’, those changes have been redlined against the approved SECMP0007 legal text.

Schedule 9 ‘Smart Metering Equipment Technical Specifications 2’

These changes have been redlined against the approved SECMP0007 legal text.

These changes will be applied to the following Smart Metering Equipment Technical Specifications (SMETS)2 Device level versioning:

Prepayment Interface Device Technical Specification (PPMIDTS) Version 4.4

HAN Connected Auxiliary Load Control Switch Technical Specification (HCALCSTS) Version 5.1

## Amend Section 7.5.2.5 as follows:

      - 7.5.2.5		Activate Firmware

A Command to activate Firmware.

The PPMID shall be capable of installing new Firmware using a mechanism that is robust against failure and loss of data.

The new Firmware shall include version information. Where new Firmware is successfully installed, the PPMID shall be capable of recording the version information of that new Firmware in Firmware Version (7.6.4.1).

## Amend Section 8.5.1.8 as follows:

      - 8.5.1.8		Receive Firmware

A Command to receive Firmware.

In executing the Command the HCALCS shall be capable of:

- only accepting new Firmware from an Authorised and Authenticated source; and

- verifying the Authenticity and integrity of new Firmware before installation.

Appendix AD ‘DCC User Interface Specification’

Where the associated change was impacted by the SECMP0007, those changes have been redlined against the approved SECMP0007 legal text and confirmed following the associated header.

The other changes have been redlined against Appendix AD version 4.0.

These changes will be applied to Appendix AD version 5.0

## Amend Table 2 ‘Additional or Alternative Validation Conditions for SMETS1 Devices’ as follows:

    - 1.4.6	Additional or Alternative Validation Conditions for SMETS1 Service Requests

In relation to SMETS1 Service Requests, the DCC shall apply the additional or alternative validation in this clause 1.4.6, additionally or as alternatives to the “Specific Validation for this Request” specified in clause 3.8.

| Service Reference Variant | Response Code | Applicable to SMETS1 Devices only or Amended conditions for SMETS1 Devices? | Validation Check |
| --- | --- | --- | --- |
| 1.1.1 | E010102 | SMETS1 Devices only | If the target Device is a SMETS1 ESME according to the Smart Metering Inventory, the XML element named HybridTariff must not be included in the ElecPriceElementsPrimary element within PriceElement (see 3.8.1.2). For clarity, SMETS1 ESME are not required to support such tariffs. |
| 1.2.1 | E010201 | SMETS1 Devices only | If the target Device is a SMETS1 ESME according to the Smart Metering Inventory, the XML element named HybridTariff must not be included in the ElecPriceElementsPrimary element within PriceElement (see 3.8.1.2). For clarity, SMETS1 ESME are not required to support such tariffs. |
| 3.3 | E030302 | SMETS1 Devices only | Check that if the ESMEEventLogType is ALCS then the target Device is not a SMETS1 Device according to the Smart Metering Inventory. For clarity, SMETS1 ESMEs are not required to support such logs. |
| 5.1 | E050110 | SMETS1 Devices only | Where DeviceID in the Service Request is a SMETS1 Device according to the Smart Metering Inventory, check that the DSPScheduledServiceReferenceVariant is supported for a SMETS1 Device according to clause 3.1. |
| 6.7 | E060701 | Amended condition for SMETS1 Devices | If the target Device is a SMETS1 GSME according to the Smart Metering Inventory, the XML element named UncontrolledGasFlowRateDecimal must not be included in the Service Request.For clarity, only the XML element named UncontrolledGasFlowRate may be used with SMETS1 devices |
| 6.13 | E061305 | SMETS1 Devices only | Check that if the LogToRead is ALCSEvent or PowerEvent then the target Device is not a SMETS1 Device according to the Smart Metering Inventory. For clarity, SMETS1 ESME are not required to support such logs. |
| 6.15.1 | E061508 | SMETS1 Devices only | Check that the role of the User submitting the Service Request and RemotePartyRole align. Specifically if the User’s role is ES or GS, the RemotePartyRole must be Supplier. If the User’s role is GT or ED, the RemotePartyRole must be NetworkOperator. |
| 8.9 | E080901 | SMETS1 Devices only | Check that if the target Device is a SMETS1 Device according to the Smart Metering Inventory then the target Device’s Device Type is CHF according to the Smart Metering Inventory. For clarity, SMETS1 does not require any other Device Logs be supported. |
| 11.1 | W110101 | Amended condition for SMETS1 Devices | Invalid conditions listed in this Update Firmware warning may relate to a SMETS1 CHF, a SMETS1 PPMID or a Smart Meter. An additional condition for listing a SMETS1 CHF or a SMETS1 PPMID in the InvalidDeviceIDList is where the DCC User submitting the Update Firmware Service Request is not the Responsible Supplier for the SMETS1 ESME which is connected to the same home area network as the SMETS1 CHF / PPMID. The Response Code description within the InvalidDeviceIDList shall not contain the OtherRequestsInProgressList Update Firmware Warning as this is not applicable for SMETS1 Devices. |

Table 2 : Additional or Alternative Validation Conditions for SMETS1 Devices

## Amend Table 41 ‘DCC Alert Codes’ as follows:

      - 3.6.3.4	DCC Alert Codes

| DCC Alert Code | Alert Name | Event | Trigger | DCC Alert Recipient | Device Applicability |
| --- | --- | --- | --- | --- | --- |
| AD1 | Power Outage Event | Power Outage Event received from CSP | Communications Service Provider (CSP) notification of loss of DC power as detected at the Communications Hub in the Consumer Premises for a time equal to or greater than three (3) minutes | IS ED (User ID with User Role IS / ED for an Electricity Smart Meter associated with the Communications Hub Function reporting the Power Outage) GS GT (User ID with User Role GS / GT for a Gas Smart Meter associated to the Communications Hub Function reporting the Power Outage) | SMETS2+ |
| N1 | Electricity Smart Meter Decommission or withdrawal | Decommissioning or withdrawal of an Electricity Smart Meter Device | Upon successful completion of Service Request: - Decommission Device (8.3) - or Service Opt Out (8.5) for an Electricity Smart Meter Device | ED and, if applicable, ES | All |
| N2 | Gas Smart Meter Decommission or withdrawal | Decommissioning or withdrawal of Gas Smart Meter Device | Upon successful completion of Service Request: - Decommission Device (8.3) - or Service Opt Out (8.5) for a Gas Smart Meter Device | GT | All |
| N3 | Cancellation of “Future Dated Response Pattern (DSP)” requests because of Change of Tenancy | Cancellation of “Other User” “Future Dated Response Pattern (DSP)” Commands not yet submitted to the Devices in the Electricity or Gas Smart Metering System | Upon successful completion of Service Request Restrict Access for Change of Tenancy (3.2) | All applicable Future Dated (DSP) request senders | All |
| N4 | Schedule removal because of Change of Tenancy | Removal of “Other User” “DCC Scheduled” schedules for Devices in the Electricity or Gas Smart Metering System | Upon successful completion of Service Request Restrict Access for Change of Tenancy (3.2) | All applicable Schedule “owners” | All |
| N5 | Schedule removal because of Device withdrawal | “DCC Scheduled” schedule removal | Upon successful completion of Service Request Service Opt Out for a Device (8.5) For Gas Smart Meter withdrawals, schedule removal are additionally applicable to Gas Proxy Function | All applicable Schedule “owners” | SMETS2+ |
| N6 | Schedule removal because of Device decommission | “DCC Scheduled” schedule removal | Upon successful completion of Service Request Decommission Device for a Device (8.3) | All applicable Schedule “owners” | All |
| N7 | “DSP Scheduled” / “Future Dated Response Pattern (DSP)” access control failure | “DCC Scheduled” / “Future Dated Response Pattern (DSP)” access control failure (Authorisation, Device status, GBCS compatibility check) | “DCC Scheduled” / “Future Dated Response Pattern (DSP)” Command generation access control failure | Schedule “owner” / “Future Dated Response Pattern (DSP) request sender | All |
| N8 | Device removed from Inventory- Pending Status expired | Removal of Device from Inventory | Device in a status of ‘Pending’ for > 36 months | Original User that requested addition of the Device to the DCC Inventory | All |
| N9 | Communications Hub Decommission | Decommission of Communications Hub | Upon successful completion of Service Request Decommission Device for a Communications Hub (8.3) | All Responsible Suppliers for that CH function, other than the IS / GS that instigated the Decommissioning ED GT | All |
| N10 | “Future Dated Response Pattern (Device)” Command time-out | “Future Dated Response Pattern (Device)” Command time-out | “Future Dated Response Pattern (Device)” Command response not received from the device within the Target Response Time from the ExecutionDateTime | “Future Dated Response Pattern (Device)” request sender | SMETS2+ Device |
| N11 | “DSP Scheduled” / “Future Dated Response Pattern (DSP)” Command time-out | “DCC Scheduled” / “Future Dated Response Pattern (DSP)” Command time-out | “DCC Scheduled” Schedule instance / “Future Dated Response Pattern (DSP)” Command not sent to or response not received from the Device within the Target Response Time from the ExecutionDateTime | Schedule “owner” / “Future Dated Response Pattern (DSP) request sender | All |
| N12 | Failure to deliver Command to Device | Failure to deliver Command to Device | Failure to receive an acknowledgement notification from a CSP or S1SP via the SM WAN for an “On Demand” or “Future Dated” Command | Request sender | All |
| N13 | Failure to receive Response from Device | Failure to receive Response from Device | Failure to receive a response from a Device for an “On Demand” Command or “Future Dated” Command Acknowledgement | Request sender | All |
| N14 | Sequenced Request Failure | Sequenced Request Failure | Previous Command in sequence failed or timed-out | Request sender | All |
| N15 | Sequenced Request received out of order | Sequenced Request received out of order | Preceding Request not received during “Wait Period” | Request sender | All |
| N16 | Device Identity Confirmation | Device Identity Confirmation by Responsible Supplier – either first setting (as part of Installation and Commissioning process) or update to previous setting | Upon successful receipt of Service Response Code I0 from Service Request Update HAN Device Log (initial setting) (8.11) OR Upon successful processing of a Service Request 8.4 Update Inventory for an update to MPxN . | ED GT | All |
| N17 | Schedule removal because of CoS | Previous Responsible Supplier “DCC Scheduled” schedule removal | Upon successful completion of Service Request Update Security Credentials (CoS) (6.23) | Previous IS Previous GS | All |
| N18 | Firmware Version / Hash mismatch | Firmware Version / Hash mismatch | Firmware Hash calculated by CSP or S1SP doesn’t match Firmware Version | Update Firmware request sender | All |
| N19 | Firmware Distribution Device ID identification failure | Firmware Distribution Device ID identification failure | CSP or S1SP unable to identify Communications Hub Device Id a Firmware Image is to be sent to | Update Firmware request sender | All |
| N20 | Firmware image provided is too large | Firmware image provided is too large | CSP or S1SP unable to process request, because the Firmware Image is too large | Update Firmware request sender | All |
| N21 | Unknown Firmware Version | Unknown Firmware Version | CSP or S1SP unable to process request, because it doesn’t recognise the Firmware Version | Update Firmware request sender | All |
| N22 | Failure to deliver Update Firmware Command to CSP | Failure to deliver Update Firmware Command to CSP | Failure to receive an acknowledgement notification from a CSP or S1SP via the SM WAN for an Update Firmware Command | Update Firmware request sender | All |
| N23 | Failure to receive Update Firmware Command Validation response from CSP | Failure to receive Update Firmware Command Validation response from CSP | Failure to receive Update Firmware Command Validation response from CSP | Update Firmware request sender | All |
| N24 | Successful Communications Hub Function Whitelist Update | Communications Hub Function Whitelist Update | The DCC has received positive confirmation that the requested addition to the Communications Hub Function’s whitelist resulted in establishing communications with the Device | Update HAN Device Log request sender | All |
| N25 | Potentially Unsuccessful Communications Hub Function Whitelist Update | Communications Hub Function Whitelist Update | The DCC has not received positive confirmation that the requested addition to the Communications Hub Function’s whitelist resulted in establishing communications with the Device | Update HAN Device Log request sender | All |
| N26 | Update Security Credentials (CoS)– access control failure | Update Security Credentials (CoS)– access control failure | Request has failed CoS Party access control or, for Future Dated Requests, DSP access control at the point the Request is to be sent to the CoS Party | Update Security Credentials (CoS) request sender | All |
| N27 | Device CoS | New Import Supplier for Device | Upon successful completion of Service Request Update Security Credentials (CoS) (6.23) | Previous IS Previous GS | All |
| N28 | Device Suspended | Device Suspended | Suspension of Device | IS GS ED GT | All |
| N29 | Device Restored from Suspension | Device Restored from Suspension | Restoration of Device following Previous Suspension | IS GS ED GT | All |
| N30 | CHF Device Log Restored | CHF Device Log Restored | Upon successful completion of Service Request 8.12.1 Restore HAN Device Log | GS IS All Responsible Suppliers for the CHF, other than the IS / GS that submitted the Request | SMETS2+ |
| N31 | GPF Device Log Restored | GPF Device Log Restored | Upon successful completion of Service Request 8.12.2 Restore GPF Device Log if the sender is not the GS. | GS IS All Responsible Suppliers for the CHF, other than the IS / GS that submitted the Request | SMETS2+ |
| N33 | Cancellation of Future Dated Response Pattern (DSP) requests because of Device Decommission | Cancellation of all Future Dated Response Pattern (DSP) Services not yet submitted to the Device | Upon successful completion of Service Request 8.3 Decommission Device for a Device | All applicable Future Dated Response Pattern (DSP) Request senders | All |
| N34 | Cancellation of Future Dated Response Pattern (DSP) requests because of CHF Decommission | Cancellation of all Future Dated Response Pattern (DSP) Services not yet submitted to the CHF and its associated GPF | Upon successful completion of Service Request 8.3 Decommission Device for a Device | All applicable Future Dated Response Pattern (DSP) Request senders | All |
| N35 | Cancellation of Future Dated Response Pattern (DSP) requests because of Device Withdrawal | Cancellation of all Future Dated Response Pattern (DSP) Services not yet submitted to the Device | Upon successful completion of Service Request 8.5 Service Opt Out for a Device | All applicable Future Dated Response Pattern (DSP) Request senders | SMETS2+ |
| N36 | Cancellation of Future Dated Response Pattern (DSP) requests because of CHF Withdrawal | Cancellation of all Future Dated Response Pattern (DSP) Services not yet submitted to the CHF and Devices in its Whitelist | Upon successful completion of Service Request 8.4 Update Inventory for a CHF Withdrawal | All applicable Future Dated Response Pattern (DSP) Request senders | SMETS2+ |
| N37 | Schedule removal because of CHF Withdrawal | “DCC Scheduled” schedule removal for ESME, GSME and GPF in the Whitelist | Upon successful completion of Service Request 8.4 Update Inventory for a CHF Withdrawal | All applicable Schedule “owners” | SMETS2+ |
| N38 | Cancellation of Future Dated Response Pattern (DSP) requests because of CoS | Cancellation of all “Future Dated (DSP)” Services not yet submitted to the Device from the previous Responsible Supplier | Upon successful completion of Service Request 6.23 Update Security Credentials (CoS) | Previous IS Previous GS | All |
| N39 | PPMID Alert | A PPMID Device generates an Alert as defined by GBCS | PPMID Alert received by the DSP Access Control Broker | IS for the Primary Import MPAN in the Smart Metering System GS for the Import MPRN in the Smart Metering System | SMETS2+ |
| N40 | Schedule removal because of Device Suspension | “DSP Scheduled” schedule removal | Suspension of Device | All applicable Schedule “owners” | All |
| N41 | Cancellation of “Future Dated (DSP)” requests because of Device Suspension | Cancellation of all “Future Dated (DSP)” Services not yet submitted to the Device | Suspension of Device | All applicable Future Dated (DSP) Request senders | All |
| N42 | Security Credentials updated on the device | Security Credentials updated on Device by Service Request 6.15.1 or 6.21 | Success Response from Update Security Credentials where the Remote Party whose certificate has been placed on the Device is not the sender of the Service Request | The Remote Party whose certificate has been placed on the Device. | All |
| N43 | PPMID Removal | A PPMID Device has been removed from the HAN via Service Request 8.11 | Success Response from Update HAN Device Log (Remove) where the removed Device is a PPMID. DCC Alert is only sent if the PPMID device was joined to both an ESME and the GSME as identified by the Smart Metering Inventory. | All Responsible Suppliers for the CHF, other than the IS / GS that submitted the Service Request | SMETS2+ |
| N44 | Recovery Complete (ACB Credentials) | Recovery is complete and at least one of the KRP Device Security Credentials on the Device has been replaced with those from an ACB Certificate | Recovery is complete and KRP Device Security Credentials on the Device replaced by those from an ACB by the Recovery Process | IS GS | SMETS2+ |
| N45 | Recovery Complete | Recovery is complete and all required Device Security Credentials on the Device have been replaced | Recovery is complete and Device Security Credentials on the Device have been replaced by the Recovery Process | IS GS ED GT | SMETS2+ |
| N46 | Quarantined Request – Anomaly Detection User Threshold Breach | An Anomaly Detection User-specific volume threshold has been exceeded | Request quarantined, because an Anomaly Detection User-specific volume threshold has been exceeded | Service Request / Signed Pre-Command sender | All |
| N47 | Quarantined Request – Anomaly Detection DCC Threshold Breach | An Anomaly Detection DCC system-wide volume threshold has been exceeded | Request quarantined, because an Anomaly Detection DCC system-wide volume threshold has been exceeded | Service Request / Signed Pre-Command sender | All |
| N48 | Quarantined Request – Anomaly Detection Attribute Limits Breach | An Anomaly Detection Attribute Limit has been breached | Request quarantined, because an Anomaly Detection Attribute Limit has been breached | Service Request / Signed Pre-Command sender | SMETS2+ |
| N49 | Firmware Version Updated in the Smart Metering Inventory | Device’s Firmware Version updated in the Smart Metering Inventory | Upon successful completion of Service Request 11.2 Read Firmware Version where the target Device is an ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device is different from that in the SMI and it matches an entry on the CPL with a status of “Current” | IS GS (Only sent if the IS / GS did not submit the Service Request) | All |
| N50 | Firmware Version no longer valid on the CPL | Device’s Firmware Version updated in the Smart Metering Inventory, but Device Status not set to ‘Suspended’ | Upon successful completion of Service Request 11.2 Read Firmware Version where the target Device is an ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device is different from that in the SMI and it matches an entry on the CPL with a status of “Removed” OR Upon completion of Service Request 11.3 Activate Firmware where the Firmware Version returned by the Device is different from that in the SMI and it matches an entry on the CPL with a status of “Removed” OR Future Dated Firmware Activation Alert (Alert Code 0x8F66 or 0x8F67 and Message Code 0x00CA) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it matches an entry on the CPL with a status of “Removed” OR PPMID Firmware Activation Alert (Alert Code 0x8F8B) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it matches an entry on the CPL with a status of “Removed” | IS GS | All |
| N51 | Invalid Firmware Version | Device’s Firmware Version is unknown (not in the CPL) Device’s Firmware Version not updated in the Smart Metering Inventory | Upon successful completion of Service Request 11.2 Read Firmware Version where the target Device is an ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR Upon completion of Service Request 11.3 Activate Firmware where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR Future Dated Firmware Activation Alert (Alert Code 0x8F66 or 0x8F67 and Message Code 0x00CA) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL OR PPMID Firmware Activation Alert (Alert Code 0x8F8B) received by the DCC Systems where the Firmware Version returned by the Device is different from that in the SMI and it doesn’t match an entry on the CPL | IS GS | All |
| N52 | GSME Firmware Version Mismatch | GSME’s Firmware Version returned by the GPF is different from that in the Smart Metering Inventory | Upon successful completion of Service Request 11.2 Read Firmware Version where the target Device is GPF and the GSME Firmware Version returned by the GPF is different from that in the SMI | GS | All |
| N53 | Command not delivered to ESME | Receipt by the DCC Access Control Broker of an Alert 0x8F84 - Failure to Deliver Remote Party Message to ESME (as defined by GBCS) from the CHF (Please note that DCC Alert N53 does not replace existing N12 or N13 DCC Alerts from the DCC, which will continue to be produced to confirm the DCC processing of the relevant Service Request. DCC Alert N53 is triggered by the CHF Alert and should be regarded as additional information which may be used by the User to adjust the frequency of requests being sent to the relevant ESME device. It is likely that after receipt of a DCC Alert N53 a User shall receive a subsequent DCC Alert N13 at the end of the Final Retry Period for the Service Request sent if applicable) | Receipt by the DCC Access Control Broker of an Alert from a Communications Hub Function with Alert Code 0x8F84 and Message Code 0x00D5 | Request sender | SMETS2+ |
| N54 | Dual Band CH Sub GHz Alert | The DCC Systems receive a Sub GHz Alert | A Sub GHz Alert is received by the DCC Access Control Broker as defined by GBCS section 16.1, being one of: - Alerts without specific payload: - 0x8F21 (Duty Cycle fallen below Normal-Limited Duty Cycle Threshold) - 0x8F22 (Critical Duty Cycle Action Taken) - 0x8F23 (Duty Cycle fallen below Limited-Critical Duty Cycle Threshold) - 0x8F24 (Regulated Duty Cycle Action Taken) - 0x8F25 (Duty Cycle fallen below Critical-Regulated Duty Cycle Threshold) - 0x8F27 (Sub GHz Channel Scan initiated) - 0x8F29 (Three Lost GSME Searches Failed) - 0x8F2B (Sub GHz Channel not changed due to Frequency Agility Parameters) - Alerts with specific payload: - 0x8F20 (Limited Duty Cycle Action Taken) - 0x8F26 (Sub GHz Channel Changed) - 0x8F28 (Sub GHz Channel Scan Request Assessment Outcome) - 0x8F2A (Sub GHz Configuration Changed) - 0x8F2C (Message Discarded Due to Duty Cycle Management) - 0x8F2D (No More Sub GHz Device Capacity) | IS GS | SMETS2+ |
| N55 | SMETS1 Service Provider Alert | DCC Alert initiated by a SMETS1 Service Provider | The trigger is indicated in the S1SPAlert code provided in the DCC Alert. The payload is delivered in the S1SPAlert format; see section 3.9.15 | Service Request sender | SMETS1 |
| N56 | SMETS1 Service Provider Provision of prepayment top-up UTRN | DCC Alert containing a prepayment top-up UTRN provided by a SMETS1 Service Provider | The trigger is a User request for a prepayment top-up. The UTRN is delivered in the S1SPAlert format; see section 3.9.15 | Service Request sender | SMETS1 |
| N57 | SMETS1 CHF or SMETS1 PPMID Firmware notification | See clauses 1.4.7.13 and 1.4.7.14. | See clauses 1.4.7.13 and 1.4.7.14. | Gas Supplier for the GSME on the same home area network | SMETS1 |
| N58 | ALCS/HCALCS configuration change | ALCS/HCALCS configuration changed on ESME | Upon successful completion of Service Request 6.14.2 Update Device Configuration (Auxilliary Load Control Scheduler) OR Upon successful completion of Service Request 6.14.1 Update Device Configuration (Auxilliary Load Control Descriptions) OR Upon successful completion of Service Request 6.14.3 Update Device Configuration (Auxiliary Controller Scheduler) OR Future Dated Execution Of Instruction Alert (DLMS COSEM) Alert (Alert Code 0x8F66 and Message Code 0x00CC) corresponding to AuxiliaryLoadControlSwitchesCalendar received by the DCC Data Systems | ED | SMETS2+ |
| N59 | Firmware update in progress | DCC Alert indicating that the CSP’s checks are passed and the image will be sent to the Comms Hub for processing. | Notification from CSP indicating completion of validation. | All Responsible Suppliers for the Device, other than the IS / GS that submitted the Service Request | SMETS2+ |
| N60 | Failed to deliver Firmware image to Comms Hub | Failed to deliver Firmware image to Comms Hub | Notification from CSP indicating that the image could not be delivered to the Comms Hub. | Update Firmware request sender | SMETS2+ |
| N61 | Firmware image successfully delivered to Comms Hub | Firmware image successfully delivered to Comms Hub | Notification from CSP indicating that the image has been successfully delivered to the Comms Hub. | Update Firmware request sender | SMETS2+ |
| N62 | Comms Hub Alert | A Comms Hub generates a Device Alert as defined by GBCS | An Alert from a Comms Hub is received by the DSP Access Control Broker. | IS GS | SMETS2+ |
| N999 | DUIS Version Mismatch | User’s DUIS version is incompatible with the DCC Alert or Service Response to be sent | The DCC Alert or Service Response is not compatible with the DUIS version used by the User | Recipient of the incompatible DCC Alert or Service Response | All |

Table 41 : DCC Alert Codes

## Amend Section 3.8 ‘Service Request Definitions’ as follows:

    - 3.8.123	Update Firmware

      - 3.8.123.1	Service Description

| Service Request Name | - UpdateFirmware |  |
| --- | --- | --- |
| Service Reference | - 11.1 |  |
| Service Reference Variant | - 11.1 |  |
| Eligible Users | Import Supplier (IS) Gas Supplier (GS) |  |
| Security Classification | Non Critical |  |
| BusinessTargetID - Device Type applicable to this request | DCC Access Control Broker |  |
| Can be future dated? | No |  |
| On Demand? | No |  |
| Capable of being DCC Scheduled? | No |  |
| Command Variants applicable to this Request (Only one populated | 8 – Non-Device |  |
| Common Header Data Items | See clause 3.4.1.1 |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause 3.5 for more details on processing patterns - Acknowledgement - Response to a Non-Device Service Request Also see Response Section below for details specific to this request |  |
| Response Codes possible from this Service Request | See clause 3.5.10 for Common Response Codes |  |
| GBCS Cross Reference | Electricity | Gas |
| GBCS MessageCode | N/A | N/A |
| GBCS Use Case | N/A | N/A |

      - 3.8.123.2	Specific Data Items for this Request

UpdateFirmware Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| FirmwareImage | The Firmware Image corresponding to the Firmware Version This is a base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image”. The Firmware Image is the full OTA Upgrade Image as defined in GBCS. Note that this includes not only the Manufacturer Image but also additional signature and OTA Header information. Please see GBCS for details of how to construct the OTA Upgrade Image | Restriction of xs:base64Binary (max Length = 10240000) | Yes | None | N/A |
| FirmwareVersion | An identifier representing a firmware image that has been approved for release by the User concerned. The Firmware version as held in the CPL and presented in the format XXXXXXXX where each X is one of the characters 0 to 9 or A to F. This value must align with the firmware version value listed on the Central Products List (excluding the colon separator between octet values) and the Firmware Hash held on the CPL must match the Manufacturer Image Hash pursuant to clause 6.1(i) of the Service Request Processing Document. For avoidance of doubt, there is no direct comparison made between this FirmwareVersion value to the File Version value contained in the OTA Header (as defined by GBCS). | sr:FirmwareVersion (Restriction of xs:string (minLength=1, maxLength = 8)) | Yes | None | N/A |
| DeviceIDList | Comma separated list of Device IDs. Containing a minimum of 1 and a maximum of 50000 Device IDs, each as defined by sr:EUI | Restriction of xs:string (minLength = 23 maxLength = 1199999 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”) | Yes Minimum 1 and Maximum 50,000 Device IDs | None | N/A |

Table 254 : UpdateFirmware (sr:UpdateFirmware) data items

      - 3.8.123.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’

| Response Code | Response Code Description |
| --- | --- |
| E110101 | The Firmware Version is not approved. The Firmware version ID does not align with an entry on the Central Products List. |
| W110101 | The Update Firmware Warning contains between one and five lists of Device IDs for which the validation failed for each DeviceID: - InvalidDeviceIDList . The DeviceID does not exist or the User is not the Meter or HCALCS Import Supplier - NotCommissionedDeviceIDList. The Meter status is not ‘Commissioned’ or ‘Suspended’ or the Device is not a Meter or a HCALCS - NotApplicableFirmwareDeviceIDList. The Firmware Version is not applicable to the Device - OtherRequestsInProgressList. The Device already has another Firmware update request in progress. - InvalidGBCSVersionList. The CHF associated with the HCALCS Device has a GBCS Version prior to v4.1, or the HCALCS Device currently has a GBCS Version that is prior to v4.1. |
| E110102 | The firmware is not marked as active in the Central Products List and Smart Meter Inventory. |
| E110103 | The DCC Systems calculated Hash value over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request differs from that held in the CPL for the specified FirmwareVersion. |
| E110105 | The firmware image is not constructed as per the GBCS definition, i.e. the FirmwareImage does not contain both an OTA Header and a Firmware Image concatenated together , or where the Target Device is a SMETS2+ Device, the Length of the base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image” exceeds 1024000. |

      - 3.8.123.4	Specific Data Items in the Response

This Service Request synchronous response is defined in the XSD DSPUpdateFirmwareWarning XML element, which contains the list(s) of Device IDs that failed DCC Authorisation / Validation.

The DCC shall return the following data items which are specific to this Service Request for Successful Responses

DSPUpdateFirmwareWarning Definition:

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| InvalidDeviceIDList | Comma separated list of Device IDs for which the User ID is not the Import Supplier or the DeviceIds don’t exist. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotCommissionedDeviceIDList | Comma separated list of Device IDs which aren’t in a status of “Commissioned” or “Suspended” or the Device is not a Meter or a HCALCS. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotApplicableFirmwareDeviceIDList | Comma separated list of Device IDs for which the Firmware is not applicable. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |

| OtherRequestsInProgressList | Comma separated list of Device IDs for which another Firmware Update request is in progress. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| --- | --- | --- | --- | --- | --- |
| InvalidGBCSVersionList | Comma separated list of Device IDs for which either; - the HCALCS DeviceID for which the GBCS Version of the associated CHF is prior to v4.1 - HCALCS Device IDs for which the current GBCS Version is prior to v4.1 | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |

Table 255 : DSPUpdateFirmwareWarning (sr:DSPUpdateFirmwareWarning) data items

      - 3.8.123.5	Additional DCC System Processing

The DCC Systems shall calculate a Hash over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request and ensure that the Hash calculated matches that held in the CPL for the specified FirmwareVersion. Where the DCC identifies any mismatch in Hash values, a Response Code of E110103 shall be returned to the User.

    - 3.8.124	Read Firmware Version

      - 3.8.124.1	Service Description

| Service Request Name | - ReadFirmwareVersion |  |  |  |
| --- | --- | --- | --- | --- |
| Service Reference | - 11.2 |  |  |  |
| Service Reference Variant | - 11.2 |  |  |  |
| Eligible Users | Import Supplier (IS) Export Supplier (ES) Gas Supplier (GS) Registered Supplier Agent (RSA) Electricity Distributor (ED) Gas Transporter (GT) Other User (OU) |  |  |  |
| Security Classification | Non Critical |  |  |  |
| BusinessTargetID - Device Type applicable to this request | Electricity Smart Meter(ESME) Gas Smart Meter (GSME) Gas Proxy Function (GPF) Communications Hub Function (CHF) HAN Connected Auxiliary Load Control Switch (HCALCS) PrePayment Interface Device (PPMID) |  |  |  |
| Can be future dated? | DSP |  |  |  |
| On Demand? | Yes |  |  |  |
| Capable of being DCC Scheduled? | No |  |  |  |
| Command Variants applicable to this Request (Only one populated | 1 - Send (Non-Critical) 2 - Return for local delivery (Non-Critical) 3 - Send and Return for local delivery (Non-Critical) |  |  |  |
| Common Header Data Items | See clause 3.4.1.1 |  |  |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |  |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause 3.5 for more details on processing patterns - Acknowledgement - Service Response from Device – GBCSPayload - Response to a Command for Local Delivery Request – LocalCommand Format Also see Response Section below for details specific to this request |  |  |  |
| Response Codes possible from this Service Request | See clause 3.5.10 for Common Response Codes |  |  |  |
| GBCS Cross Reference | Electricity (ESME) and Communications Hub | Gas | HCALCS | PPMID |
| GBCS version earlier than v4.1 MessageCode | 0x0059 | 0x0084 | N/A | N/A |
| GBCS version earlier than v4.1 Use Case | ECS52 | GCS38 | N/A | N/A1 |
| GBCS v4.1 or later MessageCode | 0x0059 | 0x0084 | 0x0129 | 0x0129 |
| GBCS v4.1 or later Use Case | ECS52 | GCS38 | CS08 | CS08 |

| GBCS Commands - Versioning Details |  |  |
| --- | --- | --- |
| DCC System creates the following GBCS Commands or Response Codes based on the following combinations, |  |  |
| Device Type | ESME |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS v1.0 or later |  |
| DEFAULT - No specific XML criteria | ECS52 |  |
| Device Type | GSME |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS v1.0 or later |  |
| DEFAULT - No specific XML criteria | GCS38 |  |
| Device Type | PPMID |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS version earlier than v4.1 | GBCS v4.1 or later |
| DEFAULT - No specific XML criteria | CS081 | CS08 |
| Device Type | HCALCS |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS version earlier than v4.1 | GBCS v4.1 or later |
| DEFAULT - No specific XML criteria | Response Code - E57 | CS08 |

      - 3.8.124.2	Specific Data Items for this Request

ReadFirmwareVersion Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ExecutionDateTime | A User shall only add this Data Item to the Service Request where they require the Service Request to be executed at a future date and time. The UTC date and time the User requires the command to be executed on the Device ID - Date-time in the future that is either <= current date + 30 days or the date = ‘3000-12-31T00:00:00Z’ | xs:dateTime | No | None | UTC Date-Time |

Table 256 : ReadFirmwareVersion (sr:ReadFirmwareVersion) data items

      - 3.8.124.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for Execution Date Time validation.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’.

      - 3.8.124.4	Additional DCC System Processing

Upon receipt of a Response to this Service Request containing a Firmware Version value:

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model, but is different from that stored in the SMI, the DCC Systems shall update the Firmware Version in the SMI to the value returned by the Device. Note that updating the Firmware Version may also update the Device’s GBCS Version in the SMI.

  - If the target Device is CHF, the associated GPF Firmware Version shall also be updated.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Current” and the Read Firmware Version Service Request was not submitted by the Responsible Supplier, DCC Alert N49 shall be sent to the Responsible Supplier.

  - If the Firmware Version entry on the CPL for that Device Model has a status of “Removed”, the SMI Firmware Version shall be updated, but the Device Status shall not be set to ‘Suspended’. In this case DCC Alert N50 shall be sent to the Responsible Supplier as a warning.

  - Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current”, the DCC Systems shall update the Device Status to the status it held immediately prior to its “Suspension” and DCC Alert N29 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

- if the Target Device Type is ESME, GSME, HCALCS, PPMID or CHF and the Firmware Version returned by the Device does not match an entry on the CPL for that Device Model, DCC Alert N51 will be sent to the Responsible Supplier as a warning and the SMI Firmware Version will not be updated.

- if the Target Device Type is GPF and the GSME Firmware Version returned by the GPF is different from that stored in the SMI, DCC Alert N52 will be sent to the Responsible Supplier as a warning and the SMI Firmware Version will not be updated.

    - 3.8.125	Activate Firmware

      - 3.8.125.1	Service Description

| Service Request Name | - ActivateFirmware |  |  |
| --- | --- | --- | --- |
| Service Reference | - 11.3 |  |  |
| Service Reference Variant | - 11.3 |  |  |
| Eligible Users | Import Supplier (IS) Gas Supplier (GS) |  |  |
| Security Classification | Critical |  |  |
| BusinessTargetID - Device Type applicable to this request | Electricity Smart Meter (ESME) Gas Smart Meter (GSME) HAN Connected Auxiliary Load Control Switch (HCALCS) |  |  |
| Can be future dated? | Device |  |  |
| On Demand? | Yes |  |  |
| Capable of being DCC Scheduled? | No |  |  |
| Command Variants applicable to this Request (Only one populated | For Service Request 4 – Transform For Signed Pre-Commands, choice of: 5 - Send (Critical) 6 - Return for local delivery (Critical) 7 - Send and Return for local delivery (Critical) |  |  |
| Common Header Data Items | See clause 3.4.1.1 |  |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause3.5 for more details on processing patterns - Acknowledgement - Response to Transform Request - PreCommand Format - Service Response from Device – GBCSPayload - Service Response (from Device) - FutureDatedDeviceAlertMessage - Response to a Command for Local Delivery Request – LocalCommand Format Also see Response Section below for details specific to this request |  |  |
| Response Codes possible from this Service Request | See clause 3.5.10 for Common Response Codes |  |  |
| GBCS Cross Reference | Electricity (ESME) | Gas | Electricity (HCALCS) |
| GBCS version earlier than v4.1 MessageCode | 0x0012 | 0x0012 | N/A |
| GBCS version earlier than v4.1 Use Case | CS06 | CS06 | N/A |
| GBCS v4.1 or later MessageCode | 0x0012 | 0x0012 | 0x0012 |
| GBCS v4.1 or later Use Case | CS06 | CS06 | CS06 |

| GBCS Commands - Versioning Details |  |  |
| --- | --- | --- |
| DCC System creates the following GBCS Commands or Response Codes based on the following combinations, |  |  |
| Device Type | ESME |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS v1.0 or later |  |
| DEFAULT - No specific XML criteria | CS06 |  |
| Device Type | GSME |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS v1.0 or later |  |
| DEFAULT - No specific XML criteria | CS06 |  |
| Device Type | HCALCS |  |
| GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request | GBCS version earlier than v4.1 | GBCS v4.1 or later |
| Prior to DUIS v5.0: DEFAULT - No specific XML criteria | Response Code - E57 | Response Code - E57 |
| DUIS v5.0 or later: DEFAULT - No specific XML criteria | Response Code - E57 | CS06 |

      - 3.8.125.2	Specific Data Items for this Request

ActivateFirmware Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ExecutionDateTime | A User shall only add this Data Item to the Service Request where they require the Service Request to be executed at a future date and time. The date and time at which the firmware will be activated • Date-time in the future that is either <= current date + 30 days or the date = ‘3000-12-31T00:00:00Z’ | xs:dateTime | No | None | N.A |
| FirmwareHash | Hash calculated over the Manufacturer Image part of the FirmwareImage as defined by GBCS. The Firmware Hash as held in the CPL and presented in the format XX..XX (64 characters) where each X is one of the characters 0 to 9 or A to F. This data item must align with the value on the CPL (excluding the colon separator between octet values). Note that a hexBinary value of length 32 is defined as 32 octets; an octet is represented as 2 characters. | Restriction of xs:hexBinary (minLength = 32, maxLength = 32) | Yes | None | N/A |

Table 257 : ActivateFirmware (sr:ActivateFirmware) data items

      - 3.8.125.3	Specific Validation for this Request

No specific validation is applied for this Request, see clause 3.2.5 for general validation applied to all Requests and clause 3.10.2 for Execution Date Time validation.

      - 3.8.125.4	Additional DCC System Processing

The DCC Systems shall monitor all Responses received to this Service Request.

Where the DCC identifies any Response where the current Firmware Version returned by the Device matches an entry on the CPL for that Device Model and that Firmware Version is different to the value currently held in the Smart Metering Inventory for that Device, an update to the Smart Metering Inventory shall be made by the DCC.

The DCC Systems shall update the DeviceFirmwareVersion data item within the Smart Metering Inventory to record the new DeviceFirmwareVersion value for the specified Device ID received in the Response. Where the DeviceFirmwareVersion for the specified Device is now the current valid version, and the Device Status was ‘Suspended’ and the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Current” the DCC Systems shall update the Device Status to the status it held immediately prior to its Suspension” and DCC Alert N29 will be sent to the Responsible Supplier and to the Electricity Distributor or Gas Transporter.

If the Firmware Version returned by the Device matches an entry on the CPL for that Device Model with a status of “Removed”, the SMI Firmware Version will be updated, but the Device Status will not be set to ‘Suspended’. In this case DCC Alert N50 will be sent to the Responsible Supplier as a warning.

Note that if the Firmware Version returned by the Device is invalid (doesn’t match an entry on the CPL for that Device Model) DCC Alert N51 will be sent to the Responsible Supplier as a warning and the Smart Metering Inventory Firmware Version will not be updated.

    - 3.8.126	Update PPMID Firmware

      - 3.8.126.1	Service Description

| Service Request Name | - UpdatePPMIDFirmware |
| --- | --- |
| Service Reference | - 11.4 |
| Service Reference Variant | - 11.4 |
| Eligible Users | Import Supplier (IS) Gas Supplier (GS) |
| Security Classification | Non Critical |
| BusinessTargetID - Device Type applicable to this request | DCC Access Control Broker |
| Can be future dated? | No |
| On Demand? | No |
| Capable of being DCC Scheduled? | No |
| Command Variants applicable to this Request (Only one populated | 8 – Non-Device |
| Common Header Data Items | See clause 3.4.1.1 |
| Data Items Specific to this Service Request | See Specific Data Items Below |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause 3.5 for more details on processing patterns - Acknowledgement - Response to a Non-Device Service Request Also see Response Section below for details specific to this request |
| Response Codes possible from this Service Request | See clause 3.5.10 for Common Response Codes |
| GBCS Cross Reference | PPMID |
| GBCS MessageCode | N/A |
| GBCS Use Case | N/A |

      - 3.8.126.2	Specific Data Items for this Request

UpdateFirmware Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| FirmwareImage | The Firmware Image corresponding to the Firmware Version This is a base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image”. The Firmware Image is the full OTA Upgrade Image as defined in GBCS. Note that this includes the Manufacturer Image and the OTA Header information. Please see GBCS for details of how to construct the OTA Upgrade Image | Restriction of xs:base64Binary (max Length = 10240000) | Yes | None | N/A |
| FirmwareVersion | An identifier representing a firmware image that has been approved for release by the User concerned. The Firmware version as held in the CPL and presented in the format XXXXXXXX where each X is one of the characters 0 to 9 or A to F. This value must align with the firmware version value listed on the Central Products List (excluding the colon separator between octet values) and the Firmware Hash held on the CPL must match the Manufacturer Image Hash pursuant to clause 6.1(i) of the Service Request Processing Document. For avoidance of doubt, there is no direct comparison made between this FirmwareVersion value to the File Version value contained in the OTA Header (as defined by GBCS). | sr:FirmwareVersion (Restriction of xs:string (minLength=1, maxLength = 8)) | Yes | None | N/A |
| DeviceIDList | Comma separated list of Device IDs. Containing a minimum of 1 and a maximum of 50000 Device IDs, each as defined by sr:EUI | Restriction of xs:string (minLength = 23 maxLength = 1199999 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”) | Yes Minimum 1 and Maximum 50,000 Device IDs | None | N/A |

Table 257.1 : UpdateFirmware (sr:UpdateFirmware) data items

      - 3.8.126.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

For this Service Request and as an exception, the Authorisation Check associated to E5 allows the Device’s SMI Status to be ‘Suspended’

| Response Code | Response Code Description |
| --- | --- |
| E110401 | The Firmware Version is not approved. The Firmware version ID does not align with an entry on the Central Products List. |
| W110401 | The Update Firmware Warning contains between one and five lists of Device IDs for which the validation failed for each DeviceID: - InvalidDeviceIDList . The DeviceID does not exist or the User is not the Import Supplier or Gas Supplier for a Meter connected to the same HAN as the PPMID. - NotCommissionedDeviceIDList. The PPMID status is not ‘Commissioned’ or ‘Suspended’ or the Device is not a PPMID - NotApplicableFirmwareDeviceIDList. The Firmware Version is not applicable to the PPMID - OtherRequestsInProgressList. The PPMID already has another Firmware update request in progress. - InvalidGBCSVersionList. The CHF associated with the PPMID Device has a GBCS Version prior to v4.1. |
| E110402 | The firmware is not marked as active in the Central Products List and Smart Meter Inventory. |
| E110403 | The DCC Systems calculated Hash value over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request differs from that held in the CPL for the specified FirmwareVersion. |
| E110405 | The firmware image is not constructed as per the GBCS definition, i.e. the FirmwareImage does not contain both an OTA Header and a Firmware Image concatenated together, or the Length of the base64 encoded version of the GBCS defined Firmware “OTA Upgrade Image” exceeds 1024000. |

      - 3.8.126.4	Specific Data Items in the Response

This Service Request synchronous response is defined in the XSD DSPUpdateFirmwareWarning XML element, which contains the list(s) of Device IDs that failed DCC Authorisation / Validation.

The DCC shall return the following data items which are specific to this Service Request for Successful Responses

DSPUpdatePPMIDFirmwareWarning Definition:

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| InvalidDeviceIDList | Comma separated list of Device IDs for which the User ID is not the Import Supplier or Gas Supplier or the DeviceIds don’t exist. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = “([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2},)*([A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2}-[A-Fa-f0-9]{2})”)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotCommissionedDeviceIDList | Comma separated list of Device IDs which aren’t in a status of “Commissioned” or “Suspended” or the Device is not a PPMID. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| NotApplicableFirmwareDeviceIDList | Comma separated list of Device IDs for which the Firmware is not applicable. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| OtherRequestInProgressList | Comma separated list of Device IDs for which another Firmware Update request is in progress. Containing a minimum of 1 Device ID, each as defined by sr:EUI | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |
| InvalidGBCSVersionList | Comma separated list of Device IDs for which the CHF associated with PPMID device has a GBCS version prior to v4.1 | sr:DeviceIDList (Restriction of xs:string (minLength = 23 pattern = as for InvalidDeviceIDList above)) | No The Response will contain at least one of the 5 Lists and it could contain all of them | None | N/A |

Table 257.2 : DSPUpdatePPMIDFirmwareWarning (sr:DSPUpdatePPMIDFirmwareWarning) data items

      - 3.8.126.5	Additional DCC System Processing

The DCC Systems shall calculate a Hash over the Manufacturer Image part of the FirmwareImage provided by the User within the Service Request and ensure that the Hash calculated matches that held in the CPL for the specified FirmwareVersion. Where the DCC identifies any mismatch in Hash values, a Response Code of E110403 shall be returned to the User.

## Amend Table 279 ‘RequestIDType data items’ as follows:

    - PPMID Alert

      - Specific Data Items for this DCC Alert

PPMIDAlert Data Items Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| DeviceID | The Device ID of the PPMID that generated the Alert | sr:EUI | Yes | None | N/A |
| DeviceAlertCode | The Alert Code of the Alert generated by the PPMID Valid set: • 8F1E • 8F30 • 8F3D • 8F3E • 8F3F • 8F78 • 8F8B | xs:hexBinary | Yes | None | N/A |
| Timestamp | The timestamp at which the event that is responsible for this alert has occurred. | xs:dateTime | No | None | N/A |
| PPMIDFirmware UpgradeAlert | The Firmware Upgrade Alert sent by a PPMID | sr:PPMIDFirmwareUpgradeAlert (See PPMIDFirmwareUpgradeAlert Data Items Definition for details) | No | None | N/A |

Table 279 : RequestIDType data items

## Amend Table 284 ‘FirmwareVersionMismatch ( sr: FirmwareVersionMismatch) data items’ as follows:

    - 3.9.13	FirmwareVersionMismatch

      - 3.9.13.1	Specific Data Items for this DCC Alert

FirmwareVersionMismatch Data Items Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| DeviceID | The Device ID with a Firmware Version mismatch between the SMI and the Device | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| DeviceType | Device Type of the Device with a Firmware Version mismatch between the SMI and the Device Valid set: - ESME - GSME - CHF - PPMID - HCALCS | sr:DeviceType (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| FirmwareVersionSMI | N49, N50. The Device’s Firmware Version in SMI prior to its replacement with the value returned by the Device N51, N52. The Device’s Firmware Version in SMI The Firmware version as held in the CPL and presented in the format XXXXXXXX where each X is one of the characters 0 to 9 or A to F. This data item matches the value on the CPL (excluding the colon separator between octet values) | Restriction of xs:string (minLength = 1, maxLength = 8) | Yes | None | N/A |
| FirmwareVersionDevice | N49, N50. The Device’s Firmware Version held on the Device and now updated in the SMI post response returned by the Device The Firmware version as held in the CPL and presented in the format XXXXXXXX where each X is one of the characters 0 to 9 or A to F. This data item matches the value on the CPL (excluding the colon separator between octet values) N51. Firmware Version returned by the Device, which is unknown (it doesn’t match an item on the CPL) N52. GSME Firmware Version returned by the GPF | Restriction of xs:string (minLength = 1, maxLength = 8) | Yes | None | N/A |

Table 284 : FirmwareVersionMismatch ( sr: FirmwareVersionMismatch) data items

## Amend Table 296 ‘ServiceResponseVersionMismatch ( sr: ServiceResponseVersionMismatch) data items’ as follows:

ServiceResponseVersionMismatch Items Definition

| Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| RequestID | The Request ID of the Service Request incompatible with the User’s DUIS XSD version | sr:RequestIDType (see clause 3.10.1.1) | Yes | None | N/A |
| ServiceReference | The Service Reference of the Service Request incompatible with the User’s DUIS XSD version | xs:string | Yes | None | N/A |
| ServiceReferenceVariant | The Service Reference Variant of the Service Request incompatible with the User’s DUIS XSD version | xs:string | Yes | None | N/A |
| ServiceRequestDUISVersion | The DUIS Version of the Service Request incompatible with the User’s DUIS XSD version Valid Values; - 1.0 - 2.0 - 3.0 - 3.1 - 4.0 - 5.0 | xs:string | Yes | None | N/A |

Table 296 : ServiceResponseVersionMismatch ( sr: ServiceResponseVersionMismatch) data items

## Amend the tables within Sections 3.9.20, 3.9.21 and 3.9.22 as follows:

    - 3.9.20	FirmwareUpgradeRequested

      - 3.9.20.1	Specific Data Items for this DCC Alert

FirmwareUpgradeRequested Data Items Definition

| Data Type / Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| DeviceType | The type of the Device the Firmware is applicable to. Valid set: - PPMID | sr:DeviceType (Restriction of xs:string (Enumeration)) | Yes | None | N/A |
| FirmwareVersion | The firmware version of the Upgrade Image | sr:FirmwareVersion (Restriction of xs:string) | Yes | None | N/A |
| DeviceList | List of Device IDs that successfully passed validation by DSP and CSP. | sr:DeviceIDList | Yes | None | N/A |

Table 297b : FirmwareUpgradeRequested Data Items

    - 3.9.21	CSPFirmwareDeliveryStatus

      - 3.9.21.1	Specific Data Items for this DCC Alert

CSPFirmwareDeliveryStatus Data Items Definition

| Data Type / Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| UpdateFirmwareRequestID | Request ID of the Update Firmware Service Request associated to this Alert | sr:RequestIDType (see clause 3.10.1.1) | Yes | None | N/A |
| CommsHubDeviceID | The Device ID of the Comms Hub that is the destination for the delivery of the firmware image | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| TargetDeviceID | The ID of the Device the firmware image is targeted at | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| FirmwareVersion | The firmware version of the Upgrade Image | sr:FirmwareVersion (Restriction of xs:string) | Yes | None | N/A |
| Timestamp | The time at which this status update was recorded by the CSP. | xs:dateTime | Yes | None | N/A |
| CommsHubTransferStatus | Indicates whether a firmware image has been delivered to a Comms Hub. Valid set: For DCC Alert N61: - Success For DCC Alert N60: - Failure (N60) | Restriction of xs:string (Enumeration) | Yes | None | N/A |

Table 297c : CSPFirmwareDeliveryStatus Data Items

    - 3.9.22	CommsHubAlert

      - 3.9.22.1	Specific Data Items for this DCC Alert

CommsHubAlert Data Items Definition

| Data Type / Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| CommsHubDeviceID | The Device ID of the Comms Hub that generated the Device Alert | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| AlertCode | The Alert Code of the Alert generated by the Comms Hub. Valid set: For Firmware Transfer Alert: - 8F89 - 8F8A | xs:hexBinary | Yes | None | N/A |
| Timestamp | The timestamp at which the event that is responsible for this alert has occurred. | xs:dateTime | No | None | N/A |
| FirmwareTransferAlert | The Alert sent by a Comms Hub to report transfer status of a firmware image to the target Device. Only to report a firmware distribution status. | sr:FirmwareTransferAlert (See FirmwareTransferAlert Data Items Definition for details) | No | None | N/A |

Table 297d : CommsHubAlert Data Items

FirmwareTransferAlert Data Items Definition

| Data Type / Data Item | Description / Allowable values | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| UpdateFirmwareRequestID | Request ID of the Update Firmware Service Request associated to this Alert | sr:RequestIDType (see clause 3.10.1.1) | No | None | N/A |
| OtherDeviceID | The ID of the Device the firmware image is targeted at | sr:EUI (See clause 3.10.1.3) | Yes | None | N/A |
| FirmwareVersion | The firmware version of the transferred or discarded Upgrade Image | sr:FirmwareVersion (Restriction of xs:string) | Yes | None | N/A |
| TransferResponseCode | Valid set: For Alert Code 0x8F8A: - FileTransferSuccess For Alert Code 0x8F89: - FirmwareImageDiscarded - HardwareVersionMismatch - FileTransferFailure | Restriction of xs:string (Enumeration) | Yes | None | N/A |

Table 297e : FirmwareTransferAlert Data Items

## Add the updated DUIS v5.0 XML Schema as follows:

Appendix AF ‘Message Mapping Catalogue’

Where the associated change was impacted by the SECMP0007, those changes have been redlined against the approved SECMP0007 legal text and confirmed following the associated header.

The other changes have been redlined against Appendix AF version 4.0.

These changes will be applied to Appendix AF version 5.0.

## Amend Table 10 ‘Device Alert GBCS Message Codes’ as follows:

    - 4.2.1	Message codes for Device Alerts

All Device Alerts include both a Message Code within the Header element (see section 3) and an Alert Code within DeviceAlertContent.

Many Device Alerts will share the same common message codes, being either 0x1000 (denoting Critical) or 0x1001 (denoting Non Critical). The Device Alerts that use a specific Message Code instead of such generic codes shall be as defined in GBCS and as listed in Table 10 as set out immediately below.

Device Alerts which use the generic message codes are defined in GBCS and are not listed below.

The MMC Output Format for all Device Alerts which have specific Message Codes shall include additional information within the Payload data item, as set out in Section 4.2.2 of this document. Message Codes shall conform to the “xs:hexBinary” XML type and shall omit any leading “0x” value that would ordinarily appear in the corresponding GBCS code as shown in the table below.

| GBCS Version | Message Code | Purpose | Alert Codes |
| --- | --- | --- | --- |
| >= 1.0 | 0x0061 | GBCS Use Case ECS68 ESME Critical Sensitive Alert (Billing Data Log) | 0x8F0A |
| >= 1.0 | 0x0067 | GBCS Use Case ECS80 Supply Outage Restore Alert from ESME | 0x8F35, 0x8F36, 0x8F37, 0x8F38, 0x8F39, 0x8F3A, 0x8F3B, 0x8F3C |
| >= 1.0 | 0x008B | GBCS Use Case GCS53, GSME Push Billing Data Log as an Alert | 0x8F0A |
| >= 1.0 | 0x00CA | Future Dated Firmware Activation Alert | 0x8F66, 0x8F67 |
| >= 1.0 | 0x00CB | Future Dated Updated Security Credentials Alert | 0x8F66, 0x8F67 |
| >= 1.0 | 0x00CC | Future Dated Execution Of Instruction Alert (DLMS COSEM) | 0x8F66, 0x8F67 |
| >= 1.0 | 0x00CD | Future Dated Execution Of Instruction Alert (GBZ) | 0x8F66, 0x8F67 |
| >= 1.0 and modified in 4.1 | 0x00CE | Firmware Distribution Receipt Alert (ESME/HCALCS) | 0x8F72, 0x8F1C |
| >= 1.0 | 0x00CF | Firmware Distribution Receipt Alert (GSME) | 0x8F72, 0x8F1C |
| >= 2.0 | 0x00F0 | Meter Integrity Issue Warning Alert - ESME | 0x81A0 |
| >= 2.0 | 0x00F2 | Meter Integrity Issue Warning Alert – GSME | 0x81A0 |
| >= 1.0 | 0x1000 | Generic Critical Alert | As set out in GBCS |
| >= 1.0 | 0x1001 | Generic Non Critical Alert | As set out in GBCS |
| >= 4.0 | 0x0120 | GBCS Use Case ECS100 Command not supported by Device. | 0x8F85 |
| >= 4.0 | 0x0121 | GBCS Use Case ECS101 Limit APC [n] Level Command processed | 0x8F86 |
| >= 4.0 | 0x0122 | GBCS Use Case ECS102 Limit APC [n] Level ended or cancelled | 0x8F87 |
| >= 4.0 | 0x0123 | GBCS Use Case ECS200 Operational Update. | 0x8F88 |
| >= 4.0 | 0x0124 | Future Dated Updated Security Credentials Alert (Load Controller only) | 0x8F66, 0x8F67 |

Table 10 : Device Alert GBCS Message Codes

## Amend Table 11 ‘Device Alerts with Additional Payload’ as follows:

    - 4.2.2	Payload in Device Alerts

The MMC Message Format for all Device Alerts which contain additional Payload data are as listed in Table 11 as set out immediately below. Where a Device Alert is associated with a specific GBCS Use Case, this is indicated in Table 11.

| GBCS Version | Alert Code(s) | Description | GBCS Use Case (where applicable) | Message Code | Section detailing XML contents | Applicable to SMETS1 Alerts? |
| --- | --- | --- | --- | --- | --- | --- |
| >= 1.0 | 0x8F0A | Billing Data Log Updated (Electricity) | ECS68 ESME Critical Sensitive Alert (Billing Data Log) | 0x0061 | 6.2 | No |
| >= 1.0 | 0x8F0A | Billing Data Log Updated (Gas) | GCS53, GSME Push Billing Data Log as an Alert | 0x008B | 6.2 | No |
| >= 1.0 and modified in 4.1 | 0x8F1C | Firmware Verification Status (Firmware Distribution Receipt) - Failed | CS05b | 0x00CE (Firmware Distribution Receipt Alert (ESME/HCALCS)), 00CF (Firmware Distribution Receipt Alert (GSME)) | 6.1 | Yes |
| >= 1.0 | 0x8F35 0x8F36, 0x8F37, 0x8F38, 0x8F39, 0x8F3A, 0x8F3B, 0x8F3C | Supply Outage Restored Alert | ECS80 Supply Outage Restore Alert from ESME | 0x0067 | 6.3 | No |
| >= 1.0 and modified in 4.0 | 0x8F66 | Future-Dated Command Outcome Action Successful | Any GBCS Use Case that supports Future Dated Services. | 0x00CA (Future Dated Firmware Activation Alert), 0x00CB (Future Dated Updated Security Credentials Alert), 0x00CC (Future Dated Execution Of Instruction Alert - DLMS COSEM) or 0x00CD (Future Dated Execution Of Instruction Alert - GBZ) GBCS v4.0 or later: 0x0124 (Future Dated Update Load Controller Security Credentials Alert) | 6.4 | No |
| >= 1.0 | 0x8F67 | Future-Dated Command Outcome Action Failed | Any GBCS Use Case that supports Future Dated Services. | As for Alert Code 0x8F66 | 6.4 | No |
| >= 1.0 and modified in 4.1 | 0x8F72 | Firmware Verification Status (Firmware Distribution Receipt) - Successful | CS05b | 0x00CE (Firmware Distribution Receipt Alert (ESME/HCALCS)), 00CF (Firmware Distribution Receipt Alert (GSME)) | 6.1 | Yes |
| >= 2.0 | 0x81A0 | Smart Meter Integrity Issue – Warning | Smart Meter Integrity Issue – Warning from ESME or GSME | 0x00F0 (Meter Integrity Issue Warning Alert – ESME) 0x00F2 (Meter Integrity Issue Warning Alert – GSME) | 6.5 | No |
| >= 4.0 | 0x8F85 | ECS100 Command not supported by Device | ECS100 | 0x0120 | 6.6 | No |
| >= 4.0 | 0x8F86 | ECS101 Limit APC [n] Level Command processed | ECS101 | 0x0121 | 6.7 | No |
| >= 4.0 | 0x8F87 | ECS102 Limit APC [n] Level ended or cancelled | ECS102 | 0x0122 | 6.8 | No |
| >= 4.0 | 0x8F88 | ECS200 Operational Update | ECS200 | 0x0123 | 6.9 | No |

Table 11 : Device Alerts with Additional Payload

## Amend Table 242 ‘Read Firmware Version MMC Output Format Header data items’ as follows:

This change has been drafted against the SECMP0007 legal text

GBCS version earlier than v4.1:

| Data Item | Electricity Response | Gas Response |
| --- | --- | --- |
| GBCSHexadecimalMessageCode | 0x0059 | 0x0084 |
| GBCS Use Case | ECS52 | GCS38 |
| SupplementaryRemotePartyID | ra:EUI (see clause 2.4.1) Where originator is Unknown Remote Party |  |
| SupplementaryRemotePartyCounter | xs:nonNegativeInteger Where originator is Unknown Remote Party |  |

Table 242 : Read Firmware Version MMC Output Format Header data items – GBCS version earlier than v4.1

## Amend new Table 242 (now 242.1) ‘Read Firmware Version MMC Output Format Header data items – GBCS v4.1 or later’ as follows:

This change has been drafted against the SECMP0007 legal text

GBCS v4.1 or later:

| Data Item | Electricity Response |  |  | Gas Response |
| --- | --- | --- | --- | --- |
| DeviceType | ESME/ Communications Hub Function | PPMID | HCALCS |  |
| GBCSHexadecimalMessageCode | 0x0059 | 0x0129 | 0x0129 | 0x0084 |
| GBCS Use Case | ECS52 | CS08 | CS08 | GCS38 |
| SupplementaryRemotePartyID | ra:EUI (see clause 2.4.1) Where originator is Unknown Remote Party | ra:EUI (see clause 2.4.1) Originator is always an Unknown Remote Party as Command created by ACB |  | ra:EUI (see clause 2.4.1) Where originator is Unknown Remote Party |
| SupplementaryRemotePartyCounter | xs:nonNegativeInteger Where originator is Unknown Remote Party | xs:nonNegativeInteger Originator is always an Unknown Remote Party as Command created by ACB |  | xs:nonNegativeInteger Where originator is Unknown Remote Party |

Table 242.1 : Read Firmware Version MMC Output Format Header data items – GBCS v4.1 or later

## Amend Table 244 ‘Activate Firmware Version MMC Output Format Header data items’ as follows:

This change has been drafted against the SECMP0007 legal text

GBCS version earlier than v4.1:

| Data Item | Electricity Response | Gas Response |
| --- | --- | --- |
| GBCSHexadecimalMessageCode | 0x0012 | 0x0012 |
| GBCS Use Case | CS06 | CS06 |
| Timestamp | xs:dateTime |  |

Table 244 : Activate Firmware Version MMC Output Format Header data items

## Amend new Table 244 (now 244.1) ‘Activate Firmware Version MMC Output Format Header data items’ as follows:

This change has been drafted against the SECMP0007 legal text

GBCS v4.1 or later:

| Data Item | Electricity Response |  | Gas Response |
| --- | --- | --- | --- |
| DeviceType | ESME | HCALCS |  |
| GBCSHexadecimalMessageCode | 0x0012 | 0x0012 | 0x0012 |
| GBCS Use Case | CS06 | CS06 | CS06 |
| Timestamp | xs:dateTime |  |  |

Table 244.1 : Activate Firmware Version MMC Output Format Header data items – GBCS v4.1 or later

## Add Table 248.1 ‘Firmware Verification Device Alerts MMC Output Format Header data items – GBCS v4.1 or later’ as follows:

# 6	Device Alert MMC Output Format definitions

  - 6.1	Firmware Verification Status (Alert Codes 0x8F1C and 0x8F72)

Upon completion of a Firmware verification performed as part of the distribution of Firmware upgrades, the associated Device Alert shall be one of the following:

- Device Alert with Alert Code 0x8F1C, indicating that the image verification failed; or

- Device Alert with Alert Code 0x8F72, indicating that the image verification was successful.

The same Payload element is included in the case of Firmware image verification success or failure.

The xml type within the DeviceAlertMessagePayload element is FirmwareVerificationDeviceAlertType. The header and body data items appear as set out immediately below.

    - 6.1.1	Specific Header Data Items

GBCS version earlier than v4.1:

| Data Item | Electricity Alert | Gas Alert |
| --- | --- | --- |
| GBCSHexadecimalMessageCode | 0x00CE | 0x00CF |
| GBCS Use Case | CS05b | CS05b |

Table 248 : Firmware Verification Device Alerts MMC Output Format Header data items – GBCS version earlier than v4.1

GBCS v4.1 or later:

| Data Item | Electricity Alert |  | Gas Alert |
| --- | --- | --- | --- |
| DeviceType | ESME | HCALCS |  |
| GBCSHexadecimalMessageCode | 0x00CE | 0x00CE | 0x00CF |
| GBCS Use Case | CS05b | CS05b | CS05b |

Table 248.1 : Firmware Verification Device Alerts MMC Output Format Header data items – GBCS v4.1 or later

## Add the updated MMC v5.0 XML Schema as follows:

For a SMETS2 PPMID, DCC Systems will create a GBCS Command conforming to GBCS Use Case CS08 regardless of the GBCS version that pertains to the Device Model recorded in the SMI for the Business Target Device ID specified in the Service Request. The Response Code E57 check is not applicable for this SRV, where the Ttarget Device Type is a PPMID.