Smart Metering Implementation Programme

Technical Specification Change Resolution Proposal (CRP)

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 16 June 2022 |
| --- | --- |
| Issue Resolution Proposal | CRP655 |
| Issues Log ID | TS1513 |
| Issue Title | Processing for gas meters without a Valve |
| Source | TSIRS / BEIS |
| Date Raised | 18 February 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GSMETS v4.3 |

Description:

Issue TS1420 was raised to establish the intended behaviour from gas meters without a stop valve.  The explanation provided was within the constraints of the specs at that time (GBCS v4.1 and GSMETS v4.3).

This CRP addressed the changes required to support gas meters without a valve with respect to all processing that relates to a valve being present e.g., prepayment based processing.

Proposed Implementation:

As expressed in the Issue description above, this CRP is intended to establish the agreed behaviour from GSME that does not have a Valve to Enable or Disable the flow of gas to Premises.

Originally in SMETS2 (from GSMETS v2.0) the requirements to Lock, Enable, Disable or Arm Supply, functionality that requires the presence of a Valve, applied only to GSME installed at Domestic Premises.  The SEC modification SECMP0048 extended the exemption to allow Large Gas meters (e.g., U16) installed at Domestic Premises to not require a Valve (from GSMETS v3.1).  This was on the basis that no manufacturer had intended to produce such a GSME and yet Large Gas Meters were occasionally required in the domestic setting.  The requirement for GSME other than Large Gas Meters installed at Domestic Premises, to have a Valve remains unchanged.

In outline, the proposed position, as discussed and support at the TSIRS meeting on 14 April 2022, is to add overarching clause identifying requirements not applicable to a GSME without a Valve more explicitly in GSMETS specification (including the associated GBCS requirements).

Interoperability and / or Compatibility:

There are no interoperability issues related to this change.  Also, the changes do not affect DCC Systems or User systems, nor do they require any changed interactions with them, the intention is purely to clarify what is expected should a SMETS2 GSME without a Valve be used.

Required Changes to Documentation:

The following changes are to be applied to GSMETS at Section 4.1 as shown by the mark-up text below:

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | None |
| Affected Parties | Consumers: No impact |
|  | DCC Users: Manufacturers and Users of GSME without a Valve |
|  | DCC & Testing: No obvious impact on DCC Testing |
|  | Additional impact on functionality for installed devices: No impact. |