Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal (IRP)

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 27 June 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP657 |
| Issues Log ID | TS1526 |
| Issue Title | 0x8F66 & 0x8F67 for future dated CS02g command |
| Source | L+G |
| Date Raised | 15 April 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.1 |

Description:

We suspect there is an omission in GBCS table 16.2 rows 87 + 88. We believe the 0x8F66 and 0x8F67 alerts should also be sent to LoadController in case of Future dated CS02g command with message code 0x0125 i.e loadControllerByLoadController

As per the logic in IRP572, why would the CS02g Load Controller security credentials replacement command be sent to the LoadController but the alert only sent to the supplier?

Proposed Position:

As per this IRP description above, we agree that for both Event / Alert Codes 0x8F66 and 0x8F67, the Known Remote Party Role (so column ‘P’ in the GBCS, Table 16.2) should say:

"Transitional Change of Supplier, Supplier or Load Controller (not CHF), or WAN Provider (CHF only)",

rather than just:

“Transitional Change of Supplier or Supplier (not CHF) or WAN Provider (CHF only)”,

As is currently.  Whilst it should be noted there is currently no ‘Load Controller’ Remote Party Role allowed in DCC systems or the SEC, we do consider that the processing intent should be reflected in GBCS.  Therefore, this IRP will correct that omission.

Interoperability and / or Compatibility

This IRP is a correction to reference a Remote Party Role that whist allowed for, is currently not in use.  Therefore, there can be no interoperability impacts between Systems and Devices, nor implication on DCC’s current systems.

Required Changes to Documentation:

Modify GBCS, Table 16.2, at cells ‘P87’ and ‘P88’, to be:

‘Transitional Change of Supplier, Supplier or Load Controller (not CHF), or WAN Provider (CHF only)’.

Once applied, the resulting Table 16.2 is as embedded below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |
| Affected Parties | Consumers: No impact identified |
|  | DCC Users: No expected impact identified |
|  | DCC & Testing: No impact identified. Note that Load Controller DCC User Role not yet implemented (as of 2022) |
|  | Impact on additional functionality in installed devices: No impact identified |