Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 11 November 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP660 |
| Issues Log ID | TS1534 |
| Issue Title | GBCS to identify endianness of firmwareVersion within CS09 |
| Source | DCC / BEIS |
| Date Raised | 29 June 2022 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v4.1 |

Description:

When reading GBCS section 3.3.8 “Octet Endianness – Informative” lines 223 to 225 state:

Table 11.9.3 denotes the “Data Type” of “firmwareVersion” as “OCTET-STRING”.

Someone intimately familiar with GBCS would know that this paragraph is the key one, and that you should go no further in the specification.  However, if one does go a little further, the very next paragraph (lines 226 to 230 state):

One could infer from the document structure that GBCS is calling an exception here to the content on lines 223-225 and commence down the following rabbit hole that points you towards the ZSE and needing to follow the implementation within:

Table 11.9.3 notes for “firmwareVersion” that “The value shall be four octets in length and shall correspond to the File Version field in the ZSE OTA Header structure.”

Table 11.2.3 makes note that the “File Version” is “As per the Zigbee OTA specification”

Zigbee Smart Energy section 11.4.2 “OTA Header Format” notes that “File Version” is an “Unsigned 32-bit integer” (and thus not an octet string) and notes that “The endianness used in each data field SHALL be little endian in order to be compliant with general ZigBee messages.”

Ask

With the above in mind, do we feel the specification is clear and prevents readers from implementing incorrect endianness for attributes such as “firmwareVersion”?  Perhaps it would be good to have an example included where appropriate?

Proposed Position:

We propose to make clarifications in the GBCS Sections for the Commands and Alerts associated with the OTA header and other aspects of Firmware handling to reflect that Big Endianness should exceptionally be used in this context despite ZigBee suggesting Little Endianness.

Also, given that endianness is often misunderstood, we propose to make additional Informational text improvements to Informative Sections of GBCS to improve the clarity of numerical and octet string formatting.

Interoperability and / or Compatibility

All devices implement the ZigBee firmwareVersion (as represented in GBCS Commands) as set out in this IRP, as such there are no interoperability or compatibility issues.

[V0_2 of this IRP removed the drafting note from statement above following TSIRS review on 10/11/22 and agreement to close]

Required Changes to Documentation:

- To improve clarity in the use of numerical hexadecimal representation in the context of an octet string formatting throughout GBCS, adding additional text in the Informative Section:

At GBCS, Section 3.3.3, add the additional paragraph as shown in the mark-up below:

<run on as is…>

- To improve the requirement for the representation of numbers and to correct existing typos, in the ‘Encoding and length of variable length unsigned integers’ Section:

at GBCS, Section 3.3.6, make the changes as shown in the mark-up text below:

<run on as is…>

- To improve the clarity of the octet string representation of OTA Header elements in the Informative text:

at GBCS, Section 3.3.8, make the changes as shown in the mark-up text below, which will create one single running paragraph, instead of the two bullets:

<… run on from existing text>

<run on as is… >

- To make the language unequivocal about using big-endian octet string representation for firmwareVersion across firmware Commands.  Also, clarifying that the firmware version is not specific to ZSE, and correcting representation of an octet string, where necessary:

  - at GBCS, Section 11.5.2.3, within the ASN-1 structure, make the changes shown in the mark-up below:

<…run on from existing text

<run on as is…>

  - at the GBCS, Section 11.7.3 in Table 11.7.3, make the changes shown in the mark-up below:

  - at GBCS, Section 11.7.4, within the ASN-1 structure, make the changes shown in the mark-up below:

<…run on from existing text

<run on as is…>

  - at GBCS, Section 11.8.4, within the ASN-1 structure, make the changes shown in the mark-up below:

<…run on from existing text

<run on as is…>

For additional drafting clarity this results in the following, shown without mark-up:

  - at GBCS, Section 11.9.3, in Table 11.9.3, make the changes shown in the mark-up below:

<run on as is…>

  - at GBCS, Section 11.9.4, within the ASN-1 structure, make the changes shown in the mark-up below:

<…run on from existing text

<run on as is…>

- To make the language in the GBCS, Mapping Table, unequivocal about using big-endian octet string representation text:

  - at GBCS, Section 20, Table 20, in the SMETS required objects tab, cell ‘CG85’, make the following changes shown in the mark-up:

‘n = 5, the first octet having the value 0x04 and the remaining 4 octets being the value ofbig-endian representation of the unsigned 32-bit integer Firmware Version’

  - at GBCS, Section 20, Table 20 in the SMETS required objects tab, cell ‘K85’, insert the text as shown in mark-up:

‘The Device's now current firmware version. The value shall be four octets in length and shall correspond to the big-endian representation of the unsigned 32-bit integer File Version field in the ZSE OTA Header structure.’

  - at GBCS, Section 20, Table 20 in the ZigBee Commands tab, cell ‘I378’ make the following changes:

‘Firmware Version (length is 5, the first octet having the value 0x04 and the remaining 4 octets being the value ofbig-endian representation of the unsigned 32-bit integer Firmware Version)’

These changes will result in the updated Table 20 and the updated Use Cases, ECS52 and GCS38, as embedded in the table below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |
| TSIRS view of impact | Consumers: No impact identified |
|  | DCC Users: Refer to ’Interoperability and / or Compatibility’ Section above. |
|  | DCC & Testing: Refer to ’Interoperability and / or Compatibility’ Section above. |
|  | Non-SMETS functionality in installed devices: Refer to ’Interoperability and / or Compatibility’ Section above. |

An Octet String has no concept of endianness, as it is a string of octets.  Each octet in the Octet String also has no concept of endianness, as endianness concerns representation of a number of more than one most / least significant bytes (e.g., an unsigned integer 256 or greater).

A number may need to be represented as a sequence of octets (i.e., bytes) in a message payload.  When representing a number as a sequence of octets, it is endianness that determines, which significant byte goes first, i.e., with Big endian the most significant byte goes first, and with Little endian the least significant byte goes first.