Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 23 November 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP662 |
| Issues Log ID | TS1548 |
| Issue Title | CH Time indication when CH has lost sync with CSP time server |
| Source | DCC |
| Date Raised | 19 October 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.2 |

Description:

Can CH continue to indicate valid time to HAN devices even though it cannot maintain its time through WAN connection to CSP time servers and hasn’t reset its internal clock?

Background:

Comms Hub maintains time synchronisation with CSP time servers through WAN connectivity.  When a CH loses WAN connectivity there are two cases to consider as to how it presents time to HAN devices:

Case 1: CH, after losing connectivity with CSP time servers, reboots and it resets its internal clock. In this situation it will indicate 0xFFFFFFFF to HAN devices.

Case 2: CH after losing connectivity with CSP time servers, does not reboot. CH assumes its clock remains valid & indicates valid time to devices in the HAN.

Ask:

Assuming case 2 above is supported then GBCS (Sec 9.1.3) needs to be clarified, as this will enable all devices to remain in sync with CH and for PPMID to display consumption data without impacting end customer.

In both cases, when CH regains time sync with CSP time servers, it will update time and that will enable devices in the HAN to synchronise.

Proposed Position:

The case 1 as laid out in the Issue Description above conforms to the both the design intention and to CHTS and GBCS requirements.  Currently, however, the case 2 does not.  Having reflected on case 2 approach from a consumer experience point of view and in consideration of the fact this is what the current CH actually do, we support the suggestion.  Further, at the TSIRS Group meeting in October 2022, DCC presented the case, and no objection was made.

Therefore, we propose to modify the requirements for CH in setting the ZCL Time Cluster in GBCS, Section 9.1.3, to allow the existing CH behaviour.

Interoperability and / or Compatibility

There is no effect on existing device interoperability and DCC systems are not affected by this IRP662.

Required Changes to Documentation:

At GBCS, Section 9.1.3, make the changes as shown in the mark-up below:

<… run on from existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |
| TSIRS view of impact | Consumers: No impact identified, changing GBCS ensures Case 2 (which keeps consumer impact low) is reflected in GBCS |
|  | DCC Users: No impact identified. |
|  | DCC & Testing: No impact identified as there is no change in CH behaviour. |
|  | Non-SMETS functionality in installed devices: No impact identified |

See slide pack embedded in the table at the end of this IRP