Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides DESNZ’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 22 February 2023 |
| --- | --- |
| Issue Resolution Proposal | IRP663 |
| Issues Log ID | TS1559 |
| Issue Title | Include Update Load Controller Security Credentials Command |
| Source | DESNZ (nee BEIS) |
| Date Raised | 2 December 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.2 |

Description:

GBCS v4.2, Section 4.3.2.5 (at Line 537) states:

“... shall not allow execution of any Remote Party Command other than an Update Security Credentials Command or Provide Security Credentials Command, nor issue any Remote Party Alerts, in relation to a Remote Party Role where the Remote Party Role stored in a Trust Anchor Cell is different than that of the Trust Anchor Cell itself”

The intention following the introduction of the SAPC device, is where a Device supports the processing of Remote Party Messages, that Device, is to also allow for the execution of an Update Load Controller Security Credentials Remote Party Command.

The part of GBCS, Section 4.3.2.5, referred to above needs to be amended to include 'Update Load Controller Security Credentials Command'

Proposed Position:

As referenced in the Issue Description this IRP proposes to add 'Update Load Controller Security Credentials Command' to the other Commands within the exception where a Device supports the processing of Remote Party Messages.

Interoperability and / or Compatibility

There is no effect on existing device interoperability and DCC systems are not affected by this IRP663.

Required Changes to Documentation:

At GBCS, Section 4.3.2.5, make the changes as shown in the mark-up below:

<… run on from existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | None |
| TSIRS view of impact | Consumers: No impact identified |
|  | DCC Users: No impact identified |
|  | DCC & Testing: No impact identified |
|  | Non-SMETS functionality in installed devices: No impact identified |