Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides DESNZ’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 22 February 2023 |
| --- | --- |
| Issue Resolution Proposal | IRP664 |
| Issues Log ID | TS1573 |
| Issue Title | Resolving Max Credit and Max Meter Balance Threshold ambiguity |
| Source | DESNZ (nee BEIS) |
| Date Raised | 20 January 2023 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.2 |

Description:

Further to the Issue TS1549 and also with reference to TS0319, the units and scaler for Maximum Credit Threshold and Maximum Meter Balance Threshold are not clearly defined in GBCS leading to ambiguity and potential differences between Devices.  This IRP provides clarification and corrected wording for incorporation into GBCS aligning to processing in place in DUIS and DCC systems.

Proposed Position:

GBCS, Section 14.1, defines the Maximum Credit Threshold and the Maximum Meter Balance Threshold, stating in both cases that the “value shall be interpreted by the Device in Currency Units”, with the Currency Units defined as GB Pounds (or EU Euro).  However, DUIS was specified in 1000th pence / cent, leading to ambiguity with what the actual associated value should be.  This has since manifested in devices representing the associated value for the attributes differently.

From the results of a recent survey of meter manufacturers (through EUA and BEAMA) it is observed that the significant majority of ESME and GSME represent the associated value in 1000th pence / cent, however, some represent the associated value in whole pounds / euro.

In 2015 the Issue TS0319 (mentioned in the description above) was raised by DCC to establish what the scalers and units should be for several financial attributes, including Maximum Credit Threshold and Maximum Meter Balance Threshold.  Whilst the intent for these two attributes was originally to be represented in whole pounds / euro, i.e. with no fractional part, with the explanation document associated with TS0319 stating the scaler would be: ‘0 as per table in GBCS section 14.1’, this was not carried forward into the GBCS.  DCC have developed DUIS and their systems to work in 1000th pence / cent and as such, for any ESME and GSME that expect to receive values in whole pounds / euro, there is no way for an energy supplier to configure useful values using the relevant Service Request.

Given most ESME and GSME are configured to expect the values to be represented in 1000th pence / cent and with the DUIS service request also working to 1000th pence / cent, then we propose to amend the GBCS to clarify these attributes are to be in 1000th pence / cent too.

Interoperability and / or Compatibility

DCC systems are not affected by this IRP664.  Where devices expect either or both threshold values to be anything other than 1000th pence / cent, then those devices are directly affected by this clarification and will require modification to allow appropriate configuration of these attributes.

Required Changes to Documentation:

- At GBCS, Section 14.1, make the changes as shown in the mark-up below:

<… run on from existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | TS1549 description: TS0319 response: |
| TSIRS view of impact | Consumers: No impact identified |
|  | DCC Users: No impact identified |
|  | DCC & Testing: No impact identified |
|  | Non-SMETS functionality in installed devices: No impact identified |

A copy of the TS0319 explanation as previously published is embedded in the table below.

We intend to amend the explanation at TS0319 in due course.