Version: T7.0

Appendix T

DCCKI Interface Design Specification

# INTRODUCTION

### Document Purpose

  - Pursuant to Section L13.13 of the Code (DCCKI Interface Design Specification), this document is the DCCKI Interface Design Specification (DCCKI IDS).

## DCCKI SERVICE INTERFACE

### Submission of DCCKI Certificate Signing Requests and Issuance of DCCKI Infrastructure Certificates

  - In order to request Issuance of a DCCKI Infrastructure Certificate, a Party or RDP that is a DCCKI Eligible Subscriber shall follow the processes defined in the DCCKI RAPP.

  - The DCC shall ensure that all DCCKI Certificate Signing Requests are required to be formatted in accordance with the PKCS#10 standard as set out in the DCCKI RAPP. The structure of a DCCKI Certificate Signing Request is defined in section 3 of this DCCKI IDS.

  - No further provision is made in this document in relation to requesting and obtaining DCCKI Infrastructure Certificates.

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

  - Not used

## DCCKI CERTIFICATE SIGNING REQUEST

### Information to be contained within DCCKI Certificate Signing Requests

  - A DCCKI Certificate Signing Request in respect of a DCCKI Infrastructure Certificate for the purpose of signing SAML assertions to the DCC shall contain the information set out in the table immediately below:

| Subject | Attributes | Values |
| --- | --- | --- |

| Version |  | V3 (as per RFC 2986) |
| --- | --- | --- |

| Subject | Common Name | <Party Signifier> |
| --- | --- | --- |

| Subject Public Key Info | Public Key Algorithm | RSA |
| --- | --- | --- |
|  | Subject Public Key (2048 bits) | Public Key value |

| Key Usage | Criticality | True |
| --- | --- | --- |
|  | Key Usage | digitalSignature |

| Signature Algorithm |  | rsa-with-SHA256 |
| --- | --- | --- |

  - A DCCKI Certificate Signing Request in respect of a DCCKI Infrastructure Certificate for the purpose of establishing TLS communications to the DCC shall contain the information set out in the table immediately below:

| Subject | Attributes | Values |
| --- | --- | --- |

| Version |  | V3 (as per RFC 2986) |
| --- | --- | --- |

| Subject | Common Name | Fully Qualified Domain Name configured on the Policy Enforcement Point |
| --- | --- | --- |
|  | Organisation Identifier | Party Signifier or RDP Signifier |

| Subject Public Key Info | Public Key Algorithm | RSA |
| --- | --- | --- |
|  | Subject Public Key (2048 bits) | Public Key Value |

| Key Usage | Criticality | True |
| --- | --- | --- |
|  | Key Usage | digitalSignature keyEncipherment |

| Signature Algorithm |  | rsa-with-SHA256 |
| --- | --- | --- |

  - A DCCKI Certificate Signing Request in respect of a DCCKI Infrastructure Certificate for the purpose of establishing FTPS communications to the DCC shall contain the information set out in the table immediately below:

| Subject | Attributes | Values |
| --- | --- | --- |

| Version |  | V3 (as per RFC 2986) |
| --- | --- | --- |

| Subject | Common Name | Party Signifier or RDP Signifier |
| --- | --- | --- |

| Subject Alt Name |  | <Fully Qualified Domain Name> |
| --- | --- | --- |

| Subject Public Key Info | Public Key Algorithm | RSA |
| --- | --- | --- |
|  | Subject Public Key (2048 bits) | Public Key value |

| Key Usage | Criticality | True |
| --- | --- | --- |
|  | Key Usage | digitalSignature keyEncipherment |

| Signature Algorithm |  | rsa-with-SHA256 |
| --- | --- | --- |

### DCCKI Certificate Signing Request format

  - DCCKI Certificate Signing Request requests shall be formatted according to PKCS#10, Base64 encoded.

  - The standard format shall be ASN.1 DER, including one of the immediately following two styles of PEM header:

    - -----BEGIN CERTIFICATE REQUEST----- and -----END CERTIFICATE REQUEST-----; or

    - -----BEGIN NEW CERTIFICATE REQUEST----- and -----END NEW CERTIFICATE REQUEST-----

### Acceptable DCCKI Certificate Signing Request variants

  - The DCC shall accept the following PKCS#10 variants:

    - Base64 all in one line;

    - Base64 with line breaks at 64 or 76 characters; and

    - if line breaks are used the \n and \r\n are both acceptable.

### Signing the DCCKI Certificate Signing Request using a Private Key associated with an Organisation Certificate

  - Following the creation of the DCCKI Certificate Signing Request in accordance with section 3.4 of this DCCKI IDS, the DCCKI Eligible Subscriber shall Digitally Sign the DCCKI Certificate Signing Request with a Private Key associated with an Organisation Certificate for which it is a Subscriber.

  - The DCCKI Eligible Subscriber shall ensure that the Digital Signature shall:

    - use, as the digital signature technique, Elliptic Curve Digital Signature Algorithm (ECDSA) (as specified in Federal Information Processing Standards Publications (FIPS PUB) 186-4) in combination with the curve P- 256 (as specified in FIPS PUB 186-4 at section D.1.2.3) and SHA-256 as the Hash function;

    - be applied to the entirety of the PKCS#10 file, including header and footer; and

    - be converted to Base64 and appended to the footer within the PKCS#10 file itself with a preceding “,” separator.

  - Prior to Digitally Signing the DCCKI Certificate Signing Request, the DCCKI Eligible Subscriber shall append to the footer of the PKCS#10 file, the Issuer which shall be URL encoded (as specified in the RFC 1738 and RFC 2253) and serial number of the Organisation Certificate with preceding “,” separators.

### Availability and Service Continuity

  - The DCC shall ensure that the DCCKI Service Interface is available, in accordance with Section L13.12 (the DCCKI Service Interface) of the Code.

  - The DCC shall notify Parties and RDPs in advance of any planned outages of the DCCKI Service Interface.

## ANNEX A

## DEFINITIONS

In this document, except where the context otherwise requires:

- expressions defined in Section A of the Code (Definitions and Interpretation) have the same meaning as is set out in that section; and

- any expressions not defined here or in Section A of the Code have the meaning given to them in the DCCKI Certificate Policy, the DCCKI Registration Authority Policies and Procedures or the Self Service Interface Access Control Specification.

| Issuer | The name of the signer of the DCCKI Infrastructure Certificate as described in the DCCKI Certificate Policy. |
| --- | --- |