Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

`

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 15 November 2019 |
| --- | --- |
| Issue Resolution Proposal | IRP610 |
| Issues Log ID | TS1186 |
| Issue Title | Inconsistency in Mapping Table refs for Twin Element ESME |
| Source | L+G |
| Date Raised | 30 August 2019 |
| Status | Draft 0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

Here is an inconsistency in the specs to draw to your attention.

Looking at GBCS v3.2 Sec 20 - Mapping Table - Rows 1800 and 1801 reference both Sec 5.13.2.3 (twin element meter section) and Sec 5.7.5.10. (single element meter section).

Row 1800 refers to Secondary Active Import Register

Row 1801 refers to Secondary Tariff ToU Register Matrix

Reviewing Sec 5.7.5.10 neither Secondary active Import Register & Secondary Tariff To Register Matrix are not mentioned. Both are included in Sec 5.13.2.3

We therefore believe that the references in rows 1800 & 1801 should be for Sec 5.13.2.3 only

Proposed Position:

In relation to second element SMETS data items, there are spurious references to single element ESME requirements in Table 20, and potentially ambiguous SMETS references in Table 7.4. The proposed position is to remove / correct these cross references.

Table 20

As per the extract overleaf, there are nine rows in GBCS Table 20 (SMETS required objects tab) referring only to values measured on the secondary element of a Twin Element ESME.

Since the secondary element’s associated data items are defined for a Twin Element ESME only (so in section 5.13 of SMETS only), the SMETS references in these rows should only be to sub-sections of SMETS 5.13.

Two rows (37 and 38) have such correct references. However, the other seven rows each include an incorrect reference to sub-sections of SMETS section 5.7 (so rows 538, 1800, 1801, 1802, 1803, 1804 and 1805).

The proposed resolution is to remove the references to sub-sections of SMETS 5.7 from column A of GBCS Table 20 (SMETS required objects tab) ‘SMETS ESME Ref’ in each of rows 538, 1800, 1801, 1802, 1803, 1804 and 1805.

Related, there are two additional rows where the SMETS/CHTS Object is secondary element only, but there is a reference to a single element section, as shown below:

The two references in cells A859 and A1225 to ‘5.6.3.35’ should be replaced with ‘5.12.2.2’ (which is the equivalent twin element section).

Table 7.4

There are six rows in Table 7.4 which:

- Relate to either primary or secondary measuring element SMETS information only; and

- Are in rows tagged as either ‘aggregate’ or applicable to both primary and secondary element information.

The proposed position is to clarify the notes in these 6 rows to make clearer that they apply to either primary or secondary only, but not both:

Interoperability and / or Compatibility

This IRP relates to internal Devices processing only, and so does not affect the content or structure of messages passing between Devices or between systems and Devices.

Required Changes to Documentation:

- In GBCS, Table 7.4 revise the cells shown as green highlighted in the ‘Table 7.4 notes changes.xlsx’ document embedded below.

- In GBCS, Table 20, SMETS required objects tab:

- remove the references to the ‘5.7..’ sub-sections of SMETS from column A ‘SMETS ESME Ref’ in each of rows 538, 1800, 1801, 1802, 1803, 1804 and 1805; and

- set the two references in cells A859 and A1225 to be ‘5.12.2.2 / 5.13.1.2’.

The resultant modified Table 20 and Table 7.4 and the associated effected Use Cases are embedded below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |