Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 15 November 2019 |
| --- | --- |
| Issue Resolution Proposal | IRP615 |
| Issues Log ID | TS1205 |
| Issue Title | GCS05 - discrepancy in specifications |
| Source | GWi |
| Date Raised | 2 October 2019 |
| Status | Draft 0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

Our technical team seems to have found a discrepancy between the Table 19.3 Use Cases in GBCS v3.2 and GBZ Message Template for the same command.

In the part Use Case Details for the use case GCS05 it is specified for the Week Profile of the Non-Disablement Calendar the count of weeks 1..2 ( Non-DisablementCalendar: nextWeeks[1..2].weekIdentifier), which is expected and it is consistent with daily and monthly profile specifications.

However, the GBZ Message Template says PublishWeekProfile (0-2 instances), which is not consistent with the above specifications. Please see first row in the table below.

Proposed Position:

As stated in ZSE, Publish Calendar, allows for zero Week entries:

Number of Week Profiles (mandatory): Number of week profiles in the Week Profile Table. A value of 0x00 means no Week Profile defined.

Table D-155 further states that:

If no Week Profile is defined, it is expected that the calendar will have one repeating Day Profile.

However, although such a construction is therefore valid in ZSE, it does not align to the requirements in DUIS or requirements elsewhere in GBCS.  The proposed change is to update the message template to make clear to Device manufacturers that they may always expect at least one Week Profile in the Command.

Interoperability and / or Compatibility

This change is a clarification only, and so has no impact on Devices or Systems.

Required Changes to Documentation:

In GBCS, Table 20, ‘ZigBee Commands' tab: set cells Q1378 to Q1391 to be ‘1’.

The resulting updated Mapping Table 20 and updated Use Cases are embedded below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |