Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 25 March 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP618 |
| Issues Log ID | TS1230 |
| Issue Title | Twin Element Calendar Server support |
| Source | EDMI |
| Date Raised | 11 December 2019 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v4.0 Draft 2 |

Version 0_2: additional text, shown as mark-up below was requested to be added by a supplier party following initial review of v0_1 at the TSIRS meeting in February 2020 in order to aid clarity of the proposal approach.

Description:

Publish Calendar is on aggregate ESI, and is stated as 5.7.4.30 (non-disablement Type=3)  & 5.7.4.48 (Tariff Switching Table)  & 5.13.1.2 (Tariff Switching Table).

There seems to be nothing within the Publish Calendar message itself to differentiate between elements. So how is the meter supposed to deliver the calendar information for the TOU to HAN devices.

Whereas PublishTariffInformation uses the primary and secondary ESI. So for this we have a deterministic way of providing the information for each element. Shouldn’t the same be done for all the calendar messages?

Further information and support are provided by Chameleon in the following document on Huddle: Tech Spec Issue Resolution/Supporting Documents/Stakeholder Documents/TS1230 Twin element Calendar Server support annex.docx

https://my.huddle.net/workspace/34269213/files/#/75369288

[also embedded at the table at the end of this IRP]

Proposed Position:

In line with the GBCS Use Cases, a Twin element ESME must support four ‘calendars’:

- Tariff Switching Table (primary measuring element);

- Tariff Switching Table (secondary measuring element);

- Non-Disablement Calendar (note the DLMS object is strictly not a calendar, but is shared over the HAN by ZigBee using the calendar cluster); and

- Auxiliary Controller Calendar.

GBCS Table 7.4 uses the calendar cluster on the aggregate endpoint to support three calendars:

- Tariff Switching Table;

- Non-Disablement Calendar; and

- Auxiliary Controller Calendar.

As per ZigBee, distinguishing between those three is via Calendar Type.  However, there is only one ZigBee Calendar Type for Tariff Switching Table (0x00 named Delivered Calendar), so there is no way to distinguish between that which relates to the primary measuring element and that for the secondary measuring element.

Under this proposal there are two ESIs on Twin Element ESME that would support calendars:

1.	Twin ESME aggregate ESI Endpoint – primary element calendar (Tariff Switching Table) and calendars common to both measuring elements (Non-Disablement Calendar & Auxiliary Controller Calendar)

2.	Twin ESME secondary ESI Endpoint – secondary element calendar (Tariff Switching Table)

The Twin ESME aggregate ESI Endpoint can support three of the calendars and the Twin ESME secondary ESI Endpoint supports the fourth.

The changes proposed utiliseaddress this by requiring the secondary measuring element ESI to provide access to the secondary measuring element Tariff Switching Table.  The primary measuring element Tariff Switching Table remains on the aggregate ESI, and the documentation in Table 7.4 is updated to reflect this.

There is no way to set a different Non-Disablement Calendar for the primary and secondary measuring elements, and as such there is no need to support two independent calendars for this functionality.  Similarly, there is only a single Auxiliary Controller Calendar on ESME, regardless of variant.

Interoperability and / or Compatibility

This change affects interactions between Twin Element ESME and PPMID / Type 2 Devices on the same HAN as such ESME only.  It, therefore, has no impact on DCC (Systems or Communications Hubs) and no impact on HAN Devices where there is no Twin Element ESME.

At the time of writing, there are understood to be no Twin Element ESME in operational use and so no operational ESME affected by this change.  Correspondingly, there are no live PPMID or Type 2 Devices reading such information.  The change only affects such Twin Element ESME, and PPMIDs / Type 2 Devices, attempting to read information from such ESME.

Required Changes to Documentation:

- In GBCS, Table 7.4, revise the text in cells ‘H146’ to ‘H155’ to include the underlined text as per the mark-up below:

‘Non Disablement Calendar / Tariff Switching Table / Auxiliary Controller Calendar. Tariff Switching Table is for the primary measuring element’

- In GBCS, Table 7.4, replace the text ‘5.7.4.30 & 5.7.4.48 & 5.13.1.2’ in cells ‘I146’ to ‘I155’ with:

‘5.7.4.2 & 5.7.4.30 & 5.13.1.2’

Note, that references to SMETS2 Section 5.7.4.48 have been removed as part of this proposed resolution.  This is because Section 5.7.4.48, as per SMETS 5.13.1.2, is not applicable to Twin Element ESME.

- At the bottom of GBCS, Table 7.4, add the rows shown as green highlighted in the ‘Table 7.4 TS1230 changes’ document embedded below.

The revised GBCS Table 7.4 with these changes made, is embedded in the document ‘IRP618 Table 7.4’ below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |