Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 14 February 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP619 |
| Issues Log ID | TS1242 |
| Issue Title | GBT blocks from old Messages |
| Source | BEIS |
| Date Raised | 17 January 2020 |
| Status | Draft 0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

Due to WAN and / or HAN latency, it is possible that a Device receives a GBT Message (‘GBT block’) containing part of a Command, where the Device has already processed that Command.  For example, where:

- The DSP sends both blocks in a two-block command to an ESME;

- The ESME receives block 2 first, and, given it is the last block, asks the DSP to re-send block 1;

- The ESME receives the ‘originally sent block 1’, so now has both blocks and successfully processes the Command, sending the resulting Response;

- The DSP gets the request to re-send block 1; and

- The ESME receives the ‘re-sent block 1’.

Can the ESME immediately discard block 1, on the basis that the Message Routing Header information (which is not part of the Remote Party Command carried in the payload’s of such messages) means the ESME knows it has already processed this Command and so will not do so again?

Alternatively, does the ESME have to request that block 2 is resent (knowing that the DSP will not be able to send it, since the Command has finished processing) then wait 30 minutes and discard block 1 (so blocking the ESME’s in bound GBT buffer for that period)?

Proposed Position:

In summary:

The Device can discard block 1 immediately, based on processing the Message Routing Header information.  The proposed position is to add some clarifying text to GBCS, which makes this explicit.

To explain:

By way of context, all Remote Party Commands, that can be large enough to require delivery using GBT, are required by GBCS to have Protection Against Replay.  This means that, as per GBCS 6.2.4.2, the corresponding Execution Counter is updated by the Device, to the Originator Counter in the Command, at the point the Device has authenticated the Command (so before its sends any Response, and before it receives the re-sent block 1 as in the example used above).

In the example, this means that the re-sent block 1 will have an Originator Counter, which is part of the Message Routing Header information, equal to the Device’s corresponding Execution Counter.  Therefore, the Device knows, when it gets the re-sent block 1, it has already received (and successfully authenticated) all blocks needed for this Command, so all GBT Messages in this GBT Message Series.

As per GBCS 7.2.11.3, a GBT Message carrying part of a Remote Party Command to a Device is the concatenation:

Message Routing Header || GBT Header || GBT Block Data

As per GBCS 7.2.11.4, GBT Block Data is the part containing ‘octets of the Remote Party Message’.

GBCS 7.2.11.4 states that ‘The GBT Recipient shall not undertake any processing, in the sense of Section 6, of the Remote Party Message carried in a GBT Message Series until it has received… all GBT Messages in this GBT Message Series’.

So there is an explicit prohibition on processing GBT Block Data until all blocks have been received.

However, there is no GBCS restriction on processing the Message Routing Header or GBT Header.  Indeed, such Device processing is required on receipt of each GBT Message, so that the requirements of GBCS 7.2.11.4 can be met.

There is a requirement in GBCS 7.2.11.4 for the Device to request blocks are resent where it has not received all blocks in that GBCS Message Series,  as follows:

‘Where the GBT Recipient has not received all such GBT Messages, it shall send a GBT Message (Request Block Resend) for each missing block-number.  Where the GBT Recipient is a Device, it may discard all blocks in a GBT Message Series if it has received no response to a GBT Message (Request Block Resend) after 60 minutes’

However, in the example used above, the Message Routing Header information means the Device knows, when it gets the resent block 1, that it has already received both blocks in this GBT Message Series.  Therefore, there is no requirements for the Device to request that block 2 is resent and so the 60-minute timeout does not apply.

So, the Device can process the Message Routing Header to establish that a newly arrived block is part of a GBT Message Series it has already processed.  The Device knows that it does not need to request other (already received) blocks from that Series.

GBCS is silent on when such blocks can be discarded and so, discarding immediately would be permissible.

To make this clearer, the proposed position is to add text to the effect that Devices may discard blocks immediately in this circumstance.

Interoperability and / or Compatibility

This resolution is permissive, in terms of allowing Devices to change their internal processing.  It does not require Devices to change processing.

The resolution does not change the format or content of any messages sent between Devices and Systems.

Therefore, this change has no impact on interoperability, and does not require changes to Device functionality.

Required Changes to Documentation:

At GBCS 7.2.11.4, Insert the paragraph as shown in the following mark-up:

<…run on from existing text>

<…run on from existing text>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |

Similarly the ESME timeout of 30 minutes does not apply in this scenario: ‘Where the GBT Recipient is an ESME, the ESME shall not send a GBT Message (Request Block Resend), in relation to a GBT Message Series, until and unless 30 seconds have elapsed since receipt of the most recent GBT Message in that GBT Message Series. If 30 minutes elapse without the ESME receiving corresponding GBT Message(s) in response, the ESME may discard the GBT Messages it has received in this GBT Message Series.