Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 5 May 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP621 |
| Issues Log ID | TS1253 |
| Issue Title | HAN Only Command Validation by the ESME / GSME - GBCS error |
| Source | Honeywell |
| Date Raised | 14 February 2020 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

In Section 14.7.4.1.3 HAN Only Command Validation by the ESME / GSME

Should it read:

'If the ESME / GSME has no PPMID in its Device Log, the ESME / GSME shall apply the requirements of Section 6.2.4.2 and undertake no additional processing.

If the ESME / GSME has a PPMID in its Device Log:

•  if the receiving Device is an ESME, the ESME shall use ZSE cryptographic processes to establish whether the Command was authentically issued by the PPMID that is in its Device Log; or

•  if the receiving Device is a GSME, the GSME shall undertake Command Authenticity and Integrity Verification, as required for a Command of Message Category SME.C.PPMID-GSME to establish whether the Command was authentically issued by the PPMID that is in its Device Log.

If the Command was NOT authentically issued by the PPMID within the Device Log, the ESME / GSME shall apply the requirements of Section 6.2.4.2.

If the Command was authentically issued by the PPMID within the Device Log, the ESME / GSME shall comply with the requirements of Section 14.6.4 (but excluding requirements in Sections 14.6.4.1.1, save that the ESME / GSME shall disregard the 20th digit before undertaking any further steps), and so process the contents of the Command accordingly.'

Proposed Position:

The word ‘NOT’ is missing from the sentence highlighted in yellow. This is because:

- the only processing in section 6.2.4.2 that is relevant to Prepayment Top up processing is that which relates to a failed authentication check, so leading to an Alert, a Security Log entry and the cessation of Command processing (see Annex A);

- without the word ‘NOT’, the sentence in yellow would have no effect (since it describes a case with no failed checks), and GBCS would not require any flagging to the supplier of the failure of the authenticity checks in the two bullets above the highlighted sentence;

- with the word ‘NOT’ in the yellow highlighted sentence, all of the authenticity checks in Section 14.7.4.1.3 result in the failed check actions at 6.2.4.2 as intended, so the supplier being notified (all other authenticity checks already explicitly require this).

Further:

- unlike a GSME, an ESME can support more than one PPMID in its Device Log, and so the ESME authentication bullet should refer to ‘a PPMID’ not ‘the PPMID’;

- the checks referring to SME.C.PPMID-GSME apply to the PCS01 Command sent by the PPMID and are detailed in two sections (6.2.4.1.1 Message Identifier Validation & 6.2.4.1.3 PPMID-GSME MAC Verification).  In Section 6.2.4.1.1, one check relates to the Business Originator ID.  GBCS defines Business Originator as the sender of the Command.  For PCS01, the sender is the PPMID and so the Business Originator ID is the ‘PPMID entity ID’ from the PCS01 Command.  Similarly, the Business Target ID in another check is the ‘Meter entity ID’ from the PCS01 Command.

The proposed position is therefore to revise the drafting in Section 14.7.4.1.3 to:

- add the word ‘NOT’ to ensure all failed authentication checks result in the same actions (rather than all bar the Smart Meter, Device-to-Device cryptographic check when there is a PPMID in its Device Log);

- to add clarification that the 6.2.4.2 processing relates to failed checks;

- to change ‘the PPMID’ to ‘a PPMID’ to reflect the fact that an ESME Device Log can contain more than one PPMID; and

- to add clarification as to the values that are the Business Originator ID and Business Target ID in PCS01 Commands to be used in GBCS 6.2.4.1.1 checks by the GSME.

Note that any failed authentication in relation to UTRN entry is subject to the SMETS Smart Meter requirement that the Smart Meter controls the rate at which further attempts are made, and so controls the rate at which such Alerts could be generated, across all failed authentication reasons.  During any period of non-acceptance following a failed authentication, any Prepayment Top Up Command from a PPMID must be discarded by the Smart Meter without processing the UTRN in the Command.

Interoperability and /or Compatibility

The changes do not have any impact on message format or content, and so do not impact on interactions between DCC Systems and Devices.

Where a Smart Meter has implemented without the word NOT in the yellow text, it will alert and log failed UTRN authentication in all cases except the case where:

- there is a PPMID in the Smart Meters Device Log; and

- a UTRN top up is received which was not authentically created by that PPMID (e.g. it was created by another HAN Device which is not authorised by the Supplier).

Required Changes to Documentation:

Amend the text in GBCS Section 14.7.4.1.3 as shown:

<run on as is…>

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | None but see Interoperability and /or Compatibility above |

Annex A – applicability of GBCS 6.2.4.2 to Prepayment Top Up Command processing

Prepayment Top Up Command processing is detailed in the 4 Use Cases in GBCS Section 14. For ‘Protection Against Replay Required?, the last three refer to the first Use Case, stating ‘See Table 14.4.2’. That Table states:

The Protection Against Replay mechanisms for Prepayment Top Ups are specified in Section 14.3.6. The Protection Against Replay mechanisms specified elsewhere in the GBCS do not apply

Therefore ‘Protection Against Replay’ requirements do not apply to Prepayment Top Ups.

Further, none of the Prepayment Top Ups Command types can be future-dated and so none contain activation date-times.

Therefore, and as per the mark up below, the only requirements that apply to Prepayment Top Up Command processing by a Smart Meter are those actions required after one or more failed authentication checks. Specifically the following requirement is the only one which can apply:

Where … any of the other prior required checks for this type of Command have failed, the Device shall:

•	generate an entry in the Security Log recording failed Authentication;

•	discard the Command without execution and without sending a Response; and

•	send an Alert notifying the failed Authentication, constructed as specified in Section 6.7, populated with the relevant Alert Code from Section 16 (the one of 0x8F1E, 0x8F30 or 0x8F3D that is required), to the Known Remote Party specified in Section 16.  If the Device is an ESME or a CHF, the Alert Payload shall be a DLMS COSEM Alert Payload.  Otherwise, the Alert Payload shall be a GBZ Alert Payload.

Section 14.6.4.1 states ‘Should any of the checks fail (save for the optional UTRN Check Digit verification), the requirements of Section 6.2.4.2 shall apply.’. Thus, failure of any of the checks in the last paragraph of Section 14.7.4.1.3 will also result in the failed check actions at 6.2.4.2, as would failure of any of the preceding checks.

As per GBCS 4.3.2.5, ‘a GSME shall have a Trust Anchor Cell capable of storing Key Agreement Security Credentials for a PPMID’ and therefore can only be joined (in a GBCS sense) to the PPMID with details in that Trust Anchor Cell. Hence only one PPMID.

As per SMETS 5.5.2.2 ‘ESME shall be capable of establishing and maintaining Communications Links via its HAN Interface with a minimum of six Type 1 Devices (including a minimum of one PPMID).’

For example, SMETS 5.6.3.3 requires that ‘ESME shall be capable of …controlling the number of invalid UTRN entries entered and processed’. CPA expands on this stating ‘UTRNs input by manual entry, through the user interface or via a PPMID, during any period of non-acceptance shall be discarded without attempting to validate the UTRN. Note: Rate-limiting via the PPMID should be independent to that via the user interface. For instance, a temporary block on UTRN entry via PPMID does not in itself cause UTRN entry via the user interface to also be blocked.

And, from GBCS 4.0, an SAPC that supports Prepayment Top Up Command processing