Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP625 |
| Issues Log ID | TS1140 |
| Issue Title | GBCS Publish Top Up Log Command |
| Source | Secure Meters |
| Date Raised | 27/05/2019 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

We have come across an issue in GBCS which requires the GSME to push the latest five prepayment top-ups which does not makes sense.  On completion of processing a UTRN, the GSME will push the details.  The GPF can make use of the Get Top Up Log if any are missed.   Pushing 5 each time appears to be a duplication and as such a waste of bandwidth and device processing time (both on GSME and GPF). Additionally, ZSE1.4 only allows the last to be sent.

Please see below extracts from GBCS v3.1 and ZSE1.4

Proposed Position:

As stated in the issue, there would be duplication in a GSME pushing the details of the previous five Top Ups since, as required by ZSE D.7.2.4.5, that would require the sending of 5 Publish Top Up Log commands, 4 of which would have been sent previously, and so would already have been stored by the GPF (as per the second requirement in 10.4.2.5).

Therefore, the proposal is to change the GBCS drafting so that only details of the most recent Top Up have to be pushed by the GSME, whenever it has successfully processed a Top Up (although there would be no bar on the GSME additionally pushing the other 4 most recent ones).

This would not affect GPF requirements either to store the Top Up details from Publish Top Up Log commands, or to recover from outages using the Get Top Up Log command (which would result in a Publish Top Up Log command containing 0, 1 or several sets of Top Up details).

The resolution also aligns to the GBCS Glossary term of ‘Prepayment Top Up’ rather than ‘prepayment top-up’. Appendix A includes the corrections to other sections to align to use of this Glossary term.

Interoperability and / or Compatibility

This change does not prohibit GSME from pushing all 5 Top Ups, and so does not require changes to GSME.

The change does not affect GPF requirements to store the details from Publish Top Up Log commands, regardless of whether those commands contain one or several Top Ups.

Therefore, this change has no impact on interoperability, and does not require changes to Device functionality.

Required Changes to Documentation:

At GBCS 10.4.2.5 and 10.4.2.6, modify the text as shown in the mark-up below:

<run on as is…>

Make the changes detailed at Annex A to align to the Glossary term ‘‘Prepayment Top Up’.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | None this is a document consistency change |

Annex A – alignments to glossary term

Table 4.3.2.6

Table 13.3.4.1:

Table 13.3.4.6:

In Table 13.3.5.12:

In section 14.3.8:

In the Glossary (replace dash with a space)

In section 27: