Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP626 |
| Issues Log ID | TS1247 |
| Issue Title | Query references to Trust Anchor Cell in Sec 13.3.5.8.1 |
| Source | L&G |
| Date Raised | 29th January 2020 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

GBCS v3.2, Sec 13.3.5.8.1 as well as Table 13.3.4.1 –

We noticed references to a Trust Anchor Cell (root, digitalsignature, management). We think this is an invalid reference considering Tables 4.3.2.5 and 4.3.2.6 of GBCS v3.2 does not recognise this identifier.  Therefore, we believe the reference of digitalsignature for root is incorrect.  Perhaps this should be changed to {root, keyCertSign, management}?  Please confirm our understanding and the correct identifier applicable.

Proposed Position:

As per the SMKI Organisation Certificate Policy (SEC Appendix B) and GBCS 12.3, the only allowed keyUsages for SMKI root Certificates (so those complying with the ‘Root OCA Certificate Profile’) are ‘keyCertSign; and cRLSign’. Therefore, a keyUsage of digitalsignature is never allowed in relation to root Certificates, and so such references in GBCS are incorrect and should be corrected.

Further, the cRLSign keyUsage is not relevant to Devices, since they have no access to CRLs (Certificate Revocation Lists), and so such keyUsage is to be ignored by Devices, as stated in GBCS 12.3:

‘For clarity, Devices are not required to use the associated Public Keys in relation to the cRLSign keyUsage and shall disregard a keyUsage of cRLSign in any processing’

Therefore, the only valid Device processing is in relation to root Certificates’ keyUsage of keyCertSign. This is reflected in the GBCS 4.3.2.5 requirement for Devices to have a single root Trust Anchor Cell for root named:

{root, keyCertSign, management}.

Additionally, the DSP have confirmed that all Commands created that refer to this root Trust Anchor Cell use the value of 5 for keyUsage which, as per GBCS section 13, means keyCertSign.

Therefore all GBCS references to the root Trust Anchor Cell should be to {root, keyCertSign, management}.

Interoperability and / or Compatibility

The changes are documentation only, in that they align to the only types of root Certificates that are allowed under the SMKI Organisation Certificate Policy, and the behaviour of DCC systems since go-live (both in terms of Commands and Responses).

Therefore, the changes have no impact on systems or Devices.

Required Changes to Documentation:

Make the following mark-up changes to GBCS as shown below:

- At GBCS, Table 13.3.4.1:

- At GBCS Section, 13.3.5.8.1:

- At GBCS Section, 13.3.5.9:

- At GBCS Section, 13.3.5.11:

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | No Parties believed to be affected as this document correction aligns to the only types of root Certificates that are allowed under the SMKI Organisation Certificate Policy and in place since DCC Live |