Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP627 |
| Issues Log ID | TS1279 |
| Issue Title | Alert Code 0x8F33 reference in SMETS |
| Source | BEIS |
| Date Raised | 2 April 2020 |
| Status | Draft v0_1 |
| Documentation Reference | SMETS v4.2 & SMETS v5.0 Draft 3 |

Description:

The Alert 0x8F33 'Supply Disabled then Armed - Load Limit triggered' is specified as a mandated Alert in GBCS Table 16.2, however, it is not mentioned in SMETS.

The original intent of the Alert was to warn suppliers that supply had been disabled due to load limiting.  This is something that the supplier would already be able to conclude as they would have configured ESME to do the load limiting in the first place (through SMETS 5.7.4.22).  However, having identified the omission it seems appropriate to add words to SMETS at Section 5.5.6 to mention this Alert and doing so in a way that minimises the need for change to DCC systems or meters.

Proposed Position:

As per the issue description, the 0x8F33 'Supply Disabled then Armed - Load Limit triggered' Alert should be referenced to in SMETS as it has been mandated in all versions of GBCS from v1.0.  Therefore, the proposal is to add text to state that the Alert should be sent following the commencement of the Load Limiting period when Supply is Disabled and then Armed such that it can be later Enabled.

Interoperability and / or Compatibility

The changes are documentation only, relating to an Alert that has been mandated in all versions of GBCS from v1.0 and devices will have been built to include it.

Therefore, the changes have no impact on DCC Users, consumers, systems or Devices.

Required Changes to Documentation:

At SMETS Section 5.5.6 (iv) (a), make the changes as shown by the mark-up below:

(Noting also, that a minor typographical error is also corrected at the first sub-bullet at point ‘b’ where a space has been added to ‘theMeter Balance’)

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | No Parties believed to be affected as this is a document correction |

This correction had already been made to the baselined SMETS v5.0 Draft 3