Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP628 |
| Issues Log ID | TS1194 |
| Issue Title | Inconsistency in IRP461 - GBCS v3.2 sec 14.3.7 |
| Source | L&G |
| Date Raised | 19/08/2019 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

A perceived inconsistency in the documentation:

IRP461, identified in point 4, that Sec 14.3.7 of GBCS talks about the updating the flag for UTRN Counter Cache.  The 'proposed position' section of this document, clarifies in Point 4 and 5 that this flag is no longer required.  Although Table 14.1 has been updated to reflect that, Sec 14.3.7 of GBCS v3.2 still continues to mention the use of this flag.  Perhaps Sec 14.3.7 has to be changed to reflect the proposed position as well and to maintain consistency in the specification?

Proposed Position:

As explained in IRP461:

‘The requirement for a flag against each UTRN Counter Cache entry comes from a time when consideration was being given to a range of mechanisms to protect against potential attacks from other HAN Devices, in relation to UTRN entry. The conclusion reached on that work was the SMETS requirement which states that Smart Meters need to be capable of ‘controlling the number of invalid UTRN entries entered and processed’.

The SMETS requirement doesn’t need the flags in the UTRN Counter Cache (since the SMETS requirement relates to invalid UTRNs and the Counter Cache only has details for valid UTRNs). Further, there is no way to read those flags (they can’t be read in the Billing Data Log Use Cases and there is no requirement to share with HAN Devices).  So, the requirement for these internal flags doesn’t lead to any external, mandated behaviour.  To that end, manufacturers could choose any values for the flags. However, since the flags do not support any mandated external behaviour, the proposal is to remove the Device requirement for the flags.’

By the same reasoning, the reference to this flag should also be removed from 14.3.7.

Interoperability and / or Compatibility

This text refers to internal Smart Meter functions that are not externally visible.  Thus, the change has no impact on interoperability or compatibility between Devices and Systems.

Required Changes to Documentation:

In GBCS Section 14.3.7, make the changes as shown in the mark-up below:

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | No Parties believed to be affected as this is a document correction |

SMETS 4.5.2.2, 4.5.3.3, 5.6.2.2 and 5.6.3.3 all require that the Smart Meter must be capable of ‘controlling the number of invalid UTRN entries entered and processed'.