Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 17 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP629 |
| Issues Log ID | TS1306 |
| Issue Title | Addressing various points raised on GBCS v4.0 D2 & SMETS2 v5.0 D3 |
| Source | BEIS |
| Date Raised | 2 April 2020 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v4.0 Draft 2 & SMETS2 v5.0 Draft 3 |

Description:

Following the consultation and response related to CRP612, various points requiring corrections and clarifications have been identified since baselining GBCS v4.0 Draft 2 / SMETS2 v5.0 Draft 3 on 3 February 2020.  These corrections and clarifications are in relation to changes made in GBCS v4.0 / SMETS2 v5.0 Draft 3, so are not relevant for any earlier version of GBCS / SMETS and do not affect any operating Devices.

The purpose of this IRP629 is to detail resolutions to the points identified, with the intention that these are included in a further draft of GBCS V4.0  / SMETS2 v5.0 Draft 3, to be baselined prior to designation in SEC, to support the November 2020 DCC release.

Changes from v0_1 are shown with yellow highlight, relating to Point 5 / Correction 5 and new Point 8e.

Proposed Position:

The following corrections are proposed:

- Entries in the Auxiliary Controller Event Log related to HCALCS messages:

There is a contradiction regarding logging of Commands from HCALCS to ESME, specifically:

- Section 18.1.1.1 states the ‘Action’ field in switchNumberAndAction can have a value of 0x00 or 0x64 (so open or closed);

- Section 7.3.6.1 states it can only have a value 0x00 (so open only).

Since an HCALCS can be open or closed, the incorrect statement in 7.3.6.1 (the last sentence) is to be removed, and the ECS35g Use Case is to be amended to refer to both sections, specifically the switchNumberAndAction row in ECS35g is to say: ‘See GBCS Section 7.3.6.1 and Section 18.1.1.1’.

- Add the now published Service Reference Variant (SRV) details to new Use Cases:

The now published updates to the SEC ‘Section E - DCC User Interface Service Schedule’ (UISS) and ‘DUIS 4.0 Nov 2020’ for the November 2020 release include these SRV details. They should be reflected in the corresponding GBCS Use Cases and at GBCS Sections 13.9 and 13.10 (these details were blank in the draft GBCS v4.0, as they were not known at that time).

- ECS100 – name contradiction

In GBCS Table 7.2.9.1a, Table 16.2 and section 16.1.1, the name for Alert ECS100 is:

‘Command not supported by Device’.

In GBCS Table 20 and so the Use Case HTML, the name is:

‘Command not supported by SAPC’.

The names are to be aligned as per GBCS Table 7.2.9.1a, Table 16.2 and section 16.1.1.

- ECS100 – ESME support is not mandated.

There is no requirement for ESME to issue an ECS100 ‘Command not supported by Device’ Alert.  An ESME must support all ESME SMETS requirements, and so there is no requirement for an ESME to provide this Alert.  GBCS, Table 16.2 should be corrected to remove ESME mandated support for ECS100.

- ECS100 - Supplementary Originator Counter is missing from the Use Case specific payload

Where an SAPC sends an ECS100 Alert in Response to a Command from an unknown remote party, it is required by GBCS 7.2.9.1a to include that party’s User ID in the Alert (in the Supplementary Remote Party ID field).

However, that is not sufficient for that party to identify the specific Command.  The correction is to require SAPC to add that extra required information, specifically the Command’s Supplementary Originator Counter, to the Alert’s Use Case.  This change aligns GBCS to the draft MMC V4.0.

- Incorrect notes for PPMID HAN only commands

GBCS, Table 13.7.3a makes explicit that three SMETS HAN only commands can be sent from PPMID to both ESME and SAPC, namely ‘Request Emergency Credit Activation’, ‘Request to Add Credit’ and ‘Request to Enable ESME Supply’.  This aligns to SMETS, which permits SAPC to support the full range of ESME functionality.

However, the notes in the Use Case reference rows for these three rows state that ‘ESME is the only valid target Device’ and SAPC support is ‘n/a’.  Table 20 notes are to be corrected to align to SMETS and GBCS, Section 13.7.3.

- Firmware distribution alerts for SAPC

As stated in GBCS, Section 11:

‘Requirements and statements in this Section 11, and in Use Cases ‘CS05b Distribute Firmware to ESME / GSME’ and ‘CS06 Activate Firmware’,  shall apply to an SAPC as if it were an ESME.’

And in GBCS, Section 11.4.2:

‘The ESME / GSME shall verify the Upgrade Image contained within the OTA Upgrade Image according to Section 11.2.5, and … If the verification is successful, the ESME / GSME shall construct and send a Firmware Distribution Receipt Alert, according to Section 11.2.6.’

Therefore, SAPC must support the ESME version of Firmware Distribution Receipt Alert, and so row 144 in the GBCS Table 20, Use Case reference tab should be marked as ‘mandatory’ for SAPC support.

- Correction of typographical errors

These corrections are detailed in the ‘Required Changes to Documentation’ section of this IRP below.

Interoperability and / or Compatibility

The changes are to a GBCS version that, whilst baselined, is not yet designated in SEC Schedule 8. As such, there are no installed Devices or operational Systems that are affected.

These changes have no impact on current DCC Users, consumers, systems or Devices.

Required Changes to Documentation:

Correction 1:

Correct text at GBCS 7.6.3.1 as shown by the mark-up below:

In GBCS Table 20, SMETS required objects tab, add the underlined text to cell K565 (this results in the revised ECS35g Use Case within the HTML file embedded in the table at the bottom of this IRP):

See GBCS Section 7.3.6.1 and Section 18.1.1.1

Correction 2:

At GBCS, 13.9.2, Table 13.9.2 add the row as shown in the mark-up below:

<run on as is…>

At GBCS, 13.10.2, Table 13.10.2 add the row as shown in the mark-up below:

<run on as is…>

In GBCS, Table 20, Use Case reference tab, populate the green highlighted cells as shown. This produces updated Use Cases, where appropriate, within the HTML file embedded in the table at the bottom of this IRP.

Correction 3:

In GBCS Table 20, Use Case reference tab, populate the green highlighted cell as shown. This produces the updated ECS100 Use Case description within the ‘Table of Contents’ in HTML file embedded in the table at the bottom of this IRP.

Correction 4:

Delete the ‘x’ in the ESME column of the ECS100 row in GBCS Table 16.2, so delete ‘x’ from cell 'T337'.  This results in the revised Table 16.2, embedded in the table at the bottom of this IRP.

Correction 5:

At GBCS, Section 7.2.9.1 in the Table 7.2.9.1a add the text as shown in the mark-up below:

Correction 6:

Amend the green highlighted cells in GBCS Table 20, Use Case reference tab as shown:

Correction 7:

Amend the green highlighted cells in GBCS Table 20, Use Case reference tab as shown:

The changes made to GBCS, Table 20 for Corrections 1, 2, 3, 6 and 7 are all shown in the revised Table 20 workbook file embedded in the table at the bottom of this IRP.

Correction of typographical errors 8:

- In the fourth paragraph of GBCS, Section 13.7.4.3.2, correct the misspelling of ‘Auxiliary Controler’ to ‘Auxiliary Controller’.

- Correct the 3 occurrences in GBCS, Section 13.10.5.10 and the 3 occurrences in GBCS, Section 13.3.5.10 of the misspelt ‘targetTrustAncorCell’ to ‘targetTrustAnchorCell’

- Reformat the description of the term ‘Load Controller’ in the GBCS, Section 21, Glossary to be as shown:

- Add the missing ‘2.2’ reference to GBCS Table 13.3.5.1 as shown in the mark-up below:

- Correct GBCS, Table 20, Use Case reference tab, cell D185 (the message code for CS02g) to “See GBCS Section 13.10.4.1”.  The correction is reflected in the Table 20 workbook file embedded in the table at the bottom of this IRP.

- Correct the grammar at SMETS2 v5.0 Draft 3, Section 5.25.1 as shown in the mark-up below:

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | Organisations developing Devices aligning to GBCS v4.0 may wish to note the corrections. Noting further that changes are not relevant for any earlier version of GBCS and therefore, do not affect any currently operating Devices. |