Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 5 October 2018 |
| --- | --- |
| Issue Resolution Proposal | IRP589 |
| Issues Log ID | TS0990 |
| Issue Title | CS02b authentication sequence |
| Source | BEIS |
| Date Raised | 24 September 2018 |
| Status | Draft 0_1 |
| Documentation Reference | GBCS v3.1 |

Description:

GBCS 13.3.5.1 states that the ‘checks should be carried out in the order specified’.

GBCS 13.3.4.2 then states that ‘Should any of the checks detailed in Section 13.3.5.1 fail then the Device shall … discard the Command without execution’.

Therefore, all checks need to be successful if the Command is to be executed. Thus, the requirement that the checks should be carried out in the order specified seems unnecessarily constraining, and other sequences may be more efficient from a Device perspective (e.g. undertaking cryptographic checks first).

Proposed Position:

As stated in the issue, the constraint on sequence is unnecessary, and so the proposed position is to remove it and make clear that there is no constraint on sequence. Indeed, it may be more effective, from a Device perspective, to undertake cryptographic checks before checks that require more extensive parsing of more complex payloads (e.g. those containing Certificates).

Note that the check is a ‘should’ not a ‘shall’ requirement, and so existing Devices do not have to follow the sequence, to comply with current GBCS Versions however the indicated sequence may not be optimal for Devices, and so should be removed.

Related, to support Devices’ flexibility on the sequence of processing, the revised 13.3.5.8 text notes that the cryptographic protections that need checking, can either be identified by the Message Code in the Grouping Header or the credentialsReplacementMode in the Command Payload (in line with check 1.2 of Table 13.3.5.1).

The drafting also includes correction of a cross reference in section 13.3.5.8 (to align the first paragraph in that section to the second and third paragraphs).

A similar point about sequence of checks applies in relation to Use Cases CS02d and CS03, so corresponding changes are proposed for Sections 13.5.4 and 13.7.4.2.2 (and references to them in Section 6.2.4.1).

Interoperability and / or Compatibility

This is a documentation only change that does not require any change to existing Devices but provides clarity that such flexibility is allowed. Therefore, it has no impact on any Devices or systems.

Required Changes to Documentation:

In GBCS, make the following changes to the Sections as shown in the mark-up below:

<run on as is…>

<run on as is…>

<run on as is…>

<run on as is…>

5.

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |