Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP623 |
| Issues Log ID | TS1262 |
| Issue Title | Read ALCS Event Log values v2 |
| Source | BEIS / Critical |
| Date Raised | 27 February 2020 |
| Status | Draft v0_1 |
| Documentation References | GBCS v3.2 HOWEVER, DOES NOT APPLY TO (AND THE RESOLUTION MUST NOT BE APPLIED) GBCS v4.0 |

Description:

The following was originally raised as TS0869.  The previously associated IRP582 is now withdrawn and replaced by this IRP623.

GBCS refers (Table 20), for ECS35f:

AuxiliaryLoadControlSwitchEventLog: logEntries[1..100].logEntry.switchNumberAndAction

The first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms). The second integer shall be (0x11) On, (0x12) Off, (0x20) Revert to calendar control or (0x00) Entry relates to a Message from an HC ALCS ...

see section 18.2.1 for definition of the named structure in each entry of buffer for this OBIS code. Where this Log Entry refers to a Response received from an ALCS, the left octet shall contain the number of the HC ALCS ([n] in SMETS terms) and the right octet shall be 0x00. Otherwise, this field shall be populated with the script identifier from the Remote Party Command.

Only the script identifier values referring the trigger using remote party command (ECS47) are mentioned.

SMETS2 refers:

Section 5.6.4.1 Control HAN Connected Auxiliary Load Control Switch

A Command requesting that a HAN Connected Auxiliary Load Control Switch either closes or opens its switch for a time period specified within the Command.  The ESME shall be capable of issuing a Control HAN Connected Auxiliary Load Control Switch(5.6.4.1) Command according to the rules set out in Section 5.5.11.1.

In executing the Command, ESME shall be capable of recording the Command and Outcome to the Auxiliary Load Control Switch Event Log(5.7.5.6).

Section 5.5.11.1 Calendar-based switching of HAN Connected Auxiliary Loads

...

issuing a Control HAN Connected Auxiliary Load Control Switch(5.6.4.1) Command containing the time period remaining until the next switching event defined in the calendar for HCALCS [n]

...

SMETS2 seems to refer that all "Control HAN Connected Auxiliary Load Control Switch" commands and outcome (triggered via calendar switch and via remote party command) should be logged,

while GBCS seems to refer that only the "Control HAN Connected Auxiliary Load Control Switch" commands and outcome triggered by remote party command (use case ECS47 - Set or Reset HC ALCS or ALCS State) should be logged in ALCS Event Log.

What is the expected behaviour?

Proposed Position:

As explained at Annex A (‘detailed explanation’) of this IRP, there are, for ESME complying with GBCS versions earlier than GBCS v4.0, 30 possible values of switchNumberAndAction in the ALCS Event Log required to meet the SMETS requirements.  However, ECS35g at GBCS v3.2 suggests there are only 20 (the ten related to ALCS Calendar are not listed in the description of the field).  To allow these values, align to other parts of GBCS and so align to values used by ESME, the proposed resolution is to revise the ECS35g text (as shown in the mark-up below) to read:

The first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms). The second integer shall be (0x11) On, (0x12) Off (0x01) On from the ALCS Calendar, (0x02) Off from the ALCS Calendar, (0x11) On in a Command to the ESME, (0x12) Off in a Command to the ESME, (0x20) Revert to calendar control or (0x00) Entry relates to a Message from an HC ALCS.

Additionally, and as also explained at Annex A of this IRP, three related corrections are also proposed, to align to DCC systems and ESME in operation today.

Interoperability and / or Compatibility:

This clarification is to align to existing ESME ALCS Event logging behaviour for deployed Devices (so those complying with GBCS versions prior to GBCS v4.0), and as such, the values logged are already present in Response to ECS35g Commands.  Therefore, the IRP does not affect the structure or content of any Messages.

Required Changes to Documentation:

In a version of GBCS which has a version number less than (but not including) GBCS v4.0:

- To correct the allowable ALCS Event Log entries and align to ESME in live, revise cell ‘K565’ in the ‘SMETS Required objects’ tab of GBCS Table 20 as shown in the mark-up (This produces the corrected ECS35g Use Case embedded below):

The first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms).  The second integer shall be (0x11) On, (0x12) Off (0x01) On from the ALCS Calendar, (0x02) Off from the ALCS Calendar, (0x11) On in a Command to the ESME, (0x12) Off in a Command to the ESME, (0x20) Revert to calendar control or (0x00) Entry relates to a Message from an HC ALCS.

- To correct the ECS46c and ECS61a GBCS Section 7 cross references, replace ‘7.3.7’ with ‘7.3.8’ in each of the cells ‘K747’, ‘K756’, ‘K779’ and ‘K783’ in the ‘SMETS Required objects’ tab of GBCS Table 20.

(This produces the amended Table 20 and ECS46c and ECS61a Use Cases embedded below)

- To remove the spurious references to switch numbers ‘17..21’ in GBCS Table 7.3.8a and so align to the DCC solution built, remove the text marked as strikethrough from each of the cells D203 and D205 in GBCS Table 7.3.8a, as shown in the mark-up below:

shall be limited to only accept script entries that reference script table 0-0:10.0.103.255 with values aa01 and aa02, where aa is in the range 01..05 and 17..21, as hex expressions.

(This results in the amended GBCS Table 7.3.8 as embedded below)

- To remove the duplicated text, make change to GBCS Section 18.1.1.3 as shown in the following mark-up:

<run on as is…>

| Proposed specification version for incorporation | TBC but must not be included in any GBCS v4.0 or later version (so may only be included in GBCS v3.x versions) |
| --- | --- |
| Impact on previous RPs | Note: this IRP623 supersedes the withdrawn IRP582 |
| Attachment(s) |  |
| Affected Parties | None, this is a document consistency change |

Annex A – detailed explanation

SMETS 5.5.11.1 requires that an ESME sends a ‘Control HAN Connected Auxiliary Load Control Switch(5.6.4.1) Command’ when either:

- The Auxiliary Load Control Switch Calendar requires a changes of switch state; or

- It receives a Request Control of HAN Connected Auxiliary Load Control Switch(5.6.3.21) Command from an HCALCS; or

- It receives a Reset HAN Connected Auxiliary Load Control Switch [n] State(5.6.3.24) Command from the Supplier.

Specifically 5.5.11.1 states:

ESME shall be capable of monitoring the Auxiliary Load Control Switch Calendar(5.7.4.2) and at times defined in the calendar ... issuing a Control HAN Connected Auxiliary Load Control Switch(5.6.4.1) Command ….

On receipt of a Request Control of HAN Connected Auxiliary Load Control Switch(5.6.3.21) Command and on receipt of a Reset HAN Connected Auxiliary Load Control Switch [n] State(5.6.3.24) Command ESME shall be capable of … issuing a Control HAN Connected Auxiliary Load Control Switch(5.6.4.1) Command

SMETS 5.6.4.2 (Control HAN Connected Auxiliary Load Control Switch) requires that the ESME ‘shall be capable of recording the Command and Outcome to the Auxiliary Load Control Switch Event Log(5.7.5.6).’

GBCS Table 13.7.3b and GBCS 18.1.1.1 require that ESME implement the Control HAN Connected Auxiliary Load Control Switch Command using the ZSE Load Control Event command.

GBCS 18.1.1.1 identifies three circumstances where the ESME must send such a command aligning to the three cases in SMETS:

| SMETS requirement | GBCS 18.1.1.1 requirement |
| --- | --- |
| The Auxiliary Load Control Switch Calendar requires a changes of switch state | to control a HCALCS according to the Auxiliary Load Control Switch Calendar [which is set using ECS46c Set HCALCS and ALCS configuration in ESME (excluding labels)] |
| It receives a Request Control of HAN Connected Auxiliary Load Control Switch(5.6.3.21) Command from an HCALCS | as required by Section 18.1.1.3 |
| It receives a Reset HAN Connected Auxiliary Load Control Switch [n] State(5.6.3.24) Command from the Supplier. | successful authentication of a Command with Message Code 0x0055 [so ECS47 Set or Reset HCALCS or ALCS State] |

Therefore, the ESME must create an Auxiliary Load Control Switch Event Log entry each time it successfully sends a ZSE Load Control Event.  The intention is that each log entry should identify:

- the UTC date / time the command was sent;

- the switch number to which it was sent; and

- the requested action and what caused the ESME to request it.

The intention is that (2) and (3) should be recorded in the same way as required in the Remote Party Commands used to instruct the ESME.

Switch Number and Action

In ECS46c, the switch and action are specified by the COSEM ‘script_logical_name’ (which is fixed) and ‘script_selector’.  GBCS Table 20 states ‘See the Blue Book and section 7.3.7 of the GBCS’ for these attributes.  The GBCS reference is incorrect and should be 7.3.8 (which does contain these values) and not 7.3.7 (which doesn’t contain these values, and so can’t have been used in the design of any ESME).  The resolution includes correction of this cross reference.

GBCS Section 7.3.8a states that, in relation to the calendar used in ECS46c:

‘shall be limited to only accept script entries that reference script table 0-0:10.0.103.255 with values aa01 and aa02, where aa is in the range 01..05 and 17..21, as hex expressions.’

For this script table, 7.3.8b then states:

‘Script identifiers for HCALCS and ALCS. The left octet shall contain the number of the HCALCS / ALCS ([n] in SMETS terms). So valid values are 0x01 to 0x05. The right octet shall contain be one of the following 5 values:

0x01 = on (switch close) from the ALCS Calendar

0x02 = off (switch open) from the ALCS Calendar

0x11 = on (switch close) in a Command to the ESME

0x12 = off (switch open) in a Command to the ESME

0x20 = revert to ALCS Calendar control in a Command to the ESME’

And that:

‘For example, invoking the script with script identifier 0x0111 is an instruction to turn on the HAN ALCS / ALCS referred to by n = 1, when in an authenticated Command to the ESME.

There are therefore 25 scripts required, the identifiers for which are:

- 0x0101,0x0102,0x0111,0x0112 and 0x0120;

- 0x0201,0x0202,0x0211,0x0212 and 0x0220;

- 0x0301,0x0302,0x0311,0x0312 and 0x0320;

- 0x0401,0x0402,0x0411,0x0412 and 0x0420;

- 0x0501,0x0502,0x0511,0x0512 and 0x0520;’

In ECS47 ‘Set or Reset HC ALCS or ALCS State’, the switch and action is specified as:

‘For an ALCS or HC ALCS, a method to Open, Close or Reset to Calendar control. In terms of the input to the method, the first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms). The second integer shall be (0x11) Close, (0x12) Open, (0x20) Revert to calendar control.’

It can be seen that the reference to ‘17..21’ in Table 7.3.8a as highlighted above, is incorrect and is removed as part of the proposed changes.  This aligns to the DCC solution built.

Aside from that point, ECS47, ECS46c and Table 7.3.8 align in identifying switch and action so the intention is therefore that any of the 25 values used above can be used in the ‘Auxiliary Load Control Switch Event Log’.

However, the switchNumberAndAction field in ECS35g ‘Read ALCS Event Log’ states:

The first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms). The second integer shall be (0x11) On, (0x12) Off, (0x20) Revert to calendar control or (0x00) Entry relates to a Message from an HC ALCS.

This adds a further five values to recognise that there is also a trigger which is an HCALCS request.  So, the further 5 values are 0x0100, 0x0200, 0x0300, 0x0400 and 0x0500.

However, ECS35g does not recognise the ten ALCS calendar driven events, which SMETS requires that an ESME logs.  To allow these values, align to other parts of GBCS and so align to values used by ESME, the proposed resolution is to revise the ECS35g text, as shown below, to read:

‘The first integer shall be the number of the ALCS / HC ALCS (so '[n]' in SMETS terms). The second integer shall be (0x11) On, (0x12) Off (0x01) On from the ALCS Calendar, (0x02) Off from the ALCS Calendar, (0x11) On in a Command to the ESME, (0x12) Off in a Command to the ESME, (0x20) Revert to calendar control or (0x00) Entry relates to a Message from an HC ALCS.’

Critical Software originally raised Issue TS0869 on 19/3/2018.  IRP582 covering that Issue was published on 6/8/2018