Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 15 July 2020 |
| --- | --- |
| Issue Resolution Proposal | IRP631 |
| Issues Log ID | TS1310 |
| Issue Title | Table 7 3 8 - Correction of Inconsistency |
| Source | BEIS |
| Date Raised | 22 May 2020 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v3.2 |

Description:

The contents of cell D140 in the Objects Tab of GBCS Table 7.3.8 (so Table 7.3.8a) were changed by CRP612, and this is currently reflected in the baselined draft GBCS v4.0 but is not yet applied to GBCS v3.x.

This IRP is to incorporate the same changes in GBCS v3.x  (consistent with GBCS v4.0) to align Table 7.3.8a requirements to the overall policy intent, as documented within the explanations to TS1233, TS1255 and most specifically TS1257 (copies of these explanations are embedded in the table below).

This IRP does not trigger any requirements to end date previous versions of GBCS through the TSAT.  Any changes to TSAT will be separately consulted on through the SEC Mod process.

Proposed Position:

The proposed position is to align the text in GBCS, Table 7.3.8a, cell ‘D140’ in GBCS v3.x versions with that already included in GBCS v4.0 Draft 2.

By way of context and as referred to in the explanation to TS1257, GBCS 7.3.2 requires that:

‘All Devices which are ESME (so where deviceType = 1) shall have the constant values set for the DLMS COSEM attributes specified as requiring constant values in Table 7.3.8a, and shall ensure that such values cannot be amended, save via activation of new firmware.’

Therefore, Table 7.3.8a mandates constant ESME values that must be set before installation and must not be changed except by firmware upgrade.  Table 7.3.8a also contains some explanatory text in relation to some of the constant values.  It is that explanatory text which was changed to align to wider SMETS / GBCS requirements, specifically in relation to a DLMS class 111 (Account) object.

Interoperability and / or Compatibility

This correction affects internal ESME behaviour only, and so has no impact on interoperability.

Required Changes to Documentation:

At GBCS v3.x, replace the contents of cell D140 in the Objects Tab of Table 7.3.8 (so Table 7.3.8a) with:

‘When this passive object is activated:

1) then the BillingDataLog(SetPaymentModeOrTariffTriggeredFinancialExcExport) and the BillingDataLog(SetPaymentModeOrTariffTriggeredFinancialExportOnly) shall be updated.

2)If prepayment is being invoked then it shall activate any pending time or payment debt objects and ensure that topups can be accepted.

3)If credit mode is being invoked then this object shall disable any time and payment debt objects and shall take steps to ensure that the disconnector does not open as a result of this invocation and shall ensure that topups are not accepted’

| TS version for incorporation | GBCS v3.x & any subsequent GBCS |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | ESME Manufacturers Suppliers – meters may operate differently depending on the version of GBCS they comply with |

The ’Description’ section of this IRP631 has been rewritten following TSIRS review of the previous published version

This IRP631 is intended to be applied to v3.x meaning the 3.x version following the current v3.2 and all subsequent GBCS versions, although this will be subject to the timing inherent in the IRP Modification processes administered by SECAS. Note, there is no intention to retrospectively apply changes to GBCS v3.2 or earlier versions

The full explanation as published is embedded in the table at the bottom of this IRP