Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 16 April 2021 |
| --- | --- |
| Issue Resolution Proposal | IRP642 |
| Issues Log ID | TS1431 |
| Issue Title | Reclarifying trigger for Alert Code 8F69 |
| Source | BEIS |
| Date Raised | 15 February 2021 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v3.2 & GBCS v4.0 |

V0_2 of this IPR642 removed the word ‘key’ immediately prior to ‘establishment’ at the yellow highlight below.

Description:

Changes proposed by ‘TS0860 - Clarify trigger for the Event / Alert Code 8F69’ added additional drafting at GBCS Section, 10.2.2.2 which was implemented in GBCS 3.2, as follows:

‘Immediately following the successful establishment of the tunnel between the CHF and ESME / GSME, the ESME / GSME shall send an Alert with Alert Code 0x8F69’.

The original proposal was to clarify exactly what constituted ‘joining a ZigBee SEP Smart Metering Home Area Network’.  The proposal intended to do this by making clear that the joining required both the completion of CBKE with the CHF; and the establishment of a tunnel with the CHF.

The additional text added was also intended to clarify timing as to when the Alert was sent.  To that end, the name of the Alert was also changed from “Device Commissioned” to ‘Tunnel Established’.

However, as identified in the Issue and explanation at ‘TS1424 - Alert 0x8F89 implementation’, further clarification at GBCS 10.2.2.2 would seem to be required.

Proposed Position:

As explained in the summary of TS1424:

‘The Alert with Alert Code 0x8F69 need only be sent when a GSME first establishes full communications with a CHF, so has first completed CBKE and tunnel establishment.’

The text at GBCS, 10.2.2.2 should be modified to make it clearer that the Alert is sent immediately following the first successful establishment of a tunnel and not, as could be currently interpreted, on any subsequent tunnel establishment.

Interoperability and / or Compatibility

There is no interoperability or compatibility issues resulting from this change, other than some Devices, not working as per this IRP, may send multiple alerts where only one is expected / required.

Required Changes to Documentation:

At GBCS, Section 10.2.2.2 make the change as shown by the mark-up below

<…run on from existing text>

<run on as is…t>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | Whilst this is a documentation only change, energy suppliers may find Devices, not working as per this IRP, may send multiple alerts where only one is expected / required. |