Smart Metering Implementation Programme

Technical Specification Change Resolution Proposal (CRP)

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 10 February 2021 |
| --- | --- |
| Issue Resolution Proposal | CRP630 |
| Issues Log ID | TS1328 |
| Issue Title | Provision for additional HAN data items for APC |
| Source | TSIRS |
| Date Raised | 17 July 2020 |
| Status | Draft v0_3 |
| Documentation Reference | SMETS2 29 November 2020 and GBCS v4.0 |

Changes made to this CRP630 at v0_2 are shown by the yellow highlighting and at  v0_3 (page 6) with cyan highlight.

Description:

This change is intended to address the problem statement discussed at TSIRS meetings in the spring of 2020 and in the BEAMA paper [embedded at the table at the end of this CRP].  Concerning the current lack of the following information available to devices on the HAN:

•  The Auxiliary Controller Calendar is marked [INFO], meaning it is to be shared over the HAN. However, the ZigBee mechanism available to share such information, whilst being able to fully detail the ALCS / HCALCS calendar entries, cannot fully detail the Auxiliary Proportional Controller calendar entries, rendering the information effectively useless for many use-cases. Compressing the percentage scale down to a binary on/off will hide the entire range from 0% to 99% behind a single value of 0.

•   Auxiliary Controller [n] State (which is mandated to include two integer values between 0 and 100 per APC[n], reflecting currently configured input and output levels) is not shared with HAN Devices currently.

•   Limit applied to the Auxiliary Controller

•   Override applied to the Auxiliary Controller

•   The use of the Boost button by the consumer

Proposed Implementation:

The changes are to SMETS2 29 November 2020 and GBCS v4.0 and cannot be applied to earlier versions of either GBCS or SMETS2 because these changes only apply to APC or SAPC that were introduced at GBCS v4.0 and ESMETS v5.0 / SAPCTS v5.0.  They are as detailed in the ‘Required changes to Documentation’ section.

Interoperability and / or Compatibility:

These changes would apply to newly installed Devices only (since there are no installed ESME with APC or SAPC), and so there would be no change to already installed Devices.  Depending on when this CRP is implemented there may already be ESME with APC or SAPC installed that do not support this changed functionality.

The changes do not affect DCC Systems or User systems, nor do they require any changed interactions with them.

However, Suppliers may wish to note:

- the changes in default HAN Alert settings.  These can, on ESME, be re-configured after installation (to turn off the additional HAN alerting) by the Supplier using existing Use Cases / Service Requests.  The additional SAPC HAN alerting may not be configurable by the Supplier (since SAPC are not required to support such HAN alerting configuration), and so it may not be possible to turn off such default SAPC HAN alerting.  However, the volume of such alerts is likely to be low, given that they relate to User Interface commands; and

- there may be somewhat greater Sub GHz communication volume where either the SAPC / ESME or the requesting Type 1 / 2 Device or both are operating on Sub GHz.  This could lead to greater ‘throttling’ of Sub GHz communication for Sub GHz Devices on the same HAN.

There are no changes required to CH.  The ZigBee commands concerned (Publish Event, Get Scheduled Events, Load Control Event) are messages which the CH already has to route, as part of its functioning as a ZigBee router.

Already installed Type 1 / 2 Devices may receive additional ZigBee messages which they do not have the functionality to process, and so will discard. Such Devices may also receive ‘Publish Event Commands’ for Alert Codes ‘0x8161’ and ‘0x8162’ where some will have payload, and some will not.  Such Devices will discard any which they cannot process.

Required Changes to Documentation:

There are two groups of changes:

- One to add the requirements for ESME with APC and SAPC to send HAN Alerts when User Interface Commands are used; and

- The second, to add the requirements for ESME with APC and SAPC to provide Auxiliary Controller related information.

To add the requirements for ESME with APC and SAPC to send HAN Alerts when User Interface Commands are used:

- In GBCS v4.n, Table 16.2, in the ‘Alert HAN’ column for each of the rows for Event / Alert Codes 0x8161 and 0x8162 (so in cells G82 and G83), change:

‘N’

to

‘See GBCS Section 16.4’

This creates the revised Table 16.2 as embedded in the table below.

- In GBCS v4.n, Section 16.4, insert the text as shown in the mark-up below:

<run on as is…>

- At GBCS v4.n, Section 21, Glossary, add the definition as follows:

- At ESMETS v5.n, add a new Section at 5.29.2 as shown in the mark-up below:

<run on as is…>

- At SAPCTS v5.n, add a new Section at 9.5.4 as shown in the mark-up below:

<run on as is…>

To add the requirements for ESME with APC and SAPC to provide Auxiliary Controller related information:

- At ESMETS v5.n, add the new Sections at 5.30 as shown in the mark-up below:

<run on as is…>

- Add the following new Section to GBCS v4.n:

<run on as is…>

- In GBCS v4.n, Table 7.4, append the words ‘ and GBCS 18.1.2’ to the existing text in each of the following cells:

H46 and I46;

H47 and I47;

H160 and I160;

H161 and I161;

H552 and I552; and

H553 and I553.

This will result in the modified GBCS, Table 7.4 embedded at the table below.

| TS version for incorporation | GBCS 4.n, ESMETS v5.n & SAPCTS v5.n - TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | Notes on this embedded pdf: - The document refers to a ‘ZSE Get Load Control’ command from the DRLC Client. There appears to be no such command. This CRP630 assumes the ‘Get Scheduled Events’ command is intended. - The document does not detail the value to be used where current state is caused by SMETS specified default behaviour. This has been added as value ‘0x0D’ in the CRP630 drafting. - The document suggests that bits ‘1-10’ are populated for APC[n] by setting bits ‘2n+1’ and ‘2n+2’. However, as per SMETS, ‘n’ is between ‘1’ and ‘5’ so ‘2n+1’ and ‘2n+2’ would use bits ‘3-12’. Therefore, this CRP630 uses ‘2n-1’ and ‘2n’, to align to bits ‘1-10’. |
| Affected Parties | Applies to newly installed Devices only. Suppliers may wish to note the points made at ‘Interoperability and / or Compatibility’ earlier. |