Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 26 March 2021 |
| --- | --- |
| Issue Resolution Proposal | IRP634 |
| Issues Log ID | TS1351 |
| Issue Title | Future start date-time on Limit APC [n] Level & ECS47e |
| Source | L&G |
| Date Raised | 18 September 2020 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.0 / SMETS2 29 November 2020 |

Description:

1.  Can start date-time in ECS47e and ECS47a be specified in the future as per SMETS (line 3630) , even though they are not future dated according to GBCS ?

2.  Can we send a cancellation, by sending another/second ECS47e command?

3.  Do we send an alert via ECS102 for ECS47e rejection, what is the difference between rejection and cancellation?

Proposed Position:

NOTE: this IRP applies only to ESMETS and SAPCTS v5.0 or later versions.

As explained at Annex A, SMETS does not currently detail the Device requirements when an APC [n] Limit Period / APC [n] Setting Period is in force, and then a Command to set a new such period is received by the Device.

The drafting changes proposed are to make clear that the Device should immediately end any current period, when a valid Command to set a new period is received.  Thus, a valid new Command would cancel any prior corresponding APC [n] period.  Further, the drafting makes clear that the actions required at the end of such a prior period are taken when the prior period is ended by the Device and before the new period is applied.  To illustrate using SMETS 5.29.1.1 as an example, ending of the prior period requires the Devices to take the following actions before processing the new period:

‘When the end date-time of the APC [n] Limit Period is reached, or immediately where that date-time is in the past, ESME shall:

ix.	if it has paused the timer for any active Boost Period as a result of processing the Command, resume the timer and set the output level for APC [n] to its maximum;

x.	if ESME’s current date-time is not within an active APC [n] Setting Period, be capable of:

a.	ensuring the input or output level of the APC [n] is the level defined in the Auxiliary Controller Calendar [INFO](5.7.4.2) for that date and time, or the maximum possible corresponding level, if no corresponding level is defined in the calendar; and

b.	sending an Alert to that effect via its HAN Interface containing the current UTC date and time and the resulting input or output level;

xi.	if ESME’s current date-time is within an active APC [n] Setting Period, take no further action.’

Additionally, the IRP corrects a typographical error, namely that SMETS, 5.29.1.2 (Set APC [n] Level requires:

‘ESME shall reject the Command where the specified APC [n] Limit Period has a duration of more than 24 hours.’

Such Commands specify an APC [n] Setting Period not an APC [n] Limit Period.

Interoperability and / or Compatibility

There is no interoperability impact between Systems and Devices arising from this change, since it relates to internal Device behaviour, and so does not affect the format or content of any Messages.

Required Changes to Documentation:

In SMETS, 5.29.1.1, make the changes shown:

<…run on from existing text>

<run on as is…>

In SMETS, 5.29.1.2, make the changes shown:

<…run on from existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | Suppliers and ESME Manufacturers |

Annex A – explanatory response to the questions posed in the issue:

On Q1, as per the GBCS Use Cases, ECS47a contains ‘settingPeriodStartDateTime’ and ECS47e contains ‘limitPeriodStartDateTime’ parameters.  These date-times may be in the future relative to the Device’s current clock time.

SMETS2, ESMETS, 5.29.1 makes explicit the Device processing requirements when such date-times are in the future e.g. ‘ESME shall … if the APC [n] Limit Period is in the future according to ESME’s current time ...’.

ECS47a is not ‘Capable of future dated invocation’ in the sense of GBCS 9.2.2 (which relates only to Use Cases marked ‘Capable of future dated invocation?’ in the Mapping Table). ECS47a is marked as not capable of future dated invocation in the Mapping Table, and so none of the GBCS 9.2.2 requirements apply to ECS47a processing.

On Q2, where the second ECS47e command relates to the same Auxiliary Controller, as per the GBCS Use Case, the ESME will overwrite whatever values were set in the corresponding DLMS COSEM attributes by the first ECS47e Command.

However, SMETS does not currently detail the Device processing in relation to the APC [n] Limit Period currently in force. With the drafting added to SMETS as per this IRP634, the ESME will end the corresponding APC [n] Limit Period, and then process the period specified in the newly arrived command.  Prior to this drafting being added to SMETS the Device behaviour in this case would be uncertain.

Therefore, the second such ECS47e Command would act so as to cancel the first.

On Q3, an ECS102 Alert is only required in the circumstances at SMETS2, ESMETS, 5.29.1.1 specifically:

‘When the end date-time of the APC [n] Limit Period is reached, or immediately where that date-time is in the past, ESME shall …if ESME’s current date-time is not within an active APC [n] Setting Period, be capable of … sending an Alert’.

The end date-time of the APC [n] Limit Period being in the past has the effect of cancelling any Limit Period.  Hence, the Alert’s name is ‘ECS102 Limit APC [n] Level ended or cancelled’.

If a Command is rejected because it is not valid then, as per GBCS, Section 6, it should not be processed passed the point of sending an Alert to the supplier to notify rejection of an inauthentic Command.  Thus, rejection of a Command is unrelated to any Command specific processing (such as cancellation).