This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

MP226 ‘Incorporation of Category 3 Issue Resolution Proposals into the SEC – Batch 10’

Annex A

Legal text – version 1.0

About this document

This document contains the redlined changes to the SEC that would be required to deliver this Modification Proposal.

Schedule 8 ‘Great Britain Companion Specification’

These changes have been redlined against Great Britain Companion Specification (GBCS) version 4.2.

These changes will be applied to the next version of the GBCS v4.x series at the time the modification is implemented. These will also be applied to the next version of any subsequent GBCS series introduced on or before the modification is implemented.

## Amend Section 3.3.3 as follows:

    - 3.3.3 Octets and bytes – informative

The term ‘octet’ is used to refer to units of 8 bits of digital information, to avoid potential ambiguity with the term ‘byte’, and to align with protocol terminology.

An octet string is a sequence of octets, with the first octet shown on the left and the last octet shown on the right, for example 0x12 0xAB 0x34 0xCD. If an octet string is shown formatted as a hexadecimal number, the first octet is shown on the left (most significant) and the last octet is shown on the right (least significant).  For example, 0x12AB34CD used in the context of an octet string represents 0x12 0xAB 0x34 0xCD.

## Amend Section 3.3.6 as follows:

    - 3.3.6 Encoding and length of variable length unsigned integers

Encoding(X) shall be the encoding of a variable size unsigned integer X as follows:

- if 0<X<128, then Encoding(X) is a single octet whose value is X; or

- if 128<= X <256, then Encoding(X) is  an octet string composed of the concatenation 0x81 || Y, where Y is one octet in length and has a value equal to X; or

- if 256<=X <65,536, then Encoding(X) is  an octet string composed of the concatenation 0x82 || Y, where Y is two octets in length and has a value equal to the big endian representation of X; or

- if 65,536<= X <16,777,216 then Encoding(X) is a an octet string composed of the concatenation 0x83 || Y, where Y is three octets in length and has a value equal to the big endian representation of X.

Len(Encoding(X)) shall be the length in octets of Encoding(X).

## Amend Section 3.3.8 as follows:

    - 3.3.8 Octets Endianness - Informative

The GBCS defaults to ‘big endian’ except for parts where a protocol specifies ‘little endian’ sequence.

The ZCL specification requires elements to be ‘little endian’ in ZCL commands. This applies to all ZCL and ZSE commands, including those required by the GBCS (see Section 7.4 and elsewhere in the GBCS). It also applies to the ZCL header and ZCL payload parts of GBZ Payloads, as required by Section 7.2. Note that all other elements of the GBZ Payloads, all DLMS COSEM Payloads, all ASN.1 Security Payloads and all other parts of Messages constructed according to Section 7.2 are ‘big endian’. This is because they are defined in the GBCS and not in the ZCL specification. Thus, the GBZ Payload elements labelled ‘Profile ID’, ‘Alert Code’, ‘Timestamp’, ‘Extended Header Cluster ID’, ‘Extended Header GBZ Command Length’, ‘From Date Time’ and ‘Length of Ciphered Information’ are big endian. Note that the GBZ Payload element ‘Ciphered Information’ is an octet string and so endianness is irrelevant (since it is a sequence of single octet elements).

Note that GBCS does not use the two octet ‘Manufacturer code’ field in the ZCL Header. The other three fields in the ZCL Header are single octet elements and so endianness has no effect in relation to ZCL Header.

## Amend Section 4.3.2.5 as follows:

    - 4.3.2.5 Required Trust Anchor Cells and related Device requirements

The Trust Anchor Cells specified in Table 4.3.2.5 by TrustAnchorCellIdentifier are those required on each deviceType. Additionally:

a GSME shall have a Trust Anchor Cell capable of storing Key Agreement Security Credentials for a PPMID; and

a PPMID shall have a Trust Anchor Cell capable of storing Key Agreement Security Credentials for a GSME.

The types of Device and the corresponding value of deviceType shall be defined in ASN.1 notation by:

DeviceType ::= INTEGER {

gSME	(0),

eSME 	(1),

communicationsHubCommunicationsHubFunction	(2),

CommunicationsHubGasProxyFunction 	(3),

type1HANConnectedAuxiliaryLoadControlSwitch 	(4),

type1PrepaymentInterfaceDevice	(5),

type2	(6)

}

An SAPC shall have a deviceType of eSME (1) throughout this GBCS.

Every Device shall:

have storage allocated capable of holding Security Credentials as required  by Table 4.3.2.5 for its Device type; and

have all the Trust Anchor Cells, specified in Table 4.3.2.5 as being required for its Device type, populated with Security Credentials that comply with the requirements of this GBCS.  Critically, root, recovery and accessControlBroker Trust Anchor Cells shall be populated with valid credentials for each of those three Remote Parties.

|  |  |  |  |  |  | Type of Device (= is required; empty = is not required) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ESME /SAPC | GSME | CH (CHF) | CH (GPF) | HCALCS | PPMID |
|  | deviceType value(s) |  |  |  |  | 1 | 0 | 2 | 3 | 4 | 5 |
|  | TrustAnchorCellIdentifier |  |  |  |  |  |  |  |  |  |  |
| No | remotePartyRole | keyUsage |  | cellUsage |  |  |  |  |  |  |  |
| 1 | root | keyCertSign |  | management |  |  |  |  |  |  |  |
| 2 | recovery | digitalSignature |  | management |  |  |  |  |  |  |  |
| 3 | supplier | digitalSignature |  | management |  |  |  |  |  |  |  |
| 4 | supplier | keyAgreement |  | management |  |  |  |  |  |  |  |
| 5 | supplier | keyAgreement |  | prePaymentTopUp |  |  |  |  |  |  |  |
| 6 | networkOperator | digitalSignature |  | management |  |  |  |  |  |  |  |
| 7 | networkOperator | keyAgreement |  | management |  |  |  |  |  |  |  |
| 8 | accessControlBroker | digitalSignature |  | management |  |  |  |  |  |  |  |
| 9 | accessControlBroker | keyAgreement |  | management |  |  |  |  |  |  |  |
| 10 | transitionalCoS | digitalSignature |  | management |  |  |  |  |  |  |  |
| 11 | wanProvider | digitalSignature |  | management |  |  |  |  |  |  |  |
| 12 | loadController | digitalSignature |  | management |  |  |  |  |  |  |  |
| 13 | loadController | keyAgreement |  | management |  |  |  |  |  |  |  |

Table 4.3.2.5:  Requirements for Trust Anchor Cells by Device Type

For clarity, the GPF and CHF shall each have their own set of Trust Anchor Cells.

A specific Trust Anchor Cell shall be identified in this GBCS using the notation {remotePartyRole, keyUsage, cellUsage}.  For example {supplier, digitalSignature, management} shall refer to the Trust Anchor Cell that holds the Device’s Supplier Digital Signing Security Credentials, so including the Supplier’s:

Entity Identifier;

Remote Party Role; and

Digital Signing Public Key.

Where a Device supports the processing of Remote Party Messages, that Device:

shall support the processing of the Update Security Credentials Commands required by Section 13; and

shall not allow execution of any Remote Party Command other than an Update Security Credentials Command, Update Load Controller Security Credentials Command or Provide Security Credentials Command, nor issue any Remote Party Alerts, in relation to a Remote Party Role where the Remote Party Role stored in a Trust Anchor Cell is different than that of the Trust Anchor Cell itself.

When verifying a Cryptographic Protection applied to a Command instance it receives, a Device shall use the Remote Party Security Credentials that it holds at the time of Command processing.

Devices shall only be capable of replacing Remote Party Security Credentials on receipt of an Update Security Credentials Command specified in this GBCS.

## Amend Section 9.1.3 as follows:

    - 9.1.3 Device Requirements relating to the ZCL Time Cluster and its usage

Requirements in this Section 9.1.3 shall apply to an SAPC as if it were an ESME.

All italicised terms in this Section 9.1.3 shall have the meanings defined in the Time Cluster specification within the ZigBee Cluster Library Specification.

In relation to the ZCL Time Cluster in the Remote Communications Endpoint, a Communications Hub shall:

set the Time attribute to the UTCTime provided to it via its WAN interface, whenever such time information is available to it via its WAN interface and maintain its internal clock to the tolerance required by the CHTS;

set the Time attribute to 0xFFFFFFFF whenever it cannot maintain its time via its WAN interface to the tolerance required by the CHTS due to resetting its internal clock;

whenever its Time attribute has the value 0xFFFFFFFF, store the value 0xFFFFFFFF in any unsigned 32 bit integer format time stamps it records and store the value 0xFFFFFFFFFFFFFFFFFF8000FF in any DLMS COSEM octet-string(12) format time stamps it records; and

always have TimeStatus attributes set as:

  - Attribute Bit Number 0 (Master) equal to 0b1 (master clock);

  - Attribute Bit Number 2 (MasterZoneDst) equal to 0b0 (not master for Time Zone and DST); and

  - Attribute Bit Number 3 (Superseding) equal to 0b1 (time synchronisation can supersede).

In relation to any ZigBee Time Cluster on the ESME, the ESME shall always have TimeStatus attributes set as:

Attribute Bit Number 0 (Master) equal to 0b0 (not master clock); and

Attribute Bit Number 3 (Superseding) equal to 0b0 (time synchronisation should not be superseded).

At power on of the clock, an ESME or GSME shall:

set its Time Status as ‘Invalid’;

attempt to synchronise time, using the Communications Hub’s Time Cluster; and

where a valid Time (so not 0xFFFFFFFF) is provided by the Communications Hub before any Set Clock Command is received,  set its time to the value of Time provided and set its Time Status to ‘Unreliable’.

ESME and GSME shall attempt to synchronise time, using the Communications Hub’s Remote Communications Endpoint Time Cluster, once every 24 hour period in line with the SMETS requirement.  ESME and GSME shall undertake the following processing dependent on the outcome of each attempted synchronisation:

if a time of 0xFFFFFFFF is provided or if no time is received the Device shall retry the synchronisation after an elapsed period of 30 minutes, for a minimum of the lesser of three retries, or a retry resulting in a valid Time (so not 0xFFFFFFFF) being provided. If no valid Time has been provided after three retries, the Device shall set its Time Status to ‘Unreliable’;

if the time provided by the Communications Hub differs from the Device’s time by more than 10 seconds, then the Device shall:

  - set its Time Status to ‘Unreliable’; and

  - if this results in a change to Time Status, the Device shall construct and issue an Alert with Alert Code 0x8F0C, meaning that its time would have been shifted by more than 10 seconds.  If the Device is a GSME, the Alert Payload shall be a GBZ Alert Payload.  If the Device is an ESME, the Alert Payload shall be a DLMS COSEM Alert Payload.

if the time provided by the Communications Hub differs from the Device’s time by 10 seconds or less, then the Device shall adjust its time to the Communications Hub’s time.

## Amend Section 11.5.2.3 as follows:

    - 11.5.2.3 Activate Firmware Command, Response and Alert Payloads – structure definition

Each instance of @ActivateFirmware.CommandPayload and of @ActivateFirmware.ResponsePayload and of @ActivateFirmware.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.5.2.3 which specifies the structure in ASN.1 notation.

ActivateFirmware DEFINITIONS ::= BEGIN

CommandPayload ::= 	SEQUENCE

{

-- specify the hash of the Manufacturer Image to be activated

manufacturerImageHash		OCTET STRING,

-- the Originator Counter as in the Grouping Header of the Command

originatorCounter	   INTEGER (0.. 18446744073709551615),

-- the date-time at which the Command is to execute, if future dated

executionDateTime	   GeneralizedTime OPTIONAL

}

ResponsePayload ::= 	CHOICE

{

-- if the Command is future dated, the Response will not have any details of

-- execution (those will be in the subsequent alert)

commandAccepted		NULL,

-- if the Command is for immediate execution, the Response will detail the

-- outcomes

executionOutcome		ExecutionOutcome

}

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode		INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- the Originator Counter as in the Grouping Header of the corresponding Command

originatorCounter		INTEGER (0.. 18446744073709551615),

-- detail what happened when the future dated command was executed

executionOutcome		ExecutionOutcome

}

ExecutionOutcome ::= 	SEQUENCE

{

-- Specify whether the activation was successful or not

activateImageResponseCode		ActivateImageResponseCode,

-- Specify the Device’s now current firmware version. The value shall be four

-- octets in length and shall correspond to the big-endian representation of the

-- unsigned 32-bit integer File Version field in the ZSE

-- OTA Header structure.

firmwareVersion		OCTET STRING

}

ActivateImageResponseCode::= INTEGER

{

success		(0),

noImageHeld		(1),

hashMismatch		(2),

activationFailure		(3)

}

END

## Amend Section 11.7.3 as follows:

    - 11.7.3 Construction of Alert

Grouping Header and SMD Signature shall be populated as required for an Alert of the SME.A.C Message Category. The Message Code shall be 0x0128.

The Business Originator ID field in the Grouping Header shall contain the CHF’s Entity Identifier.

The Business Target ID field in the Grouping Header shall contain the Entity Identifier in the CHF’s {accessControlBroker, digitalSignature, management} Trust Anchor Cell.

The Alert Payload shall be constructed according to the requirements of Section 11.7.4 and the Alerts shall be populated as specified in Table 11.7.3.

| Attribute name | Data Type | Value (blank cells mean the command specific value is derived by the encoding process) | Mandatory, OPTIONAL or DEFAULT value | Notes |
| --- | --- | --- | --- | --- |
| @FirmwareTransferAlert.AlertPayload | SEQUENCE |  |  |  |
| alertCode | INTEGER | 0x8F8A 0x8F89 | Mandatory | (1) 0x8F8A: OTA Firmware Image Delivery to the target Devices successful; (2) 0x8F89: OTA Firmware Image Delivery to the target Device failed; |
| executionDateTime | GeneralizedTime | The date-time at which this Alert was created | Mandatory |  |
| otherDeviceEntityIdentifier | OCTET STRING | Entity Identifier | Mandatory | The Entity Identifier of the ESME / GSME / HCALCS / PPMID the firmware upgrade is targeted at |
| firmwareVersion | OCTET STRING | Firmware version | Mandatory | The big-endian representation of the unsigned 32-bit integer firmware version of the transferred or discarded Upgrade |
| TransferResponseCode | INTEGER | fileTransferSuccess (0), imageDiscarded(1), hardwareVersionMismatch(2), fileTransferFailure (3) | Mandatory | (a) 0x8F8A: fileTransferSuccess (0) (b) 0x8F89: imageDiscarded (1); hardwareVersionMismatch (2); fileTransferFailure (3) |

Table 11.7.3: @FirmwareTransferAlert.AlertPayload population

## Amend Section 11.7.4 as follows:

    - 11.7.4 Firmware Transfer Alert – structure definition

Each instance of FirmwareTransferAlert.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.7.4 which specifies the structure in ASN.1 notation.

FirmwareTransferAlert  DEFINITIONS ::= BEGIN

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode	INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- specify the 64 bit Entity Identifier of the Device the upgrade has been

-- targeted at

otherDeviceEntityIdentifier 	OCTET STRING,

-- specify the upgrade image version if replaced or discarded by the

-- Communications Hub or if the transfer to the target device succeeded or

-- failed. The value shall be four octets in length and shall correspond to

-- the big endian representation of the unsigned 32-bit integer FileVersion

-- field in the ZSE OTA Header structure

firmwareVersion	OCTET STRING,

-- specify the outcome

transferResponseCode	TransferResponseCode

}

TransferResponseCode ::= 	INTEGER

{

fileTransferSuccess	(0),

imageDiscarded	(1),

hardwareVersionMismatch	(2),

fileTransferFailure	(3)

}

END

## Amend Section 11.8.4 as follows:

    - 11.8.4 Read PPMID/HCALCS Firmware Version Command and Response Payloads – structure definition

Each instance of @ReadPpmidHcalcsFirmwareVersion.CommandPayload and ReadPpmidHcalcsFirmwareVersion.ResponsePayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.8.4 which specifies the structure in ASN.1 notation.

ReadPpmidHcalcsFirmwareVersion  	DEFINITIONS ::= BEGIN

CommandPayload ::= 	NULL

ResponsePayload ::= 	SEQUENCE

{

-- Specify the Device’s now current firmware version. The value shall be four

-- octets in length and shall correspond to the big endian representation of the

-- unsigned 32-bit integer File Version field in the ZSE

OTA Header structure.

-- Set to 0x00000000 if reading failed.

firmwareVersion	OCTET STRING,

-- specify whether the reading of the firmware was successful.

firmwareReadCode	FirmwareReadCode

}

FirmwareReadCode ::= 	INTEGER

{

firmwareReadSuccess	(0),

firmwareReadFailure	(1)

}

END

## Amend Section 11.9.3 as follows:

    - 11.9.3 Construction of PPMID Firmware Activation Alert

Grouping Header and SMD Signature shall be populated as required for an Alert of the SME.A.C Message Category.

The Business Originator ID field in the Grouping Header shall contain the PPMID’s Entity Identifier.

The Business Target ID field in the Grouping Header shall contain the Entity Identifier in the PPMID’s {accessControlBroker, digitalSignature, management} Trust Anchor Cell.

The Alert Payload shall be constructed according to the requirements of Section 11.9.4 and the Alerts shall be populated as specified in Table 11.9.3.

| Attribute name | Data Type | Value (blank cells mean the command specific value is derived by the encoding process) | Mandatory, OPTIONAL or DEFAULT value | Notes |
| --- | --- | --- | --- | --- |
| @PpmidFirmwareActivation.AlertPayload | SEQUENCE |  |  |  |
| alertCode | INTEGER | 0x8F8B | Mandatory |  |
| executionDateTime | GeneralizedTime | The date-time at which this Alert was created | Mandatory |  |
| firmwareVersion | OCTET STRING | ZSE firmware version | Mandatory | The big endian representation of the unsigned 32-bit integer firmware version in operation or 0x00000000 if reading the firmware version failed |
| ActivateImageResultCode | INTEGER | activationSuccess(0), activationFailure(1) | Mandatory |  |

Table 11.9.3: @PpmidFirmwareActivation.AlertPayload population

## Amend Section 11.9.4 as follows:

    - 11.9.4 PPMID Firmware Activation Alert Payloads – structure definition

Each instance of @PpmidFirmwareActivation.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.9.4 which specifies the structure in ASN.1 notation.

PpmidFirmwareActivation  	DEFINITIONS ::= BEGIN

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode		INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- Specify the Device’s now current firmware version. The value shall be four

-- octets in length and shall correspond to the big-endian representation of the

-- unsigned 32-bit integer File Version field in the ZSE

-- OTA Header structure.

firmwareVersion	OCTET STRING,

-- Specify whether the activation was successful or not

activateImageResultCode		ActivateImageResultCode

}

ActivateImageResultCode::= INTEGER

{

activationSuccess		(0),

activationFailure		(1)

}

END

## Amend Section 14.1 as follows:

    - 14.1 Defined Terms

The following terms used in this Section 14 shall have the meanings defined in this Table 14.1.

| Defined Term | Meaning |
| --- | --- |
| Currency Unit | Shall be either GB Pound or European Central Bank Euro |
| Maximum Credit Threshold | Shall be the maximum value of any single Prepayment Top Up with a resolution of five decimal places. Its value shall be interpreted by the Device in whole Currency Units |
| Maximum Meter Balance Threshold | Shall be the maximum total credit value recorded on the ESME / GSME with a resolution of five decimal places. Its value shall be interpreted by the Device in whole Currency Units |
| Highest UTRN Counter | The highest numerical value of any UTRN Counter in the UTRN Counter Cache |
| Prepayment Token Decimal (PPTD) | Shall have the meaning specified in Section 14.3.1 |
| Prepayment Top Up Token (PTUT) | Shall have the meaning specified in Section 14.3.2 |
| Unique Transaction Reference Number (UTRN) | Shall have the meaning specified in Section 14.3.3 |
| UTRN Check Digit | Shall be the 20th digit of the UTRN |
| UTRN Counter Cache | Shall be an array capable of containing up to 100 entries, each entry containing an unsigned integer of 32 bits in length. At manufacture, the UTRN Counter Cache shall contain at least one zero value and shall contain no values which are non-zero. The array shall be arranged as a circular buffer such that, when full, further writes shall cause the lowest numerical value entry to be overwritten |
| UTRN Counter | The 32 most significant bits of the Originator Counter |

Table 14.1:  Meanings of Defined Terms

Maximum Credit Threshold and Maximum Meter Balance Threshold are represented using a value and associated scaler.  The scaler determines the fractional part of the value, for example, to represent £123.25478 requiring a resolution of 5 decimal places would use a value of 12325478 and an associated scaler of -5 (i.e. multiply by 10-5).  To interpret a value in whole Currency Units, the fractional part would be ignored, so for example, 12325478 would become £123.

## Amend Section 16.2 as follows:

  - 16.2	Event and Alert Codes

Table 16.2 lists the valid Event and Alert Codes, and sets out their requirements.

Table 16.2:  Event and Alert Codes

## Amend Section 18.2 as follows:

  - 18.2	DLMS COSEM Message Templates

Table 18.2 contains Message Templates for all Use Case with DLMS COSEM payloads. These Message Templates are derived from the Mapping Table, and shall be complied with in the construction and population of all such Messages.

Table 18.2:  DLMS COSEM Message Templates

## Amend Section 19.3 as follows:

  - 19.3	Embedded Use Cases

Table 19.3 contains the Use Cases that fulfil the interface requirements to cover Commands (and their Responses) and Alerts (where applicable).  In addition, it includes ZSE Message Templates.

Note:  DLMS COSEM methods that have values which have an impact on the execution of the method (that is, methods with input values that are not integer(0)), the DLMS part of the Mapping Table and the Use Case include two or more rows.  One row contains the method, and the subsequent row(s) contain the value(s) to be sent with the method.

A number of Use Cases are also covered in GBCS main body.  These are identifiable from the Table of Contents.

Table 19.3:  Use Cases

## Amend Section 20 follows:

20	Mapping Table

Table 20 contains the Mapping Table from which the Use Cases and Message Templates were generated.  These tables map between SMETS attributes and methods, SEC Service Requests, Use Cases, DLMS COSEM attributes and methods and ZSE clusters, attributes and commands.

In addition to the Use Cases, certain columns in the Mapping Table are directly referenced from this document.

Please note that in the SMETS required objects tab only rows marked ‘E’ (External to HAN) in column F are fully specified, since those rows relate to Remote Party Messages.  Other rows are only specified to the extent that these elements of Remote Party Messages rely on them.

Table 20:  Mapping Table

Schedule 9 ‘Smart Metering Equipment Technical Specifications’

These changes have been redlined against Gas Smart Metering Equipment Technical Specification (GSMETS) version 4.3.

These changes will be applied to the next version of the GSMETS v4.x series at the time the modification is implemented. These will also be applied to the next version of any subsequent GSMETS series introduced on or before the modification is implemented.

## Amend Section 4.1 as follows:

  - 4.1	Introduction

For GSME without a Valve the requirements and processing, including those for the GBCS, of the Sections:

- 4.4.7.2 Prepayment Mode (including all associated Interface and Data requirements Sections);

- 4.5.2.6 Enable Supply [PIN];

- 4.5.2.10 Test Valve;

- 4.5.3.7 Arm Supply;

- 4.5.3.10 Disable Supply;

- 4.6.4.25 Supply Depletion State; and

- 4.6.4.26 Supply Tamper State,

shall not apply.

To the extent that GSME without a Valve optionally has been designed to have the functionality set out in i to vii above the relevant requirements in this specification and the associated requirements in the GBCS shall be fully met.

Supplier and Network Operator credentials on the Communications Hub (Gas Proxy Function) relate to the supply of gas only. These Trust Anchor Cells on a Communications Hub are still required and valid where there is no GSME connected to the SMHAN, but the stores should be populated with Access Control Broker certificates (so ensuring the Gas Proxy Function functionality, apart from Update Security Credentials, is inoperable)

This Trust Anchor Cell is required on an SAPC, recognising that, in line with SMETS, the SAPC may or may not support prepayment related functionality.