Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 4 June 2019 |
| --- | --- |
| Issue Resolution Proposal | IRP604 |
| Issues Log ID | TS1105 |
| Issue Title | Query on IRP550 - Frame Control |
| Source | Flonidan |
| Date Raised | 13 March 2019 |
| Status | Draft 0_1 |
| Documentation Reference | GBCS v3.2 |

Description:

We have found a problem with IRP 550, I don’t think it has been picked up before

Our comment below:

IRP550 adds an alternate response for GCS20r "Read non-critical event and alert behaviours - GSME- Supplier" but the Frame Control for that response is not correct according to ZigBee.

The Frame Control is defined as "Profile-wide; not manufacturer specific; server-client; allow default response;" but the direction should be changed from "server-client" to "client-server". This is because the GSME is the client in the Device Management cluster and the response is sent from the client to the server.

Proposed Position:

The point made in the issue is correct.  The proposed position is to correct the GCS20r Message Template (by amending GBCS Table 20), to align to the ZSE requirements as to the use of the bits in the Frame Control field.

To explain, the ZigBee Cluster Library (ZCL) specifies how fields in the ZCL Header are to be completed.  Specifically, the ZCL specification section 2.4.1.1.3 defines bit 3 within the Frame Control Field (so the ‘Direction’ sub-field) as follows:

‘The direction sub-field specifies the client/server direction for this command. If this value is set to 1, the command is being sent from the server side of a cluster to the client side of a cluster. If this value is set to 0, the command is being sent from the client side of a cluster to the server side of a cluster.’

In the case of GCS20r, the ZCL command sent in the GBZ Command is GetEventConfiguration.  As per ZSE Table 10.2.4, the command is generated by the server (in a ZSE Cluster sense), and so is sent from the server to the client.  Therefore, a GCS20r Response is from the client to the server, meaning that bit 3 of the Frame Control Field in such responses would be 0.

IRP 550 incorrectly stated, in column N of the two additional rows (headed ‘Direction (0 = client to server)’) that the value should be 1.

Interoperability and / or Compatibility

IRP550 is marked in the issues log so that GSME should implement it from Release 2, and so IRP550 relates to GSME in operational use today.

Those GSME may have either implemented as specified in IRP550, or as required by ZSE (and so as required by this IRP).  Neither implementation should cause interoperability issues, since the values in the Frame Control field are not parsed or validated by either the DSP or P&C.

Required Changes to Documentation:

When incorporating IRP550 in GBCS, set the values column ‘N’ of the two additional rows (headed ‘Direction (0 = client to server)’) in the ‘Zigbee Commands’ tab to 0.

Note: If this IRP604 is applied after IRP550 has been incorporated, then the change will be need made to GBCS, Mapping Table 20, ‘Zigbee Commands’ tab, column ‘N’ for the rows that correspond to those highlighted in the worksheet attachment from IRP550, embedded below.

This change generates the revised GCS20r Use Case / Message Template embedded below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | Must be included at the same time as, or after IRP550 |
| Attachment(s) |  |