This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

Proposed Change: SECMP0002 - Add New Command to Reset Debt Registers

#### Requirement

This section summarises the key parts of the requirement for this SEC Modification, as currently clarified and refined by the Working Group. The following solution(s) sections detail the required changes to SEC and wider documents that are proposed to implement this requirement. Where there are multiple solutions sections, they reflect the alternatives the Working Group wished to consider.

Context

SMETS requires that there are three debt registers on GSME and ESME, specifically:

- One Payment Debt Register, where the debt is recovered by deducting a configurable percentage of each credit added; and

- Two Time Debt Registers, where the debt is recovered by deducting a specified amount from the Meter Balance per period of time

Similar to the Meter Balance, there are SMETS commands to adjust these Debt Registers and read the values in them. Unlike Meter Balance, the are no SMETS commands to reset these three Debt Registers to zero.

Change

The change here is to add Commands, so that each of the three Debt Registers can be reset to zero independently. In common with the reset Meter Balance Command, these reset Commands would be usable in both credit and prepayment mode (although would, in line with SMETS requirements, only affect any debt recovery when in prepayment mode).

The Commands would each report success if the Debt Register in question is zero once the Command has executed. This means that Responses to these Commands would not contain any data that can be considered as ‘personal’ in accordance with the Data Protection Act. Thus, there are no Response encryption requirements.

The implementation approach, in terms of the new, corresponding GBCS Use Cases is:

- on GSME, to use the same ZSE Change Debt command that is used in the existing adjust Debt Register Use Cases, but with the parameters set to achieve the reset behaviour required;

- on ESME, to use the same DLMS COSEM objects that are used in existing Use Cases for these three Debt Registers, and to use the corresponding DLMS COSEM specified method to reset debt registers on them;

- to adopt the same GBCS Message Categories and access rights as the existing adjust Debt Register Use Cases, since the same rationale applies.

In line with the wider SEC approach, there is no requirement to update already installed ESME or GSME to support these new Use Case. The new Use Cases do not require any changes to the ZigBee attributes / Commands to be supported over the SMHAN. Thus, there is no impact on the design of, or requirements for, other HAN Devices. The Communications Hub is only required to route these new Commands. There are no additional CH processing requirements.

The GSME Commands would not be TOM commands. For the avoidance of doubt, the target device for the Command is GSME and ESME.

From a DCC User perspective, access to these Commands would be provided by a Service Request, with a choice of Debt Register to reset to zero. Only one Debt Register would be able to be reset at one time.

Given that the ZSE Change Debt command is to be the subject of anomaly detection packet inspection by the DCC when used in existing Use Cases because it includes a supply affecting parameter, the new GSME Use Cases would also be subject anomaly detection packet inspection. In this case, anomaly detection packet inspection should check that the value of that supply affecting parameter is set to zero.

The new ESME Use Cases do not use DLMS COSEM mechanisms that are to be subject to anomaly detection packet inspection by the DCC for existing Use Cases because the reset behaviour is achieved by the reset method, and not a parameter that is set to zero. Thus, the ESME Use Cases are not required to be subject to anomaly detection packet inspection.

Given that the new Use Cases would be Critical, they would be in scope of threshold anomaly detection, so affecting both DCC Users and the DCC in this area.

Business Requirements relating to testing

- The DCC will provide Testing Services to support the implementation of SECMP0002 to assess the interoperability of User Systems with DCC Systems and Smart Metering Devices.

- The DCC will provide an analysis including supporting assumptions and rationale, of any testing required to the DCC Total System.

- The DCC will prepare a report setting out the scope, phases, timetable, Testing Participants, any assumptions and rationale in relation to SECMP0002 testing.

- The testing environment that the DCC provides in support of SECMP0002 as part of Testing Services will be open to all User Roles eligible to send the Service Request (Import and Gas Suppliers). This environment should be made available for a minimum of 15 Working Days, depending on the impact of the change. The DCC must provide the costs and assumptions associated with providing this Testing Service, including whether the testing costs are based on a set number of Users utilising the Testing Service, i.e. up to 10 Users, noting that at least two Large Suppliers may test the functionality. This is to ensure it operates correctly before it is put into the End-to-End and Production environments. Noting:

  - Updating Service Request mappings; and

  - Testing Issues management

shall be voluntary.

- The objective of testing as part of the Testing Services will be to ensure that, in response to the Service Request, the User receives the corresponding Service Response from the DCC.

- As part of the Testing Services, the DCC will provide Users with a corresponding version of the Parse and Correlate software and Message Mapping Catalogue.

- The acceptance criteria for testing as part of the Testing Services will be, following successful execution of the corresponding Command, the User receives the corresponding Service Response from the DCC that a debt register is reset to zero.

- The DCC will allow Users to add Type 1 and Type 2 Devices to their test Smart Metering Systems. Such Devices will be supplied by Users.

- As reasonably required, the DCC will provide:

- a reasonable number of Test CH for use in the testing environment which represent every combination of HAN and WAN Variant;

- Test Stubs (or other alternative arrangements) to emulate meter behaviour of version(s) of SMETS in force prior to the Release as well as the version of SMETS which will be effective on the Release date.

- As the DCC will apply all applicable ADTs to the Signed Pre-Commands in the test environment, Users will be expected to set User volume ADTs and data value ADTs as applicable. The process for setting the ADTs as well as for receiving notifications when a Signed Pre-Command has been quarantined will be part of the test environment.

#### Solution

This section details the required changes to SEC and wider documents that are proposed to implement the requirement for this SEC Modification. Please note that all numbering, messages codes, alert codes and so on are based on GBCS 2.0 Draft 4.

SMETS changes

Add the following 4 sections to SMETS, for the new HAN interface commands:

      - 4.5.3.24 Reset Payment Debt Register

A Command to reset the Payment Debt Register(4.6.5.13) to zero.

      - 4.5.3.25 Reset Time Debt Registers [1..2]

Commands to independently reset each of the Time Debt Registers [1 … 2](4.6.5.21) to zero.

      - 5.6.3.37 Reset Payment Debt Register

A Command to reset the Payment Debt Register [INFO](5.7.5.23) to zero.

      - 5.6.3.38 Reset Time Debt Registers [1..2]

Commands to independently reset each of the Time Debt Registers [1 … 2] [INFO](5.7.5.36) to zero.

CHTS changes

None

GBCS changes

Add the rows with green shading in each of the tabs in the embedded document ‘SECMP0002 Table 20’, to the corresponding tab in GBCS Table 20:

This adds the new Use Cases and corresponding GBZ and DLMS COSEM Message Templates. The additions are as per the following embedded documents

Resulting Use Cases and GBZ Message Templates

Resulting DLMS COSEM Message Templates:

SEC Appendix E changes

For the corresponding Smart Energy Code version:

Add the row below to SEC Appendix E: DCC User Interface Services Schedule:

[DN: this recognises a new Service Request is required]

| Service Reference | Service Reference Variant | Description | Eligible Users | Target Response Time | Non-Device Services | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 2.4 | 2.4 | Reset Debt | Import Supplier, Gas Supplier | 30 seconds |  |  |

DUIS changes

[tbc]

MMC changes

[DN: tbc]

Other SEC document changes

[DN: tbc in terms of anomaly detection]