| SECAS Contact:. Name: Sasha Townsend. Number: 020 7191 1534. Email: SEC.change@gemserv.com. Draft Legal Text |
| --- |
| SECMP0002: |
| Add new Command to reset Debt Registers |
| Summary This Modification Proposal seeks to add a new Smart Metering Equipment Technical Specification (SMETS) Command to allow Users to reset any of the three debt registers on an Electricity Smart Metering Equipment (ESME) or Gas Smart Metering Equipment (GSME). |  |
| About this document This document contains the draft SEC legal text that will deliver the intent of SECMP00002. |  |

# Draft Legal Text

This section sets out the draft legal text changes for SECMP0002. These changes have been drafted against the Smart Energy Code (SEC) version 5.10.

Please note that SEC Schedule 8 contains embedded documents which are also being amended by this modification. These embedded documents are included below as they would be set out in SEC Schedule 8. To review the changes to the embedded documents, you can click on and open these files.

## SEC Schedule 8 ‘Great Britain Companion Specification’

Add the rows with green shading in each of the tabs in the embedded document ‘SECMP0002 Table 20’, to the corresponding tab in GBCS Table 20:

This adds the new Use Cases and corresponding GB Zigbee (GBZ) and Device Language Message Specification Companion Specification for Energy Metering (DLMS COSEM) Message Templates. The additions are as per the following embedded documents.

Resulting Use Cases and GBZ Message Templates:

Resulting DLMS COSEM Message Templates:

## SEC Schedule 9 ‘Smart Metering Equipment Technical Specifications’

### Add the following 4 sections to SMETS, for the new Home Area Network (HAN) interface Commands:

      - 4.5.3.24 Reset Payment Debt Register

A Command to reset the Payment Debt Register(4.6.5.13) to zero.

      - 4.5.3.25 Reset Time Debt Registers [1..2]

Commands to independently reset each of the Time Debt Registers [1 … 2](4.6.5.21) to zero.

      - 5.6.3.37 Reset Payment Debt Register

A Command to reset the Payment Debt Register [INFO](5.7.5.23) to zero.

      - 5.6.3.38 Reset Time Debt Registers [1..2]

Commands to independently reset each of the Time Debt Registers [1 … 2] [INFO](5.7.5.36) to zero.

## SEC Schedule 11 ‘Technical Specification Applicability Tables’

### Add a new row as below:

Table 1 SMETS and Relevant Versions of GBCS

| SMETS Version | Installation Start Date | Installation End Date | Maintenance Start Date | Maintenance End Date | Relevant GBCS Version(s) | GBCS Applicability Period Start Date | GBCS Applicability Period End Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMETS v[X.X] | TBC | Not yet determined | TBC | Not yet determined | [TBC] | [TBC] | Not yet determined |

## SEC Appendix E ‘DCC User Interface Services Schedule’

### Add a new row as below:

| Service Reference | Service Reference Variant | Description | Eligible Users | Target Response Time | Non-Device Services | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 2.4 | 2.4 | Reset Debt | Import Supplier, Gas Supplier | 30 seconds |  |  |

## SEC Appendix AD ‘DCC User Interface Specification’

## SEC Appendix AF ‘Message Mapping Catalogue’

These documents are still currently with the DCC, and we will include these as part of the Modification Report Consultation.

Note that if a new version of the SEC is designated before the submission of the Final Modification Report to the Change Board or Authority, the drafting will be checked to make sure there are no consequential impacts. Our current understanding is that the specifications that SECMP0006 is amending are not expected to change as part of any forthcoming designations.

The “Installation Start Date” will be the SECMP0002 implementation date, which is to be determined by the Authority.

The “Maintenance Start Date” will be the same as the “Installation Start Date”.