# Proposed Change: SECMP0005 - Include Tariff and Register Labels in SMETS2 Devices

#### Requirement

This section summarises the key parts of the requirement for this SEC Modification, as currently clarified and refined by the Working Group. The following solution(s) sections detail the required changes to SEC and wider documents that are proposed to implement this requirement.

Summary requirement

This change is to allow the setting of tariff and Time Of Use (ToU) price labels on GSME & ESME as part of setting the tariff, and the sharing of those labels with other HAN Devices, so that those other Devices can display the labels. The price labels are for the Tariff ToU Price Matrix and, on Twin Element ESME only, the Secondary Tariff TOU Price Matrix.

Context

The ZigBee protocol is used to share information between Devices on the HAN.

There is a ZigBee parameter (in the ZigBee Publish Tariff Information command) called ‘TariffLabel’. It can hold a label for a tariff of up to 24 UTF-8 characters. The current Technical Specifications do not require this label but ZigBee requires it is set to a value. Thus, the current Technical Specifications set this label to a zero length string on GSME (ESME configuration data is not set using ZigBee and so no value is set on ESME currently).

There are also ZigBee parameters (in the PublishTierLabels command) that can hold ‘tier price labels’ for up to 48 Time Of Use (ToU) prices in a tariff [note: there is no ZigBee mechanism for block / ToU with blocks or export related price labels to be set or shared]. Each price label can hold up to 12 characters. The current Technical Specifications do not require these labels and so do not provide a way to set them.

GBCS maps each of the ToU prices to a specific, unique ToU register. Thus, ZigBee’s price labels could, if required by GBCS, be considered as register labels. The term ‘price label’ is used in the remainder of this document, to align to ZigBee.

The ZigBee command used to share Active Tariff Price with Devices on the HAN (‘Publish Price’) contains both the active price and a ‘Rate Label’ parameter, which can be used to share the label that relates to the Active Tariff Price (where that is a ToU price).

Tariff and price labels form part of ZigBee’s ‘price’ cluster functionality. This means a copy of the GSME labels cannot be maintained on the GPF using ZigBee’s mirroring mechanism. Rather, the GBCS’s ‘TOM’ mechanism would be needed to maintain such a copy.

Change

In outline, the change involves adding to the Technical Specifications so that tariff labels and price labels (encoded as UTF-8 octet strings) can be set on ESME and GSME, and can be made available to other HAN Devices for display. The price labels are for the:

- Tariff TOU Price Matrix; and

- Secondary Tariff TOU Price Matrix (for Twin Element ESME only).

Setting and reading GSME / ESME values

In SMETS, tariff label and price label configuration data is to be added for both GSME and ESME. In line with SMETS, there must be Use Cases to set and read such configuration data. Thus, corresponding new Use Cases are specified.

The Use Cases are specified to align with the Working Group clarifications that tariff labels and price labels are integral parts of tariffs and so, where set, must be set as part of the corresponding Set Tariff Use Case (so as to avoid tariffs being replaced without the corresponding labels being replaced).

In line with the general versioning approach, the corresponding existing Use Cases to set tariffs are to be removed, except for the TOM Use Cases, support for which is to be retained in CH only, for reasons of backwards compatibility [This is so that a CH supporting this Modification will continue to work with a GSME that does not].

New Use Cases are specified to read the tariff labels and price labels. These do not include any existing read Use Case functionality and so no existing Use Cases are to be removed. Specifically, no existing Use Cases to read tariff information are to be removed.

Note that the new ‘Set Tariff and Price With Labels’ Use Cases will, when future dated, result in additional ‘Future Dated Execution Of Instruction Alerts’ being generated by the GSME / ESME, as compared to the existing Use Cases. The additional one on GSME relates to price labels (the tariff label is within a ZSE command used in the existing Use Case and so covered by an alert generated by that existing Use Case). ESME will generate two additional alerts – one for the tariff label and the other for the price label. DCC and Supplier systems will need corresponding revisions. These are existing future dated execution alert Message Codes and Alert Codes, so do not need specification in GBCS [the existing specification covers how the Alerts must be constructed]. The payloads of the Alerts specify what they relate to. DCC does not process the content of such Alerts, but does count the total number of future dated Alerts received relating to a future dated command. Hence there will be some internal DCC change due to the different number of Alerts.

Mirroring GSME information

When GSME information is shared with HAN Devices, a copy of the information has to be maintained in the GPF. Given that the changes in this document relate to the ZigBee ‘price’ cluster, the GBCS TOM mechanism has to be used (since there is no ZigBee mechanism to mirror price cluster details). Thus, changes are required to CHTS and GBCS to achieve the required mirroring. Note also, that to avoid potential interoperability issues, the changes include the GPF by default reporting an empty string for each label to other HAN Devices, since the GPF may not receive values from the GSME (e.g. where the CH is a replacement but connected to a GSME that does not support these changes).

Sharing information with HAN Devices

Which Devices share which information over the HAN with other Devices is specified in GBCS section 7.4. Changes to that section are therefore specified.

Displaying information on Device’s User Interfaces

As per the Modification Proposal, there is no requirement for ESME / GSME to display tariff labels or price labels. However, the Working Group does wish labels to be displayed on IHDs and PPMIDs.

In terms of prices, IHDs and PPMIDs are only required to display the Active Tariff Price. This change requires that IHDs and PPMIDs can also display the associated Active Tariff Price Label, where the price in question is a price from the Tariff TOU Price Matrix (as earlier, there is no ZigBee mechanism to support labels for prices in the Tariff Block Price Matrix).  To support this in SMETS, ESME and GSME would have an additional item of operational data (Active Tariff Price Label).  Alongside this, the change requires IHDs and PPMIDs to be able to display the Active Tariff Price Label.

This requires changes to SMETS. Additionally, there is a GBCS change to detail how the Active Tariff Price Label is to be shared, in terms of ZSE command parameters.

DCC User access

As per Working Group discussion, access to both the existing and new ‘set tariff’ Use Cases will be provided through the existing Update Import Tariff Service Requests (SRV 1.1.1 and SRV 1.1.2). This will be achieved by adding optional elements to those SRs to allow tariff / price labels to be specified in the triggering SR.

This gives the following combination of input SR parameters, target Devices and resulting DCC action (the table below uses SRV 1.1.1 to illustrate but the structure is the same for 1.1.2):

| Input SR | Target Device | Resulting DCC action |
| --- | --- | --- |
| No labels specified | Labels (so this modification) not supported | Create Pre-Command: - GCS01a with single label set to zero length string - ECS01a |
|  | Labels supported | Create Pre-Command: - GCS01c with all labels set to zero length string - ECS01a with all labels set to zero length string |
| Labels specified | Labels not supported | Create Pre-Command: - GCS01a with single label set to zero length string - ECS01a DCC sends warning Correlate software is changed to ignore / warn on semantic difference between SR and Pre-Command. |
|  | Labels supported | Create Pre-Command: - GCS01c with all labels set as specified - ECS01a with all labels set as specified |

Note that the above approach creates a potential error case, specifically where the DCC believes the target Device supports the new Commands but the Device does not, or the DCC believes the target Device does not support the new Command but the Device does not. In both cases:

- this would be caused by the DCC’s SMI having a different firmware version recorded than that on which the Device is operating (e.g. because the Device was incorrectly notified; because a Firmware Activation Response / Alert was lost);

- this would result in the Device rejecting the Command (since the Device must not support the Critical Command in question), and so would send an Alert to its Supplier;

- this issue could then be addressed by the Supplier successfully reading firmware version from the Device, since DCC will update the SMI to the correct firmware version based on the Response to such Commands to read firmware version. The Update Import Tariff SR can then be re-executed, and the correct resulting Pre-Command created.

Access to the new Use Cases for reading tariff labels and price labels may require new SRs – to be proposed by DCC.

Wider factors

In line with the wider approach with SEC Modification Proposals, there will be no requirement to update already installed Devices to support these new Use Cases. That will need to be factored in to the design and deployment of future Devices.

HAN interoperability

A CH that supports this change would work correctly with a GSME that does not.

A GSME that supports this change would not work correctly with a CH that does not. Specifically, the GPF would not mirror tariff, prices or labels and so IHDs/PPMIDs/CADs would not have reliable access to this information and Remote Party Commands to read tariff, price or labels from the GPF would not return accurate information.

Device replacement on the HAN may lead to a mixture of support for the changes in this document within the same HAN. For instance:

- If an existing GSME (not supporting this change) is replaced with one that does, then the CH would need to be upgraded (either by firmware update or replacement) so that mirroring of tariff and price information by the GPF continues to work (although the mirror of tariff information is existing CH functionality for an unchanged CHTS requirement, it is affected by the introduction of the new Set Tariff Use Cases, and so changes to the CH would be needed);

- If CH is replaced with one that supports this change but the GSME does not support it, the GPF will report zero length strings for label values to other Devices. Thus, no upgrade to the GSME would be required;

- If a replacement IHD or PPMID is installed, it cannot be assumed that the GPF or ESME will support the sharing of labels.

More broadly, on initial installation, the CH must support the changes in this document if the GSME supports them. The converse is not required (e.g. if the CH supports them, the GSME does not have to). This will be a logistical constraint during periods where two versions of SMETS/CHTS are in force in parallel for newly installed Devices.

Security factors

Given that the Set Tariff command is to be the subject of anomaly detection packet inspection by the DCC when used in existing Use Cases, the new Set Tariff Use Cases would be subject to the same requirement (since they contain the same parameters).

Given that the new Set Tariff Use Cases would be Critical, they would fall within the scope of threshold anomaly detection, so affecting both DCC Users and the DCC in this area, including in terms of their handling of anomaly thresholds.

Business Requirements relating to testing

- The DCC will provide Testing Services to support the implementation of SECMP0005 to assess the communications and interoperability of User Systems with DCC Systems and Smart Metering Devices.

- The DCC will provide an analysis including supporting assumptions and rationale of any testing required to the DCC Total System.

- The DCC will prepare a report setting out the scope, phases, timetable, Testing Participants, any assumptions, and rationale in relation to SECMP0005 testing.

- The testing environment that the DCC provides as part of Testing Services will be open to all User Roles eligible to send the Service Request (Import and Gas Suppliers). This environment should be made available for a minimum of 15 Working Days, depending on the impact of the change. The DCC must provide the costs and assumptions associated with providing this Testing Service, including whether the testing costs are based on a set number of users utilising the Testing Service, i.e. up to 10 Users, noting that at least two large Suppliers may test the functionality. This is to ensure it operates correctly before it is put into the End-to-End and Production environments.

- The objective of testing as part of the Testing Service will be to ensure that in response to the Service Request, the User receives the corresponding Service Response from the DCC.

- As part of the Testing Service, the DCC will provide Users with a corresponding version of the Parse and Correlate Software and Message Mapping Catalogue.

- The acceptance criteria for testing will be that following successful execution of the corresponding Command, the User receives the corresponding Service Response from the DCC that:

  - where no labels are specified in SR targeted to an ESME/GSME that supports labels, following successful execution of the corresponding Command, labels on the ESME/GSME are set to a zero length string.

  - where labels are specified in SR targeted to an ESME/GSME that supports labels, following successful execution of the corresponding Command, labels on the ESME/GSME are set as specified.

  - where labels are specified in SR targeted to an ESME/GSME that does not support labels, following successful execution of the corresponding Command, labels on the ESME/GSME are set to a zero length string.

- The DCC will allow Users to add Type 1 and Type 2 Devices to their test Smart Metering Systems. Such Devices will be supplied by Users.

- As reasonably required, the DCC will provide:

- a reasonable number of Test Communications Hubs for use in the testing environment which represent every combination of HAN and WAN Variant;

- Test Stubs (or other alternative arrangements) to emulate meter behaviour of version(s) of SMETS in force prior to the Release as well as the version of SMETS which will be effective on the Release date.

- As the DCC will apply all applicable ADTs to the Signed Pre-Commands in the test environment, Users will be expected to set User volume ADTs and data value ADTs as applicable. The process for setting the ADTs as well as for receiving notifications when a Signed Pre-Command has been quarantined will be part of the test environment.

#### Solution

This section details the required changes to SEC and wider documents that are proposed to implement the requirement for this SEC Modification. Please note that all numbering, messages codes, alert codes and so on are based on GBCS 2.0 Draft 4.

SEC main body change

None

SMETS changes

For the new configuration and operational data items, add the following 7 sections to SMETS:

#### 4.6.4.35 Tariff Label

A label for the Tariff.

#### 4.6.4.36 Tariff TOU Price Label Matrix

A 1 x 4 matrix containing labels corresponding to the entries in the Tariff TOU Price Matrix(4.6.4.32).

#### 4.6.5.22 Active Tariff Price Label

If Active Tariff Price(4.6.5.2) is a Price from the Tariff TOU Price Matrix (4.6.4.32) then the corresponding label from the Tariff TOU Price Label Matrix(4.6.4.36). Otherwise empty.

#### 5.7.4.52 Tariff Label [INFO]

A label for the Tariff.

#### 5.7.4.53 Tariff TOU Price Label Matrix [INFO]

A 1 x 48 matrix containing labels corresponding to the entries in the Tariff TOU Price Matrix [INFO] (5.7.4.50).

#### 5.7.5.37 Active Tariff Price Label [INFO]

If Active Tariff Price [INFO](5.7.5.5) is a Price from the Tariff TOU Price Matrix [INFO] (5.7.4.50) then the corresponding label from the Tariff TOU Price Label Matrix [INFO] (5.7.4.53). Otherwise empty.

#### 5.13.1.3 Secondary Tariff TOU Price Label Matrix [INFO]

A 1 x 4 matrix containing labels corresponding to the entries in the Secondary Tariff TOU Price Matrix [INFO]( 5.13.1.1).

To add the IHD display requirements, add the following 4 sections:

#### 6.4.3.11 Tariff Label

A label for the Tariff.

#### 6.4.3.12 Active Tariff Price Label

A label for the Active Tariff Price[NUM](6.4.3.1).

#### 6.4.4.13 Tariff Label

A label for the Tariff.

#### 6.4.4.14 Active Tariff Price Label

A label for the Active Tariff Price[NUM](6.4.4.1).

To add the Gas PPMID display requirements, to the bottom of the list in section 7.4.5, make the following additions (underlined) and deletions (strikethrough):

xi.	the Contact Details(4.6.4.4); and

xii.	the Supply State(4.6.5.18);

xiii	the Tariff Label(4.6.4.35) and

xiv	the Active Tariff Price Label(4.6.5.22).

To add the Electricity PPMID display requirements, to the bottom of the list in section 7.4.6, make the following additions (underlined) and deletions (strikethrough):

xi.	the Contact Details(5.7.4.8); and

xii.	the Supply State(5.7.4.32);

xiii	the Tariff Label[INFO] (5.7.4.52) and

xiv	the Active Tariff Price Label[INFO] (5.7.5.37).

CHTS changes

To require GPF sharing of the labels, to the bottom of the list in section 4.5.3, make the following additions (underlined) and deletions (strikethrough):

- xxxii. Time Debt Registers [1 … 2]; and

- xxxiii. Payment-based debt payments in the Billing Data Log;

- xxxiv. Active Tariff Price Label;

- xxxv. Tariff Label; and

- xxxvii. Tariff TOU Price Label Matrix.

GBCS changes

- To require that ESME and GPF make price labels available to HAN Devices, add the rows to the end of GBCS Table 7.4 as shown by the green highlighting in the following embedded document. [Note that Tariff Labels are shared in Publish Tariff Information commands which ESME and GPF already need to support. Similarly, the Active Price Tariff Label can be shared by the Publish Price commands which ESME and GPF already need to support].

- To ensure that the Active Price Tariff Label is shared as part of Publish Price commands, add the following text to the end of GBCS Section 7.4:

Where a Publish Price command is sent by an ESME to another Device on the SMHAN, the Rate Label parameter shall, where the command relates to a price in a Time of Use price matrix, contain the value of the corresponding TierNPriceLabel and shall otherwise contain a zero length string.

- To require Devices to hold a current and a future version of labels, to only activate the future one when specified and to respond with the current one when queried, to the end of section 10.4.2.11, add the following text:

GSME and GPF shall be able to process at least two instances of the PublishTierLabels command, the current and the next instance (to be activated in the future).

In common with all other instructions in TOM Commands, a GPF shall only process a PublishTierLabels command in a Command with GBZ Payload if and when it has confirmation from the GSME that the labels have been changed.

A GSME and a GPF shall respond to a GetTierLabels command in a GBZ payload with the labels currently in use.

- To detail future dated execution mechanism in GBCS (since ZigBee does not provide one), to the end of section 10.4.2.11, add the following text:

GSME shall only execute a PublishTierLabels command from a GBZ payload at the ‘From Date Time’ specified in the GBZ Extended Header for that command.

- To support interoperability between CH and GSME at different versions of the specifications, to the end of section 10.4.2.11, add the following text:

On installation, the values in the GPF in attributes Tier1PriceLabel through to TierNPriceLabel shall each contain a zero length string, where N = 4.

- To require mirroring using the TOM mechanism, to the end of the bulleted list section 10.3.4.1, add the following bullet:

- 0x00F4 (GCS01c Set Tariff and Price on GSME with labels);

- To add the new Use Cases, apply the following changes to GBCS Table 20 (Mapping Table):

- In the SMETS Required objects tab:

  - Select all rows where column AJ contains ‘ECS01a’ and, for each selected row, append <alt+enter> with ‘ECS01e Set Tariff and Price on ESME with labels’ at the end of the existing text in column AJ;

  - Select all rows where column AJ contains ‘ECS01c’ and, for each selected rows, append <alt+enter> with ‘ECS01f Set Tariff and Price on ESME with labels - second element’ at the end of the existing text in column AJ; and

  - Select all rows where column AM contains ‘GCS01a’ and, for each selected row, append <alt+enter> with ‘GCS01c Set Tariff and Price on GSME with labels’ at the end of the existing text in column AM.

- In the ZigBee Commands tab:

  - Select all rows where column E (Use Case) contains ‘GCS01a’, copy all selected rows to the end of the tab and, for each copied rows, set column E to ‘GCS01c’; and

  - In the newly copied rows, identify the row containing ‘Tariff Label’ in column B then:

    - set the value in column G to ‘*’

    - set the value in column L to ‘*’ and

    - set the value in column I to ‘The first octet is the length of the UTF-8 string and the remaining octets are the UTF-8 string of up to 24 octets’.

- From the SMETS required objects tab in the embedded document ‘SECMP0005 Table 20’, add the last nine green shaded rows to the end of the corresponding tab of in GBCS Table 20.

- From the Use Case Reference tab in the embedded document ‘SECMP0005 Table 20’, add the last four green shaded rows to the end of the corresponding tab in GBCS Table 20.

- To remove the requirement for Smart Meters to support the existing Set Tariff and Price Use Cases (but retain the requirement that CH need to support ‘Tapping Off’ of the old GSME Use Case, to support backward compatibility):

- To remove the existing ESME Use Case and make clear the old Use Case is not to be supported by GSME, make the following changes to the Use Case reference tab of the GBCS Table 20:

  - since the GSME Use Case cannot be used on GSME to this Specification:

    - copy row 25 to the end and in the copied row:

      - clear cells in columns B:D;

      - set cell in H to ‘-‘;

      - set cell in AM to ‘None’;

      - clear cell in column AN; and

      - set cell in AE to ‘This Use Case is only retained as a CH requirement in relation to the Tapping Off Mechanism. CHs are required to process any GCS01a Commands and Responses received in line with the 'Tapping Off Mechanism''

  - since the ESME Use Cases are being removed, in rows 25 and 27: [Note: this drafting is to align to BEIS proposed marking of removed Use Cases]

    - clear all cells in the row 25 except for columns A:D;

    - set column A to ‘This row is not used’;

    - set column B to ‘x;’ and

    - add a comment to column C ‘Unused since GBCS [TBC]’; and [DN: the TBC will depend on the GBCS Version in which this Modification is implemented]

- To remove the references to the existing ESME Use Cases (ECS01a and ECS01c), in column AJ of the SMETS Required Objects tab, delete all occurrences of:

  - ‘ECS01a Set Tariff and Price on ESME’ and

  - ‘ECS01c Set Tariff and Price on ESME secondary’.

The cells in the Table 20 (Mapping Table) affected by the changes described above are shown in the embedded ‘SECMP0005 Table 20’ with green shading:

[To note that the minimum size of ECS01e Commands would now exceed one block]

Resulting Use Cases and GBZ Message Templates:

Resulting DLMS COSEM Message Templates:

DCC User Interface Services Schedule changes

[DN: tbc]

DUIS changes

[DN: tbc]

MMC changes

[DN: tbc]

Other SEC document changes

[DN: tbc in terms of anomaly detection]

Changes to documents outside of SEC

None