SECMP0007 ‘Firmware updates to IHDs and PPMIDs’

Annex C

SEC Schedule 8 ‘GB Companion Specification’ legal text – version 1.0

# About this document

This document contains the redlined changes to the GB Companion Specification (GBCS) that would be required to deliver this Modification Proposal.

These changes have been redlined against Schedule 8 ‘GB Companion Specification’ v3.2.

The temporary table of contents will be removed upon application of the changes to a final version of the GBCS.

The changes to the embedded tables in the GBCS will be made according to the documents in Appendix 1 of this document.

The Embedded Use Cases in Section 19.3 of the GBCS will be regenerated using the changes highlighted in Table 20 in Appendix 1 of this document.

Contents

About this document	1

11	Downloading firmware images to Devices	3

11.1	Introduction – informative	3

11.2	Common Requirements	4

11.2.1	Transport of firmware images	4

11.2.2	Construction of Upgrade Image	5

11.2.3	Construction of OTA Upgrade Image	6

11.2.4	Construction of Manufacturer Image Hash	6

11.2.5	Verification of the authenticity of the Upgrade Image	6

11.2.6	Construction of Firmware Distribution Receipt Alert	7

11.2.7	Activation of firmware images	7

11.3	CS05a Distribute Firmware to Communications Hub	7

11.3.1	Pre-conditions	8

11.3.2	Detailed Steps	8

11.4	CS05b Distribute Firmware to ESME / GSME / HCALCS	8

11.4.1	Pre-conditions	9

11.4.2	Detailed Steps	9

11.5	CS06 Activate Firmware	10

11.5.1	Pre-conditions	11

11.5.2	Detailed Steps	11

11.6	CS05c Distribute Firmware to PPMID	14

11.6.1	Pre-conditions	14

11.6.2	Detailed Steps	15

11.7	CCS08 Firmware Transfer Alert	16

11.7.1	Description	16

11.7.2	Use Case Cross References	16

11.7.3	Construction of Alerts	16

11.7.4	Firmware Transfer Alert – structure definition	17

11.8	CS08 Read PPMID/HCALCS Firmware Version	18

11.8.1	Description	18

11.8.2	Use Case Cross References	18

11.8.3	Construction of Command and Response	18

11.8.4	Construction of Alerts	19

11.8.5	Read PPMID/HCALCS Firmware Version Command, Response and Alert Payloads – structure definition	20

- Downloading firmware images to Devices

  - Introduction – informative

Compared to other Smart Metering messages, firmware images are large.  Further, each image is likely to be applicable to a significant number of Devices.  Thus, an end-to-end, unicast Message to each affected Device, with each message containing a copy of the image, is not efficient from a WAN perspective.

This leads to the firmware update process being separated into two stages for ESME, GSME, HCALCS and Communications Hubs:

- distribution of the image to end Devices without any activation of that image; and

- a separate and subsequent ‘activation’ Command to each Device.

The Distribute Firmware Command is not a Critical Command (since it does not affect the operating firmware) and does not need to be unicast.

The Activate Firmware Command is a Critical Command and so must be unicast – as it must be digitally signed and be for one, and only one specified Device.  Further, the Activate Command must apply to one, and only one, image and that image must have originated from the same party that signs the Activate Firmware Command (that is, the party responsible for that Device).  To meet these requirements:

- the Activate Firmware Command is of type SME.C.C and so the Signature and MAC on the Command shall have been verified by the Device prior to the Hash validation (see next bullet); and

- a Device receiving an Activate Firmware Command shall calculate a Hash over the Manufacturer Image it holds and ensure the Hash so calculated matches that in the Activate Firmware Command, before the Device attempts to activate the corresponding Manufacturer Image.

For PPMID the firmware image is delivered to the Communications Hub and transferred to the target Device. Upon successful transfer of the firmware image the Communications Hub instructs the PPMID to activate the firmware image.

The GBCS does not constrain the mechanisms used by Device manufacturers to ensure that only valid Manufacturer Images are activated on Devices manufactured by them.  The GBCS does require that the manufacturer information related to a Manufacturer Image is made available, so that the Upgrade Image and the ZigBee Over-The-Air (OTA) Header can be provided when requesting distribution of an image.

In common with other Messages, the GBCS shall not constrain the mechanisms by which the firmware Messages are transported to the Communications Hub.  The GBCS constrains HAN transport mechanisms to those provided by ZSE.

In line with the ZigBee OTA specification at section 5.1, the contents of Manufacturer Images sent to Devices are manufacturer defined.  Thus, a particular Manufacturer Image may consist of whatever the manufacturer requires to achieve the necessary update which could be a full image or just a patch to application code or any other manufacturer specified content.

Therefore the steps taken by a Device when it activates the contents of a particular Manufacturer Image are manufacturer specific and specified in the release note for that Manufacturer Image. Thus, activation of a Manufacturer Image means the set of manufacturer specified steps the Device takes in processing the content of the image and terms such as ‘activation’, ‘activate’, ‘activated’, ‘installed’ etc in the GBCS, SMETS and CHTS are meant in this sense.

Therefore, when a Device reports its ‘installed’ or ‘active’ firmware version it shall respond with either:

- the firmware version for its mostly recently successfully activated Manufacturer Image (with the value in the corresponding OTA Header and so the value in the Certified Products List (CPL)); or

- if it has never successfully activated a new Manufacturer Image, the firmware version it held on installation (which again would correspond to the CPL).

## Common Requirements

### Transport of firmware images

Italicised terms in this Section 11.2.1 shall have the meanings defined in ZigBee Document 09-5264r23.

For ESME, GSME, HCALCS and PPMID firmware image distribution, the ZigBee Over-The-Air (OTA) mechanisms shall be used for transport of the image over the HAN.  The ESME / GSME / HCALCS / PPMID firmware image delivered to the Communications Hub shall comply with ZigBee OTA format requirements.

Communications Hub firmware images shall not be transported over the HAN and so ZigBee OTA structures shall not be required.

Every Communications Hub shall be configured to act as the single OTA Server on its HAN.

ESME, GSME, HCALCS and PPMID shall be configured to act as an OTA Client.  The ESME, HCALCS and PPMID shall use the ‘Image Notify’ Command sent by the OTA Server to inform it that a new firmware image is available.  The GSME shall use the notification flags mechanism whereby a flag shall be set by the OTA Server to inform it that a new firmware image is available when requested.

The Communications Hub shall:

- as required by CHTS, have the capability to store one GSME OTA Upgrade Image and one ESME OTA Upgrade Image; and

- overwrite an ESME / GSME image with a subsequently delivered image for the same Device type unless:

      - the subsequently delivered image has Force Replace = 0x00; and

      - the Communications Hub has sent at least one Image Block Response Command relating to the already stored image but has not received a corresponding Upgrade End Request Command.

In such circumstances the Communications Hub shall not overwrite the currently stored image.

The Communications Hub shall make the GSME image available for fourteen (14) days and, after this period, replace the GSME image with an image for the PPMID / HCALCS, if one becomes available. If the transfer of a GSME image to the GSME is in process, the Communications Hub shall only replace this GSME image with an image for the PPMID / HCALCS once the GSME image transfer has completed.

The Communications Hub shall make the PPMID / HCALCS image available for fourteen (14) days unless a new PPMID / HCALCS / GSME image is available.

- If a PPMID / HCALCS / GSME Upgrade Image is discarded or replaced prior to having been successfully transported over the HAN, the Communications Hub shall send an Alert for each target Device Entity Identifier associated with the Upgrade Image File with the Alert Code 0x8F89 as specified in Section 11.7 by setting firmwareVersion to the Upgrade Image File version and transferResponseCode  to imageDiscarded.

Whenever the Communications Hub's OTA Server issues an Upgrade End Response Command to a GSME, ESME or HCALCS pursuant to this GBCS, the UpgradeTime parameter shall have the value 0xFFFFFFFF.

The OTA Server shall not issue Image Block Response Commands with WAIT_FOR_DATA status except where the OTA Server is communicating over a Sub GHz Channel and it is issuing such commands to manage Duty Cycle (with its ZSE meaning).

Contrary to section 6.13 of ZigBee Document 09-5264r23, the OTA Client shall not activate any Firmware for an ESME, GSME or HCALCS except as specified in Use Case CS06.

Whenever the Communications Hub's OTA Server issues an Upgrade End Response Command to a PPMID pursuant to this GBCS, the UpgradeTime parameter shall have the value 0x00000000 and the PPMID OTA Client shall activate the Firmware.

### Construction of Upgrade Image

For an ESME, GSME or HCALCS firmware image, the Authorising Remote Party shall be the Supplier for the target Device.

For a Communications Hub firmware image, the Authorising Remote Party shall be the WAN Provider for the target Device.

For an ESME, GSME, HCALCS or Communications Hub the Upgrade Image shall be the concatenation:

Manufacturer Image || Force Replace || 0x40 || Authorising Remote Party Signature

where:

- Manufacturer Image shall contain the firmware image the Device is to apply and any manufacturer specific data needed.  For clarity, the GBCS shall not constrain the structure or contents of Manufacturer Image;

- Force Replace shall be a single octet where Force Replace = 0x00 shall mean do not force the replacement of the currently stored image; and

- Authorising Remote Party Signature shall be calculated across the Manufacturer Image using the Authorising Remote Party’s Private Digital Signing Key.

For a PPMID firmware image, the Upgrade Image shall be the Manufacturer Image where:

Manufacturer Image shall contain the firmware image the PPMID is to apply and any manufacturer specific data needed.  For clarity, the GBCS shall not constrain the structure or contents of the PPMID Manufacturer Image.

    - Construction of OTA Upgrade Image

OTA Upgrade Image shall be the concatenation:

OTA Header || Upgrade Image

where OTA Header shall be populated according to Table 11.2.3.  For clarity, there shall be no other sub-elements present.

| OTA Header |  |  |  |
| --- | --- | --- | --- |
| ZigBee OTA Message Element | Contents | Length (octets) | Note |
| OTA upgrade file identifier | 0x0BEEF11E | 4 | Fixed by ZigBee OTA specification |
| OTA Header version | 0x0100 | 2 | Specified by current version of ZigBee OTA specification |
| OTA Header length | 0x003C | 2 | The length of ZigBee OTA Header which is decimal 60 |
| OTA Header Field control | 0x0004 | 2 | Detailing what is / is not present in ZigBee OTA Header |
| Manufacturer code | ZSE assigned identifier for the Manufacturer of the target Device | 2 | So this identifies the manufacturer producing the Manufacturer Image |
| Image type | Manufacturer specific | 2 | As per the ZigBee OTA specification, this is to differentiate products from the same manufacturer |
| File version | Manufacturer specific | 4 | As per the ZigBee OTA specification, this is to differentiate release and build numbers for the product in question |
| ZigBee Stack version | 0x0002 | 2 | ZigBee PRO |
| OTA Header string | Manufacturer specific | 32 | May be blank but is not required to be used in Device processing of the firmware image |
| Total Image size (including header) | The length in octets of OTA Upgrade Image | 4 | Contents to be interpreted as an unsigned integer |
| Minimum hardware version | Manufacturer specific | 2 |  |
| Maximum hardware version | Manufacturer specific | 2 |  |

Table 11.2.3:  Population of the OTA Header

The OTA Header shall uniquely identify a firmware image.

    - Construction of Manufacturer Image Hash

Manufacturer Image Hash shall be a Hash calculated across the whole Manufacturer Image file that is provided to the Authorising Remote Party or the Remote Party in case of the PPMID.

    - Verification of the authenticity of the Upgrade Image

The Device shall verify Upgrade Image by verifying the Authorising Remote Party Signature using Manufacturer Image and the Authorising Remote Party’s Public Key.  For clarity, this shall be the only ECDSA verification required by the GBCS and this is not the ZSE ECDSA Signature sub-element.

For an ESME, GSME or HCALCS receiving an Upgrade Image, the Authorising Remote Party’s Public Key shall be that held by the Device in the {supplier, digitalSignature, management} Trust Anchor Cell.

For a Communications Hub receiving an Upgrade Image the Authorising Remote Party’s Public Key shall be that held by the Device in the {wanProvider, digitalSignature, management} Trust Anchor Cell.

For a PPMID receiving an Upgrade Image the verification of the authenticity of the Upgrade Image in this section shall not apply.

    - Construction of Firmware Distribution Receipt Alert

If the Device is an ESME or HCALCS, the ‘Alert Payload’ fields shall be populated according to Section 7.2.9.

If the Device is a GSME, the ‘Alert Payload’ fields shall be populated according to Section 7.2.10.

In the above cases, the Device shall:

- populate the Use Case Specific Additional Content with the concatenation

0x0920 || the calculated Manufacturer Image Hash

- populate the Alert Code field with 0x8F1C (failure), or 0x8F72 (success).

- If the Device is a PPMID, no Firmware Distribution Alert shall be sent.

    - Activation of firmware images

The Activate Firmware Command shall be of type SME.C.C.

A Device other than the PPMID receiving such a Command shall undertake the verifications required of a SME.C.C Command.

If all such SME.C.C verifications succeed, the Device shall then calculate Manufacturer Image Hash over the Manufacturer Image it holds and compare that with the Manufacturer Image Hash specified in the Activate Firmware Command (see Use Case CS06 in Section 11.5 for details of the Activate Firmware Command Payload construction).

If the two Hashes match, the Device shall attempt to activate the firmware image.

If the two Hashes do not match, the Device shall not attempt to activate the firmware image.

The Device shall issue a relevant Activate Firmware Response detailing the success or failure (see Use Case CS06 in Section 11.5 for details of the Activate Firmware Command Payload construction).

See Section 11.6 for the handling of Upgrade Images for the PPMID.

  - CS05a Distribute Firmware to Communications Hub

This Use Case covers the distribution of an Upgrade Image that is intended for a Communications Hub to that Communications Hub.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command and Response |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | N/A (this Command does not have a Service Reference for CH) |
| Valid Target Device(s) | Communications Hub |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | WAN Provider |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific |

Table 11.3:  Use Case Cross References for CS05a Distribute Firmware to Communications Hub

    - Pre-conditions

None.

    - Detailed Steps

The Upgrade Image shall be constructed according to Section 11.2.2.

The Upgrade Image shall be transported to the Communications Hub.

The Communications Hub shall verify the Upgrade Image according to Section 11.2.5, verify the Upgrade Image is suitable for this Communications Hub, and discard the Upgrade Image where it fails to verify.

  - CS05b Distribute Firmware to ESME / GSME / HCALCS

This Use Case covers the distribution of an OTA Upgrade Image that is intended for a GSME, ESME or HCALCS to that GSME,ESME or HCALCS.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command and Response |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.1 |
| Valid Target Device(s) | ESME / GSME / HCALCS |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific to Communications Hub; ZigBee OTA from Communications Hub to ESME / GSME / HCALCS |

Table 11.4:  Use Case Cross References for CS05b Distribute Firmware to ESME / GSME / HCALCS

    - Pre-conditions

None.

    - Detailed Steps

Italicised terms in this Section 11.4.2 shall have the meaning specified in ZSE.

ESME, GSME and HCALCS shall use the ZCL Image Block Request and Image Block Response commands to retrieve available OTA images.

ESME, GSME and HCALCS shall not use the ZCL Query Specific File Request and Query Specific File Response commands.

The OTA Upgrade Image shall be populated according to Section 11.2.3.

The OTA Upgrade Image shall be transported to the Communications Hub through which the Device communicates.

The Communications Hub shall update its OTA Cluster to reflect availability of the OTA Upgrade Image, once the image is received by the Communications Hub.

The Communications Hub, as OTA Server, shall indicate availability of an OTA Upgrade Image differently for ESME, HCALCS and GSME:

- for ESME and HCALCS, the Communications Hub shall send a ZSE Image Notify command; and

- for GSME, the Communications Hub shall set a the New OTA Firmware flag (Bit Number 0) in FunctionalNotificationFlags.

For each ESME / GSME / HCALCS target Device Entity Identifier associated with the OTA Upgrade Image version the Communications Hub shall

- verify whether the ESME / GSME / HCALCS Hardware Version is present in the Query Next Image Request Command;

- if the Hardware Version is present and does not match the minimum and maximum Hardware Version in the OTA Header the Communications Hub shall set  transferResponseCode  to hardewareVersionMismatch as specified in Section 11.7 and process from step 5;

- if the Hardware Version is not present, or is present and matches the minimum and maximum Hardware Version in the OTA Header, the Communications Hub shall make the OTA Upgrade Image available to the ESME / GSME / HCALCS in the Query Next Image Response Command;

- the ESME / GSME / HCALCS shall download an OTA Upgrade Image when it is aware of the availability of a suitable OTA Upgrade Image using the QueryNextImage and Image Block/Page commands specified in the OTA Cluster specification4;

- upon the reception of the Upgrade End Request Command from the ESME / GSME / HCALCS the Communications Hub shall;

- set transferResponseCode to fileTransferSuccess if the transfer of the Manufacturer Image has completed successfully and process from step 4;

- set transferResponseCode  to fileTransferFailure if the transfer of the Manufacturer Image has failed and all transfer retries have been exhausted, and process from step 5;

- the Communications Hub shall create an Alert with the Alert Code field set to 0x8F8A as specified in Section 11.7, set firmwareVersion to the OTA Upgrade Image File version, include the transferResponseCode from the previous step and process from step 6;

- the Communications Hub shall create an Alert with the Alert Code field set to 0x8F89 as specified in Section 11.7, set firmwareVersion to the OTA Upgrade Image File version, include the transferResponseCode from the previous step and process from step 6;

- the Communications Hub shall send the Alert created in the previous step.

The ESME / GSME / HCALCS shall verify the Upgrade Image contained within the OTA Upgrade Image according to Section 11.2.5, and update its Event Log with the outcome of that verification.

If the verification is successful, the ESME / GSME /HCALCS shall construct and send a Firmware Distribution Receipt Alert, according to Section 11.2.6, and shall store the Manufacturer Image contained within the OTA Upgrade Image.

If the verification is not successful, the Device shall discard the OTA Upgrade Image, and send a Firmware Distribution Receipt Alert, as detailed in Section 11.2.6.

On receipt of a Firmware Distribution Receipt Alert, the Supplier may verify the cryptographic protection as specified in Section 6.7.3.

Additionally, the Supplier may verify that the Manufacturer Image received by the Device is that intended by comparing the Manufacturer Image Hash in the Firmware Distribution Receipt Alert, with the Hash which it calculates over the Manufacturer Image provided.

  - CS06 Activate Firmware

This Use Case covers the activation of a Firmware Image for ESME, GSME, CH and HCALCS.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command, Response and Alert (if future dated) |
| Message Type Category | SME.C.C |
| Capable of future dated invocation? | Yes |
| Protection Against Replay Required? | No |
| Service Reference | N/A for Communications Hub (this Command does not have a Service Reference for CH) 11.3 for ESME, GSME and HCALCS |
| Valid Target Device(s) | ESME / GSME / CH / HCALCS |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier for ESME / GSME / HCALCS WAN Provider for CH |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | ASN.1 |

Table 11.5:  Use Case Cross References for CS06 Activate Firmware

    - Pre-conditions

None.

    - Detailed Steps

      - Construction of Command

Activate Firmware Command Payloads shall be constructed according to the requirements of Section 11.5.2.3 and populated as specified in Table 11.5.2.1.

MAC Header, Grouping Header, KRP Signature and ACB-SMD MAC shall be populated as required for a Command of the SME.C.C Message Category.

| Attribute name | Data Type | Value (blank cells mean the command specific value is derived by the encoding process) | Mandatory, OPTIONAL or DEFAULT value | Notes |
| --- | --- | --- | --- | --- |
| @ActivateFirmware.CommandPayload | SEQUENCE |  |  |  |
| manufacturerImageHash | OCTET STRING | The Manufacturer Image Hash of the image to be activated. | Mandatory | An octet-string of length 32 interpreted as the Manufacturer Image Hash of the Manufacturer Image that is to be activated |
| originatorCounter | INTEGER (0.. 18446744073709551615) | The Originator Counter as in the Grouping Header of the Command | Mandatory |  |
| executionDateTime | GeneralizedTime | The date-time at which the Command is to be executed, if future dated | OPTIONAL |  |

Table 11.5.2.1: @ActivateFirmware.CommandPayload population

      - Device processing of Command and Response handling

The Device receiving an Activate Firmware Command shall undertake processing steps in the sequence defined in this Section 11.5.2.2.

The Device shall:

- undertake Command Authenticity and Integrity Verification as required for a Command of the SME.C.C Message Category;

- if executionDateTime is present then the Device shall:

      - record manufacturerImageHash, originatorCounter and executionDateTime;

      - construct and send a Response where executionOutcome is not present.  Grouping Header is constructed and Response Cryptographic Protection is applied as required for a Response of the SME.C.C Message Categories; and

      - at the date-time specified in executionDateTime, undertake the processing from step 3.

If executionDateTime is not present then the Device shall continue processing from step 3 immediately;

- if the Device does not have a stored Manufacturer Image then set activateImageResponseCode to noImageHeld and process from step 7;

- calculate Manufacturer Image Hash.  If the calculated value does not equal manufacturerImageHash then the Device shall set activateImageResponseCode to hashMismatch and process from step 7;

- attempt to activate Manufacturer Image.  If the activate fails then the Device shall set activateImageResponseCode to activationFailure and process from step 7;

- set activateImageResponseCode to success;

- populate the executionOutcome according to the requirements of Section 11.5.2.3 using the activateImageResponseCode value produced by the processing in this Section 11.5.2.2, the value of originatorCounter from the Command and the version of firmware now in operation to populate firmwareVersion;

- construct Grouping Header and apply the Response Cryptographic Protection required for a Response / Alert of the SME.C.C / SME.A.C Message Categories respectively.  In such an Alert, the Message Code shall be 0x00CA.  The Response / Alert shall be addressed to the Business Originator of the Corresponding Command.  If activateImageResponseCode is success then alertCode shall be 0x8F66 else alertCode shall be 0x8F67; and

- send the Response if executionDateTime was not present in the Command or send the Alert if executionDateTime was present in the Command.

On receipt of the Response, the recipient may undertake the ‘Response Recipient Verification’ for Responses of type SME.C.C. or for Alerts of type SME.A.C, dependent upon the Message received.

      - Activate Firmware  Command, Response and Alert Payloads – structure definition

Each instance of @ActivateFirmware.CommandPayload and of @ActivateFirmware.ResponsePayload and of @ActivateFirmware.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.5.2.3 which specifies the structure in ASN.1 notation.

ActivateFirmware DEFINITIONS ::= BEGIN

CommandPayload ::= 	SEQUENCE

{

-- specify the hash of the Manufacturer Image to be activated

manufacturerImageHash		OCTET STRING,

-- the Originator Counter as in the Grouping Header of the Command

originatorCounter	   INTEGER (0.. 18446744073709551615),

-- the date-time at which the Command is to execute, if future dated

executionDateTime	   GeneralizedTime OPTIONAL

}

ResponsePayload ::= 	CHOICE

{

-- if the Command is future dated, the Response will not have any details of

-- execution (those will be in the subsequent alert)

commandAccepted		NULL,

-- if the Command is for immediate execution, the Response will detail the

-- outcomes

executionOutcome		ExecutionOutcome

}

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode		INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- the Originator Counter as in the Grouping Header of the corresponding Command

originatorCounter		INTEGER (0.. 18446744073709551615),

-- detail what happened when the future dated command was executed

executionOutcome		ExecutionOutcome

}

ExecutionOutcome ::= 	SEQUENCE

{

-- Specify whether the activation was successful or not

activateImageResponseCode		ActivateImageResponseCode,

-- Specify the Device’s now current firmware version. The value shall be four octets in length and shall correspond to the File Version field in the ZSE OTA Header structure.

firmwareVersion		OCTET STRING

}

ActivateImageResponseCode::= INTEGER

{

success		(0),

noImageHeld		(1),

hashMismatch		(2),

activationFailure		(3)

}

END

  - CS05c Distribute Firmware to PPMID

This Use Case covers the distribution of an OTA Upgrade Image that is intended for a PPMID to the PPMID.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.4 |
| Valid Target Device(s) | PPMID |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific to Communications Hub; ZigBee OTA from Communications Hub to PPMID |

Table 11.6: Use Case Cross References for CS05c Distribute Firmware to PPMID

    - Pre-conditions

None.

    - Detailed Steps

Italicised terms in this Section 11.6.2 shall have the meaning specified in ZSE.

The OTA Upgrade Image shall be populated according to Section 11.2.3.

The OTA Upgrade Image shall be transported to the Communications Hub through which the Device communicates.

- The Communications Hub shall extract the Manufacturer Image from the Upgrade Image and update its OTA Cluster to reflect the availability of the OTA upgrade image;

- the Communications Hub, as OTA Server, shall indicate availability of an OTA upgrade image for PPMID by sending a ZSE Image Notify command;

- the PPMID should include its Hardware Version in the Query Next Image Request Command;

- the Communications Hub shall verify whether the PPMID Hardware Version is present in the Query Next Image Request Command;

- if the Hardware Version is present and does not match the minimum and maximum Hardware Version in the OTA Header the Communications Hub shall set  transferResponseCode  to hardewareVersionMismatch and process from step 7;

- if the Hardware Version is not present, or is present and matches the minimum and maximum Hardware Version in the OTA Header the Communications Hub shall make the OTA Upgrade Image available to the PPMID in the Query Next Image Response Command;

- the PPMID shall download an OTA upgrade image when it is aware of the availability of a suitable OTA upgrade image using the QueryNextImage and Image Block/Page commands specified in the OTA Cluster specification;

- upon the reception of the Upgrade End Request Command from the PPMID the Communications Hub shall

- set transferResponseCode to fileTransferSuccess if the transfer of the Manufacturer Image has completed successfully and process from step 6;

- set transferResponseCode  to fileTransferFailure if the transfer of the Manufacturer Image has failed and all transfer retries have been exhausted, and process from step 7;

- the Communications Hub shall instruct the PPMID to activate the firmware Image as defined in section 11.2.1 and create an Alert with the Alert Code field set to 0x8F8A as specified in Section 11.7, set firmwareVersion to the OTA Upgrade Image File version, include the transferResponseCode from the previous step and process from step 8;

- the Communications Hub shall create an Alert with the Alert Code field set to 0x8F89 as specified in Section 11.7, set firmwareVersion to the OTA Upgrade Image File version, include the transferResponseCode from the previous step and process from step 8;

- the Communications Hub shall send the Alert created in the previous step;

- upon receipt of the firmware activation instruction from the Communications Hub the PPMID shall activate the firmware and send an Alert with Alert Code 0x8F8B as defined in section 11.8.4 to this effect.

#### CCS08 Firmware Transfer Alert

### Description

This Section 11.7 covers the creation and processing of Alerts resulting from the requirements contained in Section 11.4.2 and Section 11.6.2.

### Use Case Cross References

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Alert |
| Message Type Category | SME.A.C |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | N/A |
| Valid Initiating Device(s) | CHF |
| Valid Business Target role(s) for Alert | Access Control Broker |
| Valid Response Recipient role(s) (only for Messages Authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | ASN.1 |

Table 11.6.3.2:  Use Case Cross References for CCS08 Firmware Transfer Alert

### Construction of Alerts

Grouping Header and SMD Signature shall be populated as required for an Alert of the SME.A.C Message Category. The Message Code shall be 0x0128.

The Business Originator ID field in the Grouping Header shall contain the CHF’s Entity Identifier.

The Business Target ID field in the Grouping Header shall contain the Entity Identifier in the CHF’s {accessControlBroker, digitalSignature, management} Trust Anchor Cell.

The Alert Payload shall be constructed according to the requirements of Section 11.7.5 and the Alerts shall be populated as specified in Table 11.7.4.

| Attribute name |  |  | Data Type |  |  |  | Value (blank cells mean the command specific value is derived by the encoding process) |  |  |  |  | Mandatory, OPTIONAL or DEFAULT value |  |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| @FirmwareTransferAlert.AlertPayload |  |  |  |  | SEQUENCE |  |  |  |  |  |  |  |  |  |  |  |  |
| __alertCode |  |  |  |  | INTEGER |  |  | 0x8F8A 0x8F89 |  |  |  |  | Mandatory |  |  |  | (1) 0x8F8A: OTA Firmware Image Delivery to the target Devices successful; (2) 0x8F89: OTA Firmware Image Delivery to the target Device failed; |
| executionDateTime | GeneralizedTime | The date-time at which this Alert was created | Mandatory |  |
| __otherDeviceEntityIdentifier |  |  |  |  | OCTET STRING |  |  | Entity Identifier |  |  |  |  | Mandatory |  |  |  | The Entity Identifier of the ESME / GSME / HCALCS / PPMID the firmware upgrade is targeted at |
| __firmwareVersion |  | OCTETT STRING |  |  |  | ZSE firmware version |  |  |  |  | Mandatory |  |  |  | The firmware version of the transferred or discarded Upgrade Image in ZSE format |  |  |
| __TransferResponseCode | INTEGER |  |  |  |  | fileTransferSuccess (0), imageDiscarded(1), hardewareVersionMismatch(2), fileTransferFailure (3) |  |  | Mandatory |  |  |  |  | (a) 0x8F8A: fileTransferSuccess (0) (b) 0x8F89: imageDiscarded (1); hardewareVersionMismatch (2); fileTransferFailure (3) |  |  |  |

Table 11.7.3: @FirmwareTransferAlert.AlertPayload population

### Firmware Transfer Alert – structure definition

Each instance of FirmwareTransferAlert.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.7.4 which specifies the structure in ASN.1 notation.

FirmwareTransferAlert  DEFINITIONS ::= BEGIN

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode	INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- specify the 64 bit Entity Identifier of the Device the upgrade has been

-- targeted at

otherDeviceEntityIdentifier 	OCTET STRING,

-- specify the upgrade image version if replaced or discarded by the

-- Communications Hub or if the transfer to the target device succeeded or

-- failed. The value shall be four octets in length and shall correspond to

-- the FileVersion field in the ZSE OTA Header structure

firmwareVersion	OCTET STRING

-- specify the outcome

transferResponseCode	TransferResponseCode

}

TransferResponseCode ::= 	INTEGER

{

fileTransferSuccess	(0),

imageDiscarded	(1),

hardewareVersionMismatch	(2),

fileTransferFailure	(3)

}

END

## CS08 Read PPMID/HCALCS Firmware Version

### Description

This Section 11.8.1 covers the creation and processing of Commands and Responses related to reading the Firmware Version, and the sending of Alerts resulting from the requirements contained in Section 11.6.2. The Message Code shall be 0x0129.

### Use Case Cross References

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command, Response and Alert |
| Message Type Category | SME.C.NC; SME.A.C |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.2 |
| Valid Target Device(s) | PPMID, HCALCS |
| Valid Business Target role(s) for Alert | Access Control Broker |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Access Control Broker, Supplier |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | ASN.1 |

Table 11.6.3.2:  Use Case Cross References for CS08 Read PPMID/HCALCS Firmware Version

### Construction of Command and Response

#### Construction of Commands

‘Read PPMID/HCALCS Firmware Version’ Command Payloads shall be constructed as specified in this Section 11.8.3.1 and Cryptographic Protection shall be applied as required for a Command of the relevant Message Category.

#### Device processing of Commands and Response handling

The Device receiving a ‘Read PPMID/HCALCS Firmware Version’ Command shall undertake processing steps in the sequence defined in this Section 11.8.3.2.

In processing a ‘Read PPMID/HCALCS Firmware Version’ Command, the Device shall:

- undertake Command Authenticity and Integrity Verification as required for a Command of this Message Category.

- set firmwareVersion to the version of firmware in operation; set  firmwareReadCode to firmwareReadSuccess and process from Step 4;

- if the version of firmware in operation is not available, set firmwareVersion to 0x00000000; set firmwareReadCode to firmwareReadFailure and process from Step 4;

- create a Response according to the requirements of Section 11.8.5, apply the Response Cryptographic Protection required for a Response of the relevant Message Category, and send the Response.

### Construction of Alerts

Grouping Header and SMD Signature shall be populated as required for an Alert of the SME.A.C Message Category.

The Business Originator ID field in the Grouping Header shall contain the PPMID’s Entity Identifier.

The Business Target ID field in the Grouping Header shall contain the Entity Identifier in the PPMID’s {accessControlBroker, digitalSignature, management} Trust Anchor Cell.

The Alert Payload shall be constructed according to the requirements of Section 11.8.5 and the Alerts shall be populated as specified in Table 11.8.4.

| Attribute name | Data Type | Value (blank cells mean the command specific value is derived by the encoding process) | Mandatory, OPTIONAL or DEFAULT value | Notes |
| --- | --- | --- | --- | --- |
| @ReadPpmidHcalcsFirmwareVersion.AlertPayload | SEQUENCE |  |  |  |
| __alertCode | INTEGER | 0x8F8B | Mandatory |  |
| executionDateTime | GeneralizedTime | The date-time at which this Alert was created | Mandatory |  |
| __firmwareVersion | OCTETT STRING | ZSE firmware version | Mandatory | The firmware version in operation in ZSE format or 0x00000000 if reading the firmware version failed |
| __ActivateImageResultCode | INTEGER | activationSuccess(0), activationFailure(1) | Mandatory |  |

Table 11.8.4: @ReadPpmidHcalcsFirmwareVersion.AlertPayload population

### Read PPMID/HCALCS Firmware Version Command, Response and Alert Payloads – structure definition

Each instance of @ReadPpmidHcalcsFirmwareVersion.CommandPayload; ReadPpmidHcalcsFirmwareVersion.ResponsePayload and ReadPpmidHcalcsFirmwareVersion.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.8.5 which specifies the structure in ASN.1 notation.

ReadPpmidHcalcsFirmwareVersion  	DEFINITIONS ::= BEGIN

CommandPayload ::= 	NULL

ResponsePayload ::= 	SEQUENCE

{

-- Specify the Device’s now current firmware version. The value shall be four

-- octets in length and shall correspond to the File Version field in the ZSE

-- OTA Header structure. Set to 0x00000000 if reading failed.

firmwareVersion	OCTET STRING

-- specify whether the reading of the firmware was successful.

firmwareReadCode	FirmwareReadCode

}

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode		INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- Specify the Device’s now current firmware version. The value shall be four

-- octets in length and shall correspond to the File Version field in the ZSE

-- OTA Header structure.

firmwareVersion	OCTET STRING

-- Specify whether the activation was successful or not

activateImageResultCode		ActivateImageResultCode

}

ActivateImageResultCode::= INTEGER

{

activationSuccess		(0),

activationFailure		(1)

}

FirmwareReadCode ::= 	INTEGER

{

firmwareReadSuccess	(0),

firmwareReadFailure	(1)

}

END

Appendix_1: New embedded GBCS tables

See section 6.10.3 of  ZigBee Document 09-5264r23

As defined in section  6.10 of ZigBee Document 09-5264r23

As defined in sections 6.10.10 and 6.8.4 of ZigBee Document 09-5264r23

ZigBee Document 09-5264r23