- Downloading firmware images to Devices

  - Introduction – informative

Compared to other Smart Metering messages, firmware images are large.  Further, each image is likely to be applicable to a significant number of Devices.  Thus, an end-to-end, unicast Message to each affected Device, with each message containing a copy of the image, is not efficient from a WAN perspective.

This leads to the firmware update process being separated into two stages for ESME, GSME, HCALCS and Communications Hubs:

- distribution of the image to end Devices without any activation of that image; and

- a separate and subsequent ‘activation’ Command to each Device.

The Distribute Firmware Command is not a Critical Command (since it does not affect the operating firmware) and does not need to be unicast.

The Activate Firmware Command is a Critical Command and so must be unicast – as it must be digitally signed and be for one, and only one specified Device.  Further, the Activate Command must apply to one, and only one, image and that image must have originated from the same party that signs the Activate Firmware Command (that is, the party responsible for that Device).  To meet these requirements:

- the Activate Firmware Command is of type SME.C.C and so the Signature and MAC on the Command shall have been verified by the Device prior to the Hash validation (see next bullet); and

- a Device receiving an Activate Firmware Command shall calculate a Hash over the Manufacturer Image it holds and ensure the Hash so calculated matches that in the Activate Firmware Command, before the Device attempts to activate the corresponding Manufacturer Image.

For IHD and PPMID the firmware image is delivered to the Communications Hub and transferred to the target Device. Upon successful transfer of the firmware image the Communications Hub instructs the PPMID / IHD when to activate the firmware image.

The GBCS does not constrain the mechanisms used by Device manufacturers to ensure that only valid Manufacturer Images are activated on Devices manufactured by them.  The GBCS does require that the manufacturer information related to a Manufacturer Image is made available, so that the Upgrade Image and the ZigBee Over-The-Air (OTA) Header can be provided when requesting distribution of an image.

In common with other Messages, the GBCS shall not constrain the mechanisms by which the firmware Messages are transported to the Communications Hub.  The GBCS constrains HAN transport mechanisms to those provided by ZSE.

In line with the ZigBee OTA specification at section 5.1, the contents of Manufacturer Images sent to Devices are manufacturer defined.  Thus, a particular Manufacturer Image may consist of whatever the manufacturer requires to achieve the necessary update which could be a full image or just a patch to application code or any other manufacturer specified content.

Therefore the steps taken by a Device when it activates the contents of a particular Manufacturer Image are manufacturer specific and specified in the release note for that Manufacturer Image. Thus, activation of a Manufacturer Image means the set of manufacturer specified steps the Device takes in processing the content of the image and terms such as ‘activation’, ‘activate’, ‘activated’, ‘installed’ etc in the GBCS, SMETS and CHTS are meant in this sense.

Therefore, when a Device reports its ‘installed’ or ‘active’ firmware version it shall respond with either:

- the firmware version for its mostly recently successfully activated Manufacturer Image (with the value in the corresponding OTA Header and so the value in the Certified Products List (CPL)); or

- if it has never successfully activated a new Manufacturer Image, the firmware version it held on installation (which again would correspond to the CPL).

## Common Requirements

### Transport of firmware images

Italicised terms in this Section 11.2.1 shall have the meanings defined in ZigBee Document 09-5264r23.

For ESME, GSME, HCALCS, PPMID and IHD firmware image distribution, the ZigBee Over-The-Air (OTA) mechanisms shall be used for transport of the image over the HAN.  The ESME / GSME / HCALCS / PPMID / IHD firmware image delivered to the Communications Hub shall comply with ZigBee OTA format requirements.

Communications Hub firmware images shall not be transported over the HAN and so ZigBee OTA structures shall not be required.

Every Communications Hub shall be configured to act as the single OTA Server on its HAN.

ESME, GSME, HCALCS, PPMID and IHD shall be configured to act as an OTA Client.  The ESME, HCALCS, PPMID and IHD shall use the ‘Image Notify’ Command sent by the OTA Server to inform it that a new firmware image is available.  The GSME shall use the notification flags mechanism whereby a flag shall be set by the OTA Server to inform it that a new firmware image is available when requested.

The Communications Hub shall be able to record the target Device Entity Identifier and the Upgrade Image File version contained in the OTA Header as detailed in Section 11.2.3 for fifteen Devices in the Upgrade Image List; if the target Device is a PPMID or IHD the Communications Hub shall also be able to recored the activation date-time contained in the Upgrade Image as detailed in Section 11.2.2.2.

The Communications Hub shall:

- as required by CHTS, have the capability to store one GSME OTA Upgrade Image and one ESME OTA Upgrade Image; and

- overwrite an ESME / GSME image with a subsequently delivered image for the same Device type unless:

      - the subsequently delivered image has Force Replace = 0x00; and

      - the Communications Hub has sent at least one Image Block Response Command relating to the already stored image but has not received a corresponding Upgrade End Request Command.

In such circumstances the Communications Hub shall not overwrite the currently stored image.

The Communications Hub shall have the capability to store up to two Upgrade Images for HCALCS / PPMID / IHD provided the storage space is not used by GSME or ESME OTA Upgrade Images. The storing of the GSME OTA Upgrade Image and the ESME OTA Upgrade Image shall take precedence:

- The Communiations Hub shall only store an HCALCS / PPMID / IHD Upgrade Image at times when the storage space is available and is not holding an ESME or GSME OTA Upgrade Image;

- The Communications Hub shall replace an HCALCS / PPMID / IHD Upgrade Image with an ESME or GSME OTA Upgrade image in case of insufficient storage capability;

- The Communications shall replace an HCALCS / PPMID IHD Upgrade Image with a new Upgrade Image for the same Device unless the Communications Hub has sent at least one Image Block Response Command relating to the already stored image but has not received a corresponding Upgrade End Request Command;

- If a PPMID / IHD / HCALS Upgrade Image has been replaced, the Communications Hub shall send an Alert for each target Device Entity Identifier associated with the Upgrade Image File version on the the Upgrade Image List with the Alert Code 0x8F89 as specified in Section 11.7 by setting firmwareVersion to the Upgrade Image File version and imageResponseCode  to imageDiscarded; and remove the corresponding entries from the Upgrade Image List.

Whenever the Communications Hub's OTA Server issues an Upgrade End Response Command to a GSME, ESME or HCALCS pursuant to this GBCS, the UpgradeTime parameter shall have the value 0xFFFFFFFF.

The OTA Server shall not issue Image Block Response Commands with WAIT_FOR_DATA status except where the OTA Server is communicating over a Sub GHz Channel and it is issuing such commands to manage Duty Cycle (with its ZSE meaning).

Contrary to section 6.13 of ZigBee Document 09-5264r23, the OTA Client shall not activate any Firmware for an ESME, GSME or HCALCS and except as specified in Use Case CS06.

For the distribution and activation of PPMID / IHD Upgrade Images see Section 11.6.

### Construction of Upgrade Image

#### Upgrade Image for ESME, GSME and HCALCS

For an ESME, GSME or HCALCS firmware image, the Authorising Remote Party shall be the Supplier for the target Device.

For a Communications Hub firmware image, the Authorising Remote Party shall be the WAN Provider for the target Device.

Upgrade Image shall be the concatenation:

Manufacturer Image || Force Replace || 0x40 || Authorising Remote Party Signature

where:

- Manufacturer Image shall contain the firmware image the Device is to apply and any manufacturer specific data needed.  For clarity, the GBCS shall not constrain the structure or contents of Manufacturer Image;

- Force Replace shall be a single octet where Force Replace = 0x00 shall mean do not force the replacement of the currently stored image; and

- Authorising Remote Party Signature shall be calculated across the Manufacturer Image using the Authorising Remote Party’s Private Digital Signing Key.

#### Upgrade Image for PPMID and IHD

For a PPMID or IHD firmware image, the Upgrade Image shall be the concatenation:

Manufacturer Image || activation date-time

where:

- Manufacturer Image shall contain the firmware image the Device is to apply and any manufacturer specific data needed.  For clarity, the GBCS shall not constrain the structure or contents of Manufacturer Image; and

- activation date-time shall be the UTC Time when the firmware image is to be activated and shall use the format defined by the ZigBee specifications; in line with the ZigBee specifications the activation date-time shall be set to 0x00000000 for immediate activation.

    - Construction of OTA Upgrade Image

OTA Upgrade Image shall be the concatenation:

OTA Header || Upgrade Image

where OTA Header shall be populated according to Table 11.2.3.  For clarity, there shall be no other sub-elements present.

| OTA Header |  |  |  |
| --- | --- | --- | --- |
| ZigBee OTA Message Element | Contents | Length (octets) | Note |
| OTA upgrade file identifier | 0x0BEEF11E | 4 | Fixed by ZigBee OTA specification |
| OTA Header version | 0x0100 | 2 | Specified by current version of ZigBee OTA specification |
| OTA Header length | 0x003C | 2 | The length of ZigBee OTA Header which is decimal 60 |
| OTA Header Field control | 0x0004 | 2 | Detailing what is / is not present in ZigBee OTA Header |
| Manufacturer code | ZSE assigned identifier for the Manufacturer of the target Device | 2 | So this identifies the manufacturer producing the Manufacturer Image |
| Image type | Manufacturer specific | 2 | As per the ZigBee OTA specification, this is to differentiate products from the same manufacturer |
| File version | Manufacturer specific | 4 | As per the ZigBee OTA specification, this is to differentiate release and build numbers for the product in question |
| ZigBee Stack version | 0x0002 | 2 | ZigBee PRO |
| OTA Header string | Manufacturer specific | 32 | May be blank but is not required to be used in Device processing of the firmware image |
| Total Image size (including header) | The length in octets of OTA Upgrade Image | 4 | Contents to be interpreted as an unsigned integer |
| Minimum hardware version | Manufacturer specific | 2 |  |
| Maximum hardware version | Manufacturer specific | 2 |  |

Table 11.2.3:  Population of the OTA Header

The OTA Header shall uniquely identify a firmware image.

    - Construction of Manufacturer Image Hash

Manufacturer Image Hash shall be a Hash calculated across the whole Manufacturer Image file that is provided to the Authorising Remote Party.

    - Verification of the authenticity of the Upgrade Image

The Device shall verify Upgrade Image by verifying the Authorising Remote Party Signature using Manufacturer Image and the Authorising Remote Party’s Public Key.  For clarity, this shall be the only ECDSA verification required by the GBCS and this is not the ZSE ECDSA Signature sub-element.

For an ESME, GSME or HCALCS receiving an Upgrade Image, the Authorising Remote Party’s Public Key shall be that held by the Device in the {supplier, digitalSignature, management} Trust Anchor Cell.

For a Communications Hub receiving an Upgrade Image the Authorising Remote Party’s Public Key shall be that held by the Device in the {wanProvider, digitalSignature, management} Trust Anchor Cell.

See Section 11.6 for the handling of Upgrade Images for the PPMID / IHD.

    - Construction of Firmware Distribution Receipt Alert

If the Device is an ESME or HCALCS, the ‘Alert Payload’ fields shall be populated according to Section 7.2.9.

If the Device is a GSME, the ‘Alert Payload’ fields shall be populated according to Section 7.2.10.

In both cases, the Device shall:

- populate the Use Case Specific Additional Content with the concatenation

0x0920 || the calculated Manufacturer Image Hash

- populate the Alert Code field with 0x8F1C (failure), or 0x8F72 (success).

- See Section 11.6 for the handling of Upgrade Images for the PPMID / IHD.

    - Activation of firmware images

The Activate Firmware Command shall be of type SME.C.C.

A Device receiving such a Command shall undertake the verifications required of a SME.C.C Command.

If all such SME.C.C verifications succeed, the Device shall then calculate Manufacturer Image Hash over the Manufacturer Image it holds and compare that with the Manufacturer Image Hash specified in the Activate Firmware Command (see Use Case CS06 in Section 11.5 for details of the Activate Firmware Command Payload construction).

If the two Hashes match, the Device shall attempt to activate the firmware image.

If the two Hashes do not match, the Device shall not attempt to activate the firmware image.

The Device shall issue a relevant Activate Firmware Response detailing the success or failure (see Use Case CS06 in Section 11.5 for details of the Activate Firmware Command Payload construction).

See Section 11.6 for the handling of Upgrade Images for the PPMID / IHD.

  - CS05a Distribute Firmware to Communications Hub

This Use Case covers the distribution of an Upgrade Image that is intended for a Communications Hub to that Communications Hub.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command and Response |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | N/A (this Command does not have a Service Reference for CH) |
| Valid Target Device(s) | Communications Hub |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | WAN Provider |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific |

Table 11.3:  Use Case Cross References for CS05a Distribute Firmware to Communications Hub

    - Pre-conditions

None.

    - Detailed Steps

The Upgrade Image shall be constructed according to Section 11.2.2.

The Upgrade Image shall be transported to the Communications Hub.

The Communications Hub shall verify the Upgrade Image according to Section 11.2.5, verify the Upgrade Image is suitable for this Communications Hub, and discard the Upgrade Image where it fails to verify.

  - CS05b Distribute Firmware to ESME / GSME

This Use Case covers the distribution of an OTA Upgrade Image that is intended for a GSME or ESME to that GSME or ESME.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command and Response |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.1 |
| Valid Target Device(s) | ESME / GSME / HCALCS |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific to Communications Hub; ZigBee OTA from Communications Hub to ESME / GSME / HCALCS |

Table 11.4:  Use Case Cross References for CS05b Distribute Firmware to ESME / GSME / HCALCS

    - Pre-conditions

None.

    - Detailed Steps

Italicised terms in this Section 11.4.2 shall have the meaning specified in ZSE.

ESME, GSME and HCALCS shall use the ZCL Image Block Request and Image Block Response commands to retrieve available OTA images.

ESME, GSME and HCALCS shall not use the ZCL Query Specific File Request and Query Specific File Response commands.

The OTA Upgrade Image shall be populated according to Section 11.2.3.

The OTA Upgrade Image shall be transported to the Communications Hub through which the Device communicates.

The Communications Hub shall update its OTA Cluster to reflect availability of the OTA Upgrade Image, once the image is received by the Communications Hub.

The Communications Hub, as OTA Server, shall indicate availability of an OTA Upgrade Image differently for ESME, HCALCS and GSME:

- for ESME and HCALS, the Communications Hub shall send a ZSE Image Notify command; and

- for GSME, the Communications Hub shall set a the New OTA Firmware flag (Bit Number 0) in FunctionalNotificationFlags.

- Upon reception of the ESME / GSME / HCACLS OTA Upgrade Image the Communications Hub shall record the target Device Entity Identifier(s) and the OTA Upgrade Image File version in the Upgrade Image List.

- For each ESME / GSME / HCALCS target Device Entity Identifier associated with the OTA Upgrade Image version in the Upgrade Image List the Communications Hub shall verify whether the ESME / GSME / HCALCS Hardware Version is present in the Query Next Image Request Command;

- if the Hardware Version is present and does not match the minimum and maximum Hardware Version in the OTA Header the Communcations Hub shall set  imageResponseCode  to hardewareVersionMismatch as specified in Section 11.7 and process from step 4;

- if the Hardware Version is present and matches the minimum and maximum Hardware Version in the OTA Header the Communcations Hub shall make the OTA Upgrade Image available to the ESME / GSME / HCALCS in the Query Next Image Response Command;

- if the Hardware Version is not present the Communcations Hub shall make the OTA Upgrade Image available to the ESME / GSME / HCALCS in the Query Next Image Response Command;

- the ESME / GSME / HCALCS shall download an OTA Upgrade Image when it is aware of the availability of a suitable OTA Upgrade Image using the QueryNextImage and Image Block/Page commands specified in the OTA Cluster specification4;

- upon the reception of the Upgrade End Request Command from the ESME / GSME / HCALCS the Communications Hub shall send an Alert with Alert Code 0x8F8A as specified in Section 11.7 with firmwareVersion set to the Upgrade Image File version;

- if the transfer of the Manufacturer Image has completed the Communications Hub shall set imageResponseCode to fileTransferSuccess;

- if the transfer of the Manufacturer Image has failed the Communications Hub shall set imageResponseCode  to fileTransferFailure; and

- the Communications Hub shall mark the corresponding entry in the Upgrade Image List with the outcome and process from step 5;

- the Communications Hub shall send an Alert with the Alert Code 0x8F89 as specified in Section 11.7 with firmwareVersion set to the OTA Upgrade Image File version and include the imageResponseCode from the previous step. The Communications Hub shall mark the corresponding entry in the Upgrade Image List with the outcome.

- once step 3 or step 4 have been executed for all Device Entity Identifiers associated with the OTA Upgrade Image File version in the Upgrade Image List, the Communications Hub shall discard the ESME / GSME / HCALCS OTA Upgrade Image and remove the associated entries in the Upgrade Image List.

The ESME / GSME / HCALCS shall verify the Upgrade Image contained within the OTA Upgrade Image according to Section 11.2.5, and update its Event Log with the outcome of that verification.

If the verification is successful, the ESME / GSME shall construct and send a Firmware Distribution Receipt Alert, according to Section 11.2.6, and shall store the Manufacturer Image contained within the OTA Upgrade Image.

If the verification is not successful, the Device shall discard the OTA Upgrade Image, and send a Firmware Distribution Receipt Alert, as detailed in Section 11.2.6.

On receipt of a Firmware Distribution Receipt Alert, the Supplier may verify the cryptographic protection as specified in Section 6.7.3.

Additionally, the Supplier may verify that the Manufacturer Image received by the Device is that intended by comparing the Manufacturer Image Hash in the Firmware Distribution Receipt Alert, with the Hash which it calculates over the Manufacturer Image provided.

  - CS06 Activate Firmware

This Use Case covers the activation of a Firmware Image.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command, Response and Alert (if future dated) |
| Message Type Category | SME.C.C |
| Capable of future dated invocation? | Yes |
| Protection Against Replay Required? | No |
| Service Reference | N/A for Communications Hub (this Command does not have a Service Reference for CH) 11.3 for ESME, GSME and HCALCS |
| Valid Target Device(s) | ESME / GSME / CH / HCALCS |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier for ESME / GSME / HCALCS WAN Provider for CH |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | ASN.1 |

Table 11.5:  Use Case Cross References for CS06 Activate Firmware

    - Pre-conditions

None.

    - Detailed Steps

      - Construction of Command

Activate Firmware Command Payloads shall be constructed according to the requirements of Section 11.5.2.3 and populated as specified in Table 11.5.2.1.

MAC Header, Grouping Header, KRP Signature and ACB-SMD MAC shall be populated as required for a Command of the SME.C.C Message Category.

| Attribute name | Data Type | Value (blank cells mean the command specific value is derived by the encoding process) | Mandatory, OPTIONAL or DEFAULT value | Notes |
| --- | --- | --- | --- | --- |
| @ActivateFirmware.CommandPayload | SEQUENCE |  |  |  |
| manufacturerImageHash | OCTET STRING | The Manufacturer Image Hash of the image to be activated. | Mandatory | An octet-string of length 32 interpreted as the Manufacturer Image Hash of the Manufacturer Image that is to be activated |
| originatorCounter | INTEGER (0.. 18446744073709551615) | The Originator Counter as in the Grouping Header of the Command | Mandatory |  |
| executionDateTime | GeneralizedTime | The date-time at which the Command is to be executed, if future dated | OPTIONAL |  |

Table 11.5.2.1: @ActivateFirmware.CommandPayload population

      - Device processing of Command and Response handling

The Device receiving an Activate Firmware Command shall undertake processing steps in the sequence defined in this Section 11.5.2.2.

The Device shall:

- undertake Command Authenticity and Integrity Verification as required for a Command of the SME.C.C Message Category;

- if executionDateTime is present then the Device shall:

      - record manufacturerImageHash, originatorCounter and executionDateTime;

      - construct and send a Response where executionOutcome is not present.  Grouping Header is constructed and Response Cryptographic Protection is applied as required for a Response of the SME.C.C Message Categories; and

      - at the date-time specified in executionDateTime, undertake the processing from step 3.

If executionDateTime is not present then the Device shall continue processing from step 3 immediately;

- if the Device does not have a stored Manufacturer Image then set activateImageResponseCode to noImageHeld and process from step 7;

- calculate Manufacturer Image Hash.  If the calculated value does not equal manufacturerImageHash then the Device shall set activateImageResponseCode to hashMismatch and process from step 7;

- attempt to activate Manufacturer Image.  If the activate fails then the Device shall set activateImageResponseCode to activationFailure and process from step 7;

- set activateImageResponseCode to success;

- populate the executionOutcome according to the requirements of Section 11.5.2.3 using the activateImageResponseCode value produced by the processing in this Section 11.5.2.2, the value of originatorCounter from the Command and the version of firmware now in operation to populate firmwareVersion;

- construct Grouping Header and apply the Response Cryptographic Protection required for a Response / Alert of the SME.C.C / SME.A.C Message Categories respectively.  In such an Alert, the Message Code shall be 0x00CA.  The Response / Alert shall be addressed to the Business Originator of the Corresponding Command.  If activateImageResponseCode is success then alertCode shall be 0x8F66 else alertCode shall be 0x8F67; and

- send the Response if executionDateTime was not present in the Command or send the Alert if executionDateTime was present in the Command.

On receipt of the Response, the recipient may undertake the ‘Response Recipient Verification’ for Responses of type SME.C.C. or for Alerts of type SME.A.C, dependent upon the Message received.

      - Activate Firmware  Command, Response and Alert Payloads – structure definition

Each instance of @ActivateFirmware.CommandPayload and of @ActivateFirmware.ResponsePayload and of @ActivateFirmware.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.5.2.3 which specifies the structure in ASN.1 notation.

ActivateFirmware DEFINITIONS ::= BEGIN

CommandPayload ::= 	SEQUENCE

{

-- specify the hash of the Manufacturer Image to be activated

manufacturerImageHash		OCTET STRING,

-- the Originator Counter as in the Grouping Header of the Command

originatorCounter	   INTEGER (0.. 18446744073709551615),

-- the date-time at which the Command is to execute, if future dated

executionDateTime	   GeneralizedTime OPTIONAL

}

ResponsePayload ::= 	CHOICE

{

-- if the Command is future dated, the Response will not have any details of

-- execution (those will be in the subsequent alert)

commandAccepted		NULL,

-- if the Command is for immediate execution, the Response will detail the

-- outcomes

executionOutcome		ExecutionOutcome

}

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode		INTEGER(0..4294967295),

-- specify the date-time of execution

executionDateTime		GeneralizedTime,

-- the Originator Counter as in the Grouping Header of the corresponding Command

originatorCounter		INTEGER (0.. 18446744073709551615),

-- detail what happened when the future dated command was executed

executionOutcome		ExecutionOutcome

}

ExecutionOutcome ::= 	SEQUENCE

{

-- Specify whether the activation was successful or not

activateImageResponseCode		ActivateImageResponseCode,

-- Specify the Device’s now current firmware version. The value shall be four octets in length and shall correspond to the File Version field in the ZSE OTA Header structure.

firmwareVersion		OCTET STRING

}

ActivateImageResponseCode::= INTEGER

{

success		(0),

noImageHeld		(1),

hashMismatch		(2),

activationFailure		(3)

}

END

  - CS05c Distribute Firmware to PPMID or IHD

This Use Case covers the distribution of an OTA Upgrade Image that is intended for a PPMID or IHD to the PPMID or IHD via a Communications Hub.

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command |
| Message Type Category | None – this is a Variant Message |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.1 |
| Valid Target Device(s) | PPMID / IHD |
| Valid Business Originator role(s) for Command invocation (and so, for DLMS COSEM Commands, which Application Association is to be used for delivery of the Command to the Device) [Remote Party Messages Only] | Supplier |
| Valid Response Recipient role(s) (only for Messages authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | WAN Provider Specific to Communications Hub; ZigBee OTA from Communications Hub to PPMID/IHD |

Table 11.6: Use Case Cross References for CS05c Distribute Firmware to PPMID / IHD

    - Pre-conditions

None.

    - Detailed Steps

Italicised terms in this Section 11.6.2 shall have the meaning specified in ZSE.

The OTA Upgrade Image shall be populated according to Section 11.2.3.

The OTA Upgrade Image shall be transported to the Communications Hub through which the Device communicates.

- Upon reception of the Upgrade Image the Communications Hub shall record the target Device Entity Identifier(s), the Upgrade Image File version and the activation date-time in the Upgrade Image List;

- the Communications Hub shall extract the Manufacturer Image from the Upgrade Image and update its OTA Cluster to reflect the availability of the OTA upgrade image;

- the Communications Hub, as OTA Server, shall indicate availability of an OTA upgrade image for PPMID and IHD by sending a ZSE Image Notify command;

- the PPMID / IHD should include its Hardware Version in the Query Next Image Request Command;

- for each target Device Entity Identifier associated with the Upgrade Image File version in the Upgrade Image List the Communications Hub shall verify whether the PPMID / IHD Hardware Version is present in the Query Next Image Request Command;

if the Hardware Version is present and does not match the minimum and maximum Hardware Version in the OTA Header the Communcations Hub shall set  imageResponseCode  to hardewareVersionMismatch and process from step 8;

if the Hardware Version is present and matches the minimum and maximum Hardware Version in the OTA Header the Communcations Hub shall make the OTA upgrade image available to the PPMID / IHD in the Query Next Image Response Command;

if the Hardware Version is not present the Communcations Hub shall make the OTA upgrade image available to the PPMID / IHD in the Query Next Image Response Command;

- the PPMID / IHD shall download an OTA upgrade image when it is aware of the availability of a suitable OTA upgrade image using the QueryNextImage and Image Block/Page commands specified in the OTA Cluster specification;

- upon the reception of the Upgrade End Request Command from the PPMID / IHD indicating success the Communications Hub shall send an Alert with Alert Code 0x8F8A as specified in Section 11.7 with firmwareVersion set to the Upgrade Image File version and imageResponseCode  set to fileTransferSuccess; and process from step 9;

if the transfer of the Manufacturer Image has failed the Communications Hub shall set imageResponseCode  to fileTransferFailure and process from step 8;

- the Communications Hub shall send an Alert with the Alert Code 0x8F89 as specified in Section 11.7 with firmwareVersion set to the Upgrade Image File version and include the imageResponseCode from the previous step and marke the corresponding entry in the Upgrade Image List with the outcome.

- The Communications Hub shall verify the activation data-time associated with the Upgrade Image, where activation date-time is set to immediat activation or is in the past the Communications Hub shall record the current time in the Upgrade Image List and mark the corresponding entry in the Upgrade Image List with the outcome.

- Whenever the Communications Hub's OTA Server issues an Upgrade End Response Command to a PPMID or IHD pursuant to this GBCS, the UpgradeTime parameter shall be set to the activation date-time associated with the Upgrade Image in the Upgrade Image List.

- Once step 8 or step 9 have been executed for all Device Entity Identifiers in the Upgrade Image List associated with the Upgrade Image File version, the Communications Hub shall discard the PPMID / IHD OTA upgrade image.

- the PPMID / IHD OTA client shall activate the received firmware image according to the Upgrade End Response Command received from the Communications Hub;

- for each PPMID / IHD with an activation date-time recorded in the Upgrade Image Liste the Communications Hub shall wait until 10 minutes after the activation date-time recorded in the Upgrade Image List and shall then request the CurrentFileVersion Attribute from each PPMID / IHD;

- upon succesfull reception the Communications Hub shall set fileOrFirmwareVersion to the CurrentFileVersion of the target Device and imageResponseCode  to firmwareReadSuccess;

otherwise the Communications Hub shall set fileOrFirmwareVersion to noFirmwareVersion and shall set imageResponseCode to firmwareReadFailure; and

send an Alert with the Alert Code 0x8F8B as specified in Section 11.7;

- remove all entries in the Upgrade Image List associated with the OTA upgrade image.

#### CCS08 Firmware Upgrade Results – PPMID and IHD

### Description

This Section 11.7 covers the creation and processing of Commands, Responses and Alerts resulting from the requirements contained in Section 11.4.2 and Section 11.6.2.

### Use Case Cross References

| Cross Reference | Value |
| --- | --- |
| Grouping | Remote Party Message |
| Message Type | Command, Response and Alert |
| Message Type Category | SME.C.NC; SEM.A.C |
| Capable of future dated invocation? | No |
| Protection Against Replay Required? | No |
| Service Reference | 11.2 |
| Valid Initiating Device(s) | CHF |
| Valid Business Target role(s) for Alert | Access Control Broker |
| Valid Response Recipient role(s) (only for Messages Authorised by the Access Control Broker on behalf of parties not known to the Device) [Remote Party Messages Only] | N/A |
| Valid initiating Device type(s) [HAN Only Messages] | N/A |
| Protocol | ASN.1 |

Table 11.6.3.2:  Use Case Cross References for CCS08 Firmware Upgrade Results – PPMID and IHD

### Construction of Command and Response

### Construction of Commands

‘Firmware Upgrade Result’ Command Payloads shall be constructed as specified in Section 11.7.5 and Cryptographic Protection I and Cryptographic Protection II shall be applied as required for a Command of the relevant Message Category. The Message Code shall be 0x0128.

The Access Control Broker’s Digital Signing Private Key shall be used in generating the KRP Signature.

#### Device processing of Commands and Response handling

The Communications Hub receiving a ‘Firmware Upgrade Result’ Command shall undertake processing steps in the sequence defined in this Section 11.7.3.2.

In processing a ‘Firmware Upgrade Result’ Command, the Communications Hub shall:

- undertake Command Authenticity and Integrity Verification as required for a Command of this Message Category. The Security Credentials used to verify Cryptographic Protection 1 shall be those held in the {accessControlBroker, digitalSignature, management} Trust Anchor Cell;

- request the CurrentFileVersion Attribute from the PPMID / IHD; upon succesfull reception set firmwareVersion to the CurrentFileVersion of the target Device; set imageResponseCode to firmwareReadSuccess and process from Step 5;

- if requesting the CurrentFileVersion Attribute from the PPMID / IHD has failed, set fileOrFirmwareVersion to noFirmwareVersion; set imageResponseCode to firmwareReadFailure and process from Step 5;

- create a Response according to the requirements of Section 13.4.7, apply the Response Cryptographic Protection required for a Response of the relevant Message Category, and send the Response.

### Construction of Alerts

Grouping Header and SMD Signature shall be populated as required for a Alert of the SME.A.C Message Category. The Message Code shall be 0x0129.

The Business Originator ID field in the Grouping Header shall contain the CHF’s Entity Identifier.

The Business Target ID field in the Grouping Header shall contain the Entity Identifier in the CHF’s {accessControlBroker, digitalSignature, management} Trust Anchor Cell.

The Alert Payload shall be constructed according to the requirements of Section 11.7.5 and the Alerts shall be populated as specified in Table 11.7.4.

| Attribute name |  |  | Data Type |  |  | Value (blank cells mean the command specific value is derived by the encoding process) |  |  |  | Mandatory, OPTIONAL or DEFAULT value |  |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| @FirmwareUpgradeResults.AlertPayload |  |  |  | SEQUENCE |  |  |  |  |  |  |  |  |  |  |  |
| ____alertCode |  |  |  | INTEGER |  |  | 0x8F89 0x8F8A 0x8F8B |  |  |  | Mandatory |  |  |  | (1) 0x8F89: OTA Firmware Image Delivery to the target Device has failed: (2) 0x8F8A: OTA Firmware Image Delivery to the target Devices was successful; (3) 0x8F8B: Result of the Communications Hub reading the Firmware version of the PPMID or IHD |
| ____otherDeviceEntityIdentifier |  |  |  | OCTET STRING |  |  | Entity Identifier |  |  |  | Mandatory |  |  |  | The Entity Identifier of the ESME / GSME / HCALCS / PPMID / IHD the firmware upgrade is targeted at |
| ____fileOrFirmwareVersion |  |  |  | CHOICE |  |  |  |  |  |  | Mandatory |  |  |  | (1) OTA file version or firmware version from target device (2) In case of read failure noFirmwareVersion must be selected |
| ______firmwareVersion |  | OCTETT STRING |  |  | ZSE firmware version |  |  |  | Optional (CHOICE) |  |  |  | (1) 0x8F89 or 0x8F8A: The firmware version of the discarded or transfered Upgrade Image in ZSE format (2) 0x8F8B: The firmware version as reported from the target Device in ZSE format |  |  |
| ______noFirmwareVersion |  |  |  | OCTETT STRING |  |  | Null-data |  |  |  | Optional (CHOICE) |  |  |  | 0x8F8B: In case of failure or unsupported Attribute |
| ______ ImageResponseCode | INTEGER |  |  |  | imageDiscarded (0), hardewareVersionMismatch (1), fileTransferFailure (2), fileTransferSuccess (3), firmwareReadFailure (4), firmwareReadSuccess (5) |  |  | Mandatory |  |  |  | (a) 0x8F89: imageDiscarded (0); hardewareVersionMismatch (1); fileTransferFailure (2) (b) 0x8F8A: fileTransferSuccess (3) (c) 0x8F8B: firmwareReadFailure (4); firmwareReadSuccess (5) |  |  |  |

Table 11.7.4 @FirmwareUpgradeResults.AlertPayload population

### Firmware Upgrade Results – PPMID and IHD Payload – structure definition

Each instance of @FirmwareUpgradeResult.CommandPayload; FirmwareUpgradeResult.ResponsePayload and FirmwareUpgradeResult.AlertPayload shall be an octet string containing the DER encoding of the populated structure defined in this Section 11.7.4 which specifies the structure in ASN.1 notation.

FirmwareUpgradeResult  DEFINITIONS ::= BEGIN

CommandPayload ::= 	OtherDeviceEntityIdentifier

-- specify the 64 bit Entity Identifier of the Device

otherDeviceEntityIdentifier ::=	OCTET STRING,

ResponsePayload ::= 	SEQUENCE

{

-- specify the 64 bit Entity Identifier of the Device the upgrade has been

-- targeted at

otherDeviceEntityIdentifier 	OCTET STRING,

-- include the discarded file version or target device firmware version

fileOrFirmwareVersion	FileOrFirmwareVersion

-- specify whether the reading of the firmware was successful. In case of

-- failure select communicationFailure(5); in case of success select

-- firmwareReadSuccess(6)

imageResponseCode	ImageResponseCode

}

AlertPayload ::=	SEQUENCE

{

-- specify the Alert Code

alertCode	INTEGER(0..4294967295),

-- specify the 64 bit Entity Identifier of the Device the upgrade has been

-- targeted at

otherDeviceEntityIdentifier 	OCTET STRING,

-- include the OTA Upgrade Image file version or the target device firmware

-- version

fileOrFirmwareVersion	FileOrFirmwareVersion

-- specify the outcome

imageResponseCode	ImageResponseCode

}

FileOrFirmwareVersion ::= 	CHOICE

{

-- specify (a) the upgrade image version if it was replaced or

-- discarded by the Communications Hub or if the transfer to the target

-- device succeded or failed; (b) the target device firmware version if it has

-- been read successfully. The value shall be four octets in length and shall

-- correspond to the FileVersion field in the ZSE OTA Header

-- structure

firmwareVersion	OCTET STRING

-- in case the firmware version cannot be obtained from the target device

noFirmwareVersion	Null

}

ImageResponseCode ::= 	INTEGER

{

imageDiscarded	(0);

hardewareVersionMismatch	(1);

fileTransferFailure	(2);

fileTransferSuccess	(3);

firmwareReadFailure	(4);

firmwareReadSuccess	(5)

}

END

Appendix_1: New Alert Codes Created (remove this from draft once Alert Codes are defined and placed into the text)

Alert-1: 0x8F89 OTA Firmware Image Delivery to PPMID or IHD - Failure

Alert-2: 0x8F8A OTA Firmware Image Delivery to PPMID or IHD - Success

Alert-3: 0x8F8B OTA Firmware Image Delivery to PPMID or IHD - Read Firmware Version Result

Appendix_2: New Use Case for reading PPMID / IHD firmware (remove this from draft once Alert Codes and Message Code are defined and placed into the text)

Note: The tables may need changing based on the message type C or NC

See section 6.10.3 of  ZigBee Document 09-5264r23

As defined in section  6.10 of ZigBee Document 09-5264r23

As defined in sections 6.10.10 and 6.8.4 of ZigBee Document 09-5264r23

ZigBee Document 09-5264r23