This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

Proposed Change: SECMP0013 - Smart meter device diagnostics

#### Context, Business Requirements and Potential Solutions

This section provides the context to this Modification Proposal and summarises the key business requirements for it, as currently clarified and refined by the Working Group.

Context:

SECMP0013 seeks to provide suppliers with both a means of performing quality assurance (QA) ahead of installation on Smart Meters intended for live use, and fault diagnostics on Smart Meters returned by meter installers. This is not currently possible as:

- Smart Meters joined to a Communications Hub cannot be then subsequently joined to another one; and

- Communications to a Smart Meter cannot be re-established after it has been removed from an SMHAN to which it was joined.

This means that if suppliers joined a Smart Meter to a Communications Hub in QA facilities, they would not be able to join the same Smart Meter to another Communications Hub in a live environment, and vice versa. Further, suppliers would not be able to re-establish communications in a QA facility with a Smart Meter returned by an installer, where that Smart Meter had previously joined an SMHAN.

Business Requirements

The core functional requirements are to have mechanisms whereby:

- A Smart Meter can be instructed to leave an SMHAN, and return to a state where it can join a new SMHAN. It is assumed in this draft that the supplier has the Smart Meter’s ZigBee Install(ation) Code and so the Install Code is not set to a new value as part of this instruction. Note that:

  - A Smart Meters that has previously been installed in Consumer premises (prior to QA) will have a specific Network Operator’s (NO) certificates installed and so can only be re-installed in that NO’s area, if the Smart Meter is re-deployed for Consumer use; and

  - A Supplier is unlikely to have access to the Install Code for Smart Meters that have churned from another Supplier, and so this mechanism would not allow re-use in such scenarios, without the Smart Meter’s return to the installing Supplier.

- Communications with a Smart Meter, that had joined an SMHAN and then been removed from those Premises, can be re-established via a different Communications Hub, including one in a supplier QA facility. A pre-requisite is that the supplier can identify the CH to which the Smart Meter was previously connected (e.g. from records in their internal systems as to the SMHAN it was previously connected to) and that that CH is still in operation;

- Where a Smart Meter is joined to a CH for QA purposes, the supplier should be able to instruct it using the full range of Service Requests. This allows the fullest range of checks to be carried out, and any configuration changes to be made prior to re-deploying the Meter. However, it is assumed that the supplier would not generally wish to use either Commission or Decommission Service Requests in the QA facility, given the wider implications of such actions (although Decommissioning would be required for Devices found to be faulty and which have previously been recorded in the Smart Metering Inventory);

The core non-functional business requirements are to:

- provide interoperable mechanisms, not requiring physical changes on Smart Meters or Manufacturer specific elements; and

- be able to use these mechanisms on a Smart Meter that is intended for live deployment in Consumer Premises / has been deployed in Consumer Premises.

Constraints

Live systems

The last point above is significant, in terms of constraining potential solutions.

If a Smart Meter is intended for live use / has been in live use, then it must have been configured to use Public Key Certificates within the SMKI live structure. This means that the corresponding private keys are those used by live DCC and Supplier systems. These keys must not, from a security perspective, be replicated anywhere else (e.g. in QA specific solutions, separate from live systems). Thus, any solution to this Modification’s requirements would need to be implemented by live Supplier and DCC systems (There is no mechanism to migrate a Smart Meter from the live SMKI to a ‘non-live’ SMKI or vice-versa).

MPxN

When instructing the DCC to add a Smart Meter to an SMHAN, the supplier currently has to provide an associated MPxN. This association is used to check initially that the supplier making the request is that associated with the MPxN in registration data. It is subsequently used extensively as part of DCC’s access control, as required by the SEC. If MPxN were not provided, there would need to be significant change to DCC access control (and associated SEC requirements) bringing greater cost / longer time scales. Further, such changes may give rise to broader security concerns. Therefore, it is assumed that suppliers installing meters in their QA facilities will provide valid MPxNs which DCC’s copy of Registration Data shows them to be the supplier for.

Implementation

This change would be complied with by all CH and Smart Meters conforming to versions of the Technical Specifications which incorporate the Modification, and so to newly installed Devices.

This modification relies upon facilities introduced by BEIS CRPXXX and so cannot be implemented prior to the release which implements CRPXXX. If this Modification were to be implemented in the same release as CRPXXX, it would be implemented on all already installed CH. This is because CRPXXX is to be implemented on all installed CH by way of firmware upgrade. Alternatively, the changes in this modification would be rolled out to all installed CH, where there is a subsequent Release requiring such upgrades to implement other changes.

Currently, there is no planned Release which would require firmware upgrade to all installed Smart Meters. Thus, this change would not be rolled out to all installed Smart Meters. However, that does not stop Suppliers electing to undertake such upgrades.

There is no specific requirement for this Modification in isolation to mandate firmware upgrades on any installed Devices.

Interoperability

For Smart Meters which comply with this change, the new Command instructing them to leave the SMHAN (see Annex A) would be routed to them and actioned by them, regardless of CH version. Related, the resulting Response would be routed back through the CH, again regardless of CH version. This is because of the GBCS CH requirement in BEIS IRP521, which CH have complied with since go-live.

If (1) the CH does not support this change and (2) the Smart Meter is removed from the SMHAN without having first processed the new Command instructing it to leave, it would not be possible to use the mechanisms covered by this modification. This is true regardless of the Technical Specification the Smart Meter supports.

Proposed Solutions

Requirement 1 – the supplier telling a Smart Meter it is to ‘divorce’ from its current SMHAN

There is no existing mechanism for a supplier or a Smart Meter to do this, and so a new Service Request from suppliers, and a corresponding new GBCS Command to Smart Meters, would be required.

Given that this Command would require a Smart Meter to change the security credentials it uses, it would seem to require that the new Command is Critical (and so the new Service Request is Critical) in line with SMETS / GBCS.

Given that this Command has to be supported by both GSME and ESME, and that there is no existing DLMS COSEM or ZigBee functionality that relates to it, it would seem that the new Command would be ASN.1 based (cf: the GBCS ‘join’ Commands, which are ASN.1 Commands for the same reason).

Where successfully executed, the Command would require, in ZigBee terms, that the Smart Meter ‘discard its network settings and link key, and revert to its install code, and automatically return to auto-join’.

A successful Response to this Command would require that the DCC updates the Smart Metering Inventory so as to:

- set the SMI status for the Smart Meter to ‘pending’, to reflect the fact that it is now in a state where it can be whitelisted on other SMHAN; and

- ensure the Device is no longer Associated with any other Devices or any MPxN.

Annex A lays out the Use Case and associated SEC changes to implement this new Service Request and GBCS Use Case.

Prior to sending this ‘divorce’ Command, suppliers may wish to make configuration changes to the Smart Meter so that it can be re-used (e.g. clearing Devices from the Smart Meter’s Device Log). This can be done using existing Service Requests.

After sending this Command, suppliers are likely to want to remove the Smart Meter details from the CHF Device Log e.g. so that replacement Smart Meters can be installed. This can be done using the existing Service Request.

Requirement 2 – re-establishing communications with a previously installed Smart Meter

If an installer returns a Smart Meter for QA that meter will be in one of three states:

- It is working, had not joined an SMHAN or had been ‘divorced’ from it. In this case, the Smart Meter will be able to join an SMHAN in the supplier’s QA facility using existing mechanisms. The supplier can identify this state by attempting such a join;

- It is working, had joined the SMHAN and had not subsequently been ‘divorced’ from it. If an attempt is made to join such a Device to an SMHAN in the supplier’s QA facility using existing mechanisms, that attempt will fail. The new mechanism described in this section would allow communications to be re-established (and so the meter functions could subsequently be checked and the smart meter ‘divorced’ from the SMHAN so that it could be re-used);

- It is not working sufficiently for communications to be established.  In this case, the attempted actions at steps 1 and 2 will fail, informing the supplier that the meter cannot be used.

The mechanism at step 2 does not exist in current technical specifications. The information required to re-establish Smart Meter communications would be known to the CH to which the Smart Meter was connected.:

- It may still be in that CH’s CHF Device Log; or

- It may have been removed from that CH’s CHF Device Log. In this case, it is assumed in this drafting that the separate BEIS CRPXXX will have added a requirement for CH to support a CHF Historic Device Log. This historic log would hold details of the Devices most recently removed from the CHF Device Log.

To re-establish communication with the Smart Meter in the supplier QA facility, this information would need to be:

- read from the relevant CH in Consumer Premises;

- written to the CHF Device Log of the CH in the supplier QA facility.

The ‘Trust Center Swap Out mechanism (which all Devices are already required to support), would then allow the Smart Meter to join the SMHAN in the supplier QA facility, and so it could be communicated with.

To achieve this, two new Service Requests and one corresponding GBCS Command / Response would be required:

- Supplier read of CHF Historic and Current Device Log (new SR and associated GBCS Use Case). This would require CH to provide access to these two DLMS COSEM objects (one of which already exists on CH; the other would be added by the BEIS change);

- Supplier specified CHF Device Log Restore. CH already support such a GBCS Command / Response, although the corresponding Service Request does not allow the supplier to specify the CHF Device Log. The new Service Request would be to allow the Supplier to specify the Device Log along with the ID of the Smart Meter they wish to connect (so one of the IDs in the Device Log), and to require the DCC (on receipt of a successful Response) to Associate the Smart Meter in question with the target CH in the QA facility.

Given that the corresponding, existing GBCS Use Cases:

- are non-critical, it is assumed that the additional facilities are non-critical; and

- use DLMS COSEM structures, it is assumed the additional facilities will do the same.

Annex B lays out the Use Case and associated SEC changes to implement the new Service Requests and GBCS Use Case.

Requirement 3 – support all Service Requests on a Meter in QA facilities

No additional changes are required to support this requirement.

Annex A – changes to support Requirement 1

The following additions are to be incorporated at the version of SEC and Technical Specifications at which this Modification is to be implemented.

APPENDIX AB: Service Request Processing Document

Immediately before clause 16, insert the following clause:

15.5 Where the DCC receives a Response from a Device indicating the Successful Execution of a 'Disconnect Smart Meter From HAN' Service Request, the DCC shall set the SMI Status of the relevant Device to ‘pending’, disassociate the Device in the Smart Metering Inventory from any Communication Hub Function with which it is Associated and end any record of linkages between the Device and MPAN(s) or MPRN (as applicable).

APPENDIX E DCC User Interface Services Schedule

Add the following row:

| Service Reference | Service Reference Variant | Description | Eligible Users | Target Response Time | Non-Device Services | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| [TBC] | [TBC] | Disconnect Smart Meter From HAN | Import Supplier, Gas Supplier | 30 seconds |  |  |

APPENDIX AD DCC User Interface Specification

[TBC but to add new Service Request]

Appendix AF - Message Mapping Catalogue

[TBC but to add new Service Request]

Schedule 9 - SME Technical Specifications Version [TBC]

In section 4.5.3 add the following new subsection:

#### TBC Disconnect from Smart Metering Home Area Network

A Command to disconnect from the ZigBee SEP Smart Metering Home Area Network over which the GSME is currently communicating, and place the GSME in a state where it is capable of establishing communications with a new ZigBee SEP Smart Metering Home Area Network.

In section 5.6.3 add the following new subsection:

#### TBC Disconnect from Smart Metering Home Area Network

A Command to disconnect from the ZigBee SEP Smart Metering Home Area Network over which the ESME is currently communicating, and place the ESME in a state where it is capable of establishing communications with a new ZigBee SEP Smart Metering Home Area Network.

Schedule 8 - GB Companion Specification Version [TBC]

Make the changes shown to Section 13.1:

  - Introduction – informative

This Section 13 includes the Use Cases related to the management of Security Credentials on Devices in terms of the relevant Commands, Responses and Alerts:

Section 13.2 - CS02a Provide Security Credential Details Command and Response;

Section 13.3 - CS02b Update Security Credentials Command, Response and Alert;

Section 13.4 - CS02c Issue Security Credentials;

Section 13.5 - CS02d Update Device Certificates on Device;

Section 13.6 - CS02e Provide Device Certificates from Device;

Section 13.7- Pair-wise Authorisation of Devices (covered by various Join / Unjoin Use Cases); and

Section 13.8 - GPF Device Log Backup and Restore (GCS59 and GCS62); and

Section 13.9 – CS02f Disconnect Smart Meter from HAN.

Add the section 13.9 in the embedded document to the end of Section 13

To Table 20, Use Case reference tab, add the first row specified in the Use Case reference tab of the Annex C embedded document.

Other documents

Changes related to testing may also be required

Annex B – changes to support Requirement 2

The following additions are to be incorporated at the version of SEC and Technical Specifications at which this Modification is to be implemented.

Note:

- No changes to CHTS are specified since there will be existing requirements for CH to support ‘Read Operational Data’ (and the CHF Historic Device Log is Operational Data), Read Configuration Data’ (and the CHF Device Log is Configuration Data) and ‘Restore CHF Device Log’. [DN: to confirm whether the BEIS CRP will include the reads already, and therefore they do not need to be part of this modification]

APPENDIX E DCC User Interface Services Schedule

Add the following rows:

| Service Reference | Service Reference Variant | Description | Eligible Users | Target Response Time | Non-Device Services | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| [TBC] | [TBC] | Read Current and Historic HAN Device Logs | Import Supplier, Gas Supplier | 30 seconds |  |  |
| [TBC] | [TBC] | Restore Specified HAN Device Log for Specified Smart Meter | Import Supplier, Gas Supplier | 30 seconds |  |  |

APPENDIX AC Inventory Enrolment and Withdrawal Procedures

Amend clause 4.3 as follows:

4.3 Following the Successful Execution of a ‘Restore Specified HAN Device Log for Specified Smart Meter’ or an 'Update HAN Device Log' Service Request requesting the addition of a Device to the Device Log of a Communications Hub Function, the DCC shall: (a) update the Smart Metering Inventory to Associate the Device with the applicable Communications Hub Function; (b) in the case of Smart Meters only, record the MPAN(s) or MPRN (as applicable) provided within the Service Request against that Smart Meter and notify the Electricity Distributor or Gas Transporter (as applicable) of the MPAN(s) and/or MPRN and of the Smart Meter's Device ID and Device Type; and (c) other than in the case of a Type 2 Device, set the SMI Status of the Device to 'whitelisted'.

APPENDIX AD DCC User Interface Specification

[TBC but to add new Service Requests. The ‘Restore Specified HAN Device Log for Specified Smart Meter’ SR will need to have 3 parameters: Smart Meter EUI-64 ID; the base64 encoded version of the CHF Device Log to be restored and the MPxN to be linked to the Smart Meter.]

Appendix AF - Message Mapping Catalogue

[TBC but to add new Service Requests]

Schedule 8 - GB Companion Specification Version [TBC]

To Table 20, Use Case reference tab, add the second row specified in the Use Case reference tab of the Annex C embedded document.

To Table 20, SMETS Required objects tab, add the rows specified in the SMETS Required objects tab of the Annex C embedded document.

This produces the new CCS07 Use Case and Message Template as in the following embedded documents:

[DN: Use Case and Message Templates to be incorporated]

Other documents

Changes related to testing may also be required

Annex C – GBCS Table 20 additions

This adds the new Use Cases and corresponding GBZ and DLMS COSEM Message Templates. The additions are as per the following embedded documents

Resulting Use Cases and GBZ Message Templates

Resulting DLMS COSEM Message Templates:

Specifically, the PANID of the SMHAN, the Smart Meter’s MAC address and the Hash of the Trust Center Link Key.