| SECAS Contact:. Name: Harry Jones. Number: 020 7090 1534. Email: SEC.change@gemserv.com. Legal Text |
| --- |
| SECMP0023: |
| Correct Units of Measure for Uncontrolled Gas Flow Rate |
| Summary The Great Britain Companion Specification (GBCS) currently limits the unit values of the Uncontrolled Gas Flow Rate (UGFR), on Gas Smart Metering Equipment (GSME), to be set in whole metres cubed per hour. This modification proposes to allow this unit to be set to an accuracy of ten-thousandths of metres cubed per hour. |  |
| About this document This document contains the draft SEC legal text that will deliver the intent of SECMP00023. |  |

# Draft Legal Text

This section sets out the draft legal text changes for SECMP0023. These changes have been drafted against the Smart Energy Code (SEC) version 5.13.

Please note that SEC Schedule 8 contains embedded documents which are also being amended by this modification. These embedded documents are included below as they would be set out in SEC Schedule 8. To review the changes to the embedded documents, you can click on and open these files.

## SEC Schedule 8 ‘GB Companion Specification Version’

### Amend Table 20 as shown in the embedded document:

Add the rows with green shading in each of the tabs in the embedded document ‘SECMP00023 Table 20’, to the corresponding tab in GBCS Table 20:

This adds the new Use Cases and corresponding GB Zigbee (GBZ) Message Templates. The additions are as per the following embedded documents.

### Amend the Use Case as shown in the embedded link:

## SEC Schedule 11 ‘Technical Specification Applicability Tables’

### Add a new row as below:

Table 1 SMETS and Relevant Versions of GBCS

| SMETS Version | Installation Start Date | Installation End Date | Maintenance Start Date | Maintenance End Date | Relevant GBCS Version(s) | GBCS Applicability Period Start Date | GBCS Applicability Period End Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMETS v[X.X] | TBC | Not yet determined | TBC | Not yet determined | [TBC] | [TBC] | Not yet determined |

## SEC Appendix AD ‘DCC User Interface Specification’

### Amend section 3.8.58 as follows:

    - 3.8.58	Update Device Configuration (Gas Flow)

      - 3.8.58.1	Service Description

| Service Request Name | - UpdateDeviceConfiguration(GasFlow) |  |
| --- | --- | --- |
| Service Reference | - 6.7 |  |
| Service Reference Variant | 6.7 |  |
| Eligible Users | Gas Supplier (GS) |  |
| Security Classification | Critical |  |
| BusinessTargetID - Device Type applicable to this request | Gas Smart Meter (GSM) |  |
| Can be future dated? | No |  |
| On Demand? | Yes |  |
| Capable of being DCC Scheduled? | No |  |
| Command Variants applicable to this Request - (Only one populated) | For Service Request 4 - Transform For Signed Pre-Commands 5 - Send (Critical) 6 - Return for local delivery (Critical) 7 - Send and Return for local delivery (Critical) |  |
| Common Header Data Items | See clause Error! Reference source not found. |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause Error! Reference source not found. for more details on processing patterns - Acknowledgement - Response to Transform Request - PreCommand Format - Service Response from Device – GBCSPayload - Response to a Command for Local Delivery Request - LocalCommand Format Also see Response Section below for details specific to this request |  |
| Response Codes possible from this Service Request | See clause Error! Reference source not found. for Common Response Codes |  |
| GBCS Cross Reference | Electricity | Gas |
| GBCS v1.0 MessageCode | N/A | 0x007D |
| GBCS v1.0 Use Case | N/A | GCS24 |
| GBCS v1.0 Use Case Name | N/A | Set Uncontrolled Gas Flow Rate and Supply Tamper State on the GSME |
| GBCS vn.0 Message Code | N/A | 0x00FC |
| GBCS vn.0 Use Case | N/A | GCS24a |
| GBCS vn.0 Use Case Name | N/A | GCS24a Set Uncontrolled Gas Flow Rate at greater resolution and Supply Tamper State on the GSME |

      - 3.8.58.2	Specific Data Items for this Request

UpdateDeviceConfigurationGasFlow Definition

| Data Item | Description / Valid Set | Type | Mandatory | Default | Units |
| --- | --- | --- | --- | --- | --- |
| UncontrolledGasFlowRate | The flow rate in units of volume per unit time used in the detection of uncontrolled flow of gas on Enablement of Supply | xs:unsignedShort | Yes | None | m3/hour |
| UncontrolledGasFlowRateDecimal | The flow rate in units of volume per unit of time used in the detection of uncontrolled flow of gas on Enablement of Supply. Accepts decimal places. | Restriction of Xs:decimal (minInclusive = 0, maxInclusive = 6.5535, fractionDigits=4) | Yes | None | m3/hour |
| SupplyDepletionState | A setting to control the state of the Supply in the case of loss of power to the Gas Smart Meter Valid set: - Unchanged - Locked | Restriction of xs:string (Enumeration) | Yes | None | N/A |
| SupplyTamperState | A setting to control the state of the Supply in the case of a Tamper Event being detected Valid set: - Unchanged - Locked | Restriction of xs:string (Enumeration) | Yes | None | N/A |
| StabilisationPeriod | Value indicating the time given to allow the flow to stabilize. It is defined in units of tenths of a second | Restriction of xs:positiveInteger (minInclusive = 1, maxInclusive = 255) | Yes | None | 10th second |
| MeasurementPeriod | Value indicating the period over which the flow is measured and compared against the Uncontrolled Flow Threshold value. It is defined in units of 1 second | Restriction of xs:positiveInteger (minInclusive = 1, maxInclusive = 65535) | Yes | None | Seconds |

Table 1 : UpdateDeviceConfigurationGasFlow (sr:UpdateDeviceConfigurationGasFlow) data items

      - 3.8.58.3	Specific Validation for this Request

See clause 3.2.5 for general validation applied to all Requests.

| Response Code | Response Code Description |
| --- | --- |
| E060701 | 1. For the GSME Devices with a Firmware version certified to a GBCS version prior to vn.0 i. If the value of uncontrolled gas flow rate is specified by using the XML tag UncontrolledGasFlowRate then the Service Request is mapped to GBCS Use Case GCS24 ii.If the value of uncontrolled gas flow rate is specified by using the XML tag UncontrolledGasFlowRateDecimal then the Service Request is rejected with error E060701 |
| E060701 | For the GSME Devices with a Firmware version certified to GBCS vn.0 or later. i. If the value of uncontrolled gas flow rate is specified by using the XML tag UncontrolledGasFlowRate then the Service Request is rejected with error E060701 ii.If the value of uncontrolled gas flow rate is specified by using the XML tag UncontrolledGasFlowRateDecimal then the Request is mapped to GBCS Use Case GCS24a |

## SEC Appendix AF ‘Message Mapping Catalogue’

### Amend section 5.55 as follows:

  - 5.55	Update Device Configuration (Gas Flow)

    - 5.55.1	Service Description

| Service Request Name | - UpdateDeviceConfiguration(GasFlow) |
| --- | --- |
| Service Reference | - 6.7 |
| Service Reference Variant | 6.7 |

    - 5.55.2	MMC Output Format

The Service Response contains only status information, as set out in Section Error! Reference source not found. of this document. The xml type within the SMETSData element is UpdateDeviceConfigurationGasFlowRsp. The header data items appear as set out immediately below.

      - 5.55.2.1	Specific Header Data Items

| Data Item | Gas Response |
| --- | --- |
| GBCS v1.0 GBCSHexadecimalMessageCode | 0x007D |
| GBCS v1.0 GBCS Use Case (for reference - not in header) | GCS24 |
| GBCS vn.0 GBCSHexadecimalMessageCode | 0x00FC |
| GBCS vn.0 GBCS Use Case (for reference - not in header) | GCS24a |

Table 2 : Update Device Configuration (Gas Flow) MMC Output Format Header data items

Note that if a new version of the SEC is designated before the submission of the Final Modification Report to the Change Board or Authority, the drafting will be checked to make sure there are no consequential impacts. Our current understanding is that the specifications that SECMP0006 is amending are not expected to change as part of any forthcoming designations.

The “Installation Start Date” will be the SECMP0023 implementation date, which is to be determined by the Authority.

The “Maintenance Start Date” will be the same as the “Installation Start Date”.