# Proposed Change: SECMP0023 - Correct Units of Measure for Uncontrolled Gas Flow Rate

#### Requirement

This section summarises the key parts of the requirement for this SEC Modification Proposal, as currently clarified and refined by the Working Group. The following solution(s) sections detail the required changes to SEC and wider documents that are proposed to implement this requirement. Where there are multiple solutions sections, they reflect the alternatives the Working Group wished to consider.

Context

SMETS requires that GSME have an Uncontrolled Gas Flow Rate (UGFR) configuration data item. Its function relates to Supply being Enabled on the User Interface of the GSME. Specifically, when Supply is Enabled, the GSME has to check that the flow rate does not exceed the UGFR. If it does exceed the value set, then the Supply is disabled.

Currently GBCS requires that the UGFR is set in whole numbers of meters cubed per hour.

Functional Requirements

This modification is to require in GBCS that the UGFR can be set on GSME in ten-thousandths of meters cubed per hour, so in tenths of a litre per hour. [Note, the Working Group agreed a greater level of granularity than specified in the original modification].

There is no requirement in this modification for previously installed GSME to be upgraded to support this change.

As per CHTS 4.6.3.9, a copy of this configuration data item is not maintained on the GPF (and so not available to other Devices on the HAN). Thus, no related changes are required for Devices other than GSME.

Implementation Obligations

Supplier licences require the installation of GSME that comply with SMETS; SMETS requires that GSME comply with GBCS. Therefore, the GBCS requirement specified in this modification would be a requirement on Suppliers in relation to GSME they install, which must comply with the version of GBCS in to which these changes have been incorporated.

Changes

To deliver the Functional Requirements in line with the approach that revisions are not made to existing Use Cases, the existing GBCS Use Case ‘GCS24 Set Uncontrolled Gas Flow Rate and Supply Tamper State on the GSME’ is to be replaced by a new Use Case (‘GCS24a Set Uncontrolled Gas Flow Rate at greater resolution and Supply Tamper State on the GSME’). The only difference in the new Use Case is the fixed value of the Divisor parameter in the Set Uncontrolled Flow Threshold Command, which changes from 1 to 10,000.

As agreed by the Working Group, there would be corresponding changes to DUIS to modify the existing SRV 6.7 ‘Update Device Configuration (Gas Flow)’ with a choice to specify:

- the UGFR without decimal places (to align with the existing GBCS Use Case (GCS24); or

- the UGFR with decimal places (to align with the new GBCS Use Case (GCS24a).

Note that if the GSME complies with the version of SMETS which implements this change, the DCC will send the new Use Case to the GSME, regardless of whether there are any digits after the decimal point.

If the UGFR is without decimal places and is targeted to a GSME that does not support this modification, the DCC will send the existing Use Case to the GSME.

If the UGFR is with decimal places and the Service Request is targeted to a GSME that does not support this modification, this will result in the DCC returning an error. A new error code will be required to accommodate this modification.

In line with the input from SSC, anomaly detection packet inspection is required for resulting Commands to ensure that the ‘Uncontrolled flow threshold’ is greater than a specified value.

Compatibility

Given that, as stated earlier, this change is not visible to other HAN Devices, there should be no issues of interoperability between Devices.

Dependencies

None identified.

Business Requirements relating to testing

- The DCC will provide Testing Services to support the implementation of SECMP0023 to assess the interoperability of User Systems with DCC Systems and Smart Metering Devices.

- The DCC will provide an analysis including supporting assumptions and rationale, of any testing required to the DCC Total System.

- The DCC will prepare a report setting out the scope, phases, timetable, Testing Participants, any assumptions and rationale in relation to SECMP0023 testing.

- The testing environment that the DCC provides in support of SECMP0023 as part of Testing Services will be open to all User Roles eligible to send the Service Request (Gas Suppliers). This environment should be made available for a minimum of 15 Working Days, depending on the impact of the change. The DCC must provide the costs and assumptions associated with providing this Testing Service, including whether the testing costs are based on a set number of Users utilising the Testing Service, i.e. up to 10 Users, noting that at least two Large Suppliers may test the functionality. This is to ensure it operates correctly before it is put into the End-to-End and Production environments.

- The objective of testing as part of the Testing Services will be to ensure that, in response to the Service Request, the User receives the corresponding Service Response from the DCC.

- As part of the Testing Services, the DCC will provide Users with a corresponding version of the Parse and Correlate software and Message Mapping Catalogue.

- The acceptance criteria for testing as part of the Testing Services will be, following successful execution of the corresponding Command, the User receives the corresponding Service Response from the DCC that the UGFR has been successfully set.

- As reasonably required, the DCC will provide:

- a reasonable number of Test CH for use in the testing environment which represent every combination of HAN and WAN Variant;

- Test Stubs (or other alternative arrangements) to emulate meter behaviour of version(s) of SMETS in force prior to the Release as well as the version of SMETS which will be effective on the Release date.

- As the DCC will apply all applicable Anomaly Detect Thresholds (ADTs) to the Signed Pre-Commands in the test environment, Users will be expected to set User volume ADTs and data value ADTs as applicable. The process for setting the ADTs as well as for receiving notifications when a Signed Pre-Command has been quarantined will be part of the test environment.

#### Solution

This section details the required changes to SEC and wider documents that are proposed to implement the requirement for this SEC Modification Proposal. Please note that all numbering, messages codes, alert codes and so on are based on GBCS 2.0 Draft 4 and related document versions.

SEC main body change

None

SMETS changes

None

CHTS changes

None

GBCS changes

To create the new Use Case and remove the old one, in Table 20 of GBCS:

- In the ZigBee Command tab:

  - Select all rows where column E (Use Case) contains GCS24;

  - Copy all selected rows to the end of the tab

  - In the copied rows, set column E (Use Case) to be ‘GCS24a’

  - Select the row where column E (Use Case) contains ‘GCS24a’ and column B (Parameter) contains ‘Divisor’ and, in this row, set column G (Value) to be ‘0x2710’, which is 10,000 in hexadecimal

  - Select all rows where column E (Use Case) is ‘GCS24’ and, in all selected rows, clear all cells then set column A to ‘Intentionally blank’;

- In the SMETS Required objects tab:

  - Select all rows where column AM (ZSE Update UC) contains ‘GCS24’ and set the contents of column AM to ‘GCS24a Set Uncontrolled Gas Flow Rate at greater resolution and Supply Tamper State on the GSME’

  - Set cell CT1396 to ‘0x2710’, to align to the above Divisor

- In the Use Case reference tab:

  - Copy row 126, which relates to the existing Use Case, to the bottom of the tab;

  - In the newly created row:

    - Set column F (Use Case (GBZ)) to ‘GCS24a Set Uncontrolled Gas Flow Rate at greater resolution and Supply Tamper State on the GSME’

    - Set column G (Message Code (Gas)) to ‘0x00FC’.

  - In row 126 (so the existing Use Case):

    - clear all cells in the row except for columns F:G, since these are to be retained to avoid re-use of the old Message Code and Use Case tag;

    - set column A to ‘This row is not used’;

    - set column E to ‘x’; and

    - add a comment to column F ‘Unused since GBCS [TBC]’.

[DN: the TBC will depend on the GBCS Version in which this modification is implemented]

The resulting changes to Table 20, and new Use Case and Message Template are as embedded below:

DUISS changes

[tbc dependent on DUIS changes]

DUIS changes

[tbc by the DCC to cater for the new SR]

MMC changes

None

Other SEC document changes

[tbc dependent on DUIS changes, for example in relation to testing documents]

Changes to documents outside of SEC

None