This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

SECMP0025 ‘Electricity Network Party Access to Load Switching Information’

Annex A

Solution Design Specifications – Version 2.0

About this document

This document contains the Business Requirements that would be required to deliver this Modification Proposal.

# Business Requirements

## Business Requirements relating to accessing the ALCS Event Log through Service Request 6.13:

- The User Role of Electricity Distributor (ED) is to be provided with access to the ALCS Event Log through SR6.13 “Read Event Or Security Log” for ESME built according to the Specifications where this Modification is in force [Note there is no requirement to retrospectively apply this to installed ESME, and so access via SR6.13 will not be possible for such previously installed ESME]. In turn, this requires that, in the Specifications where this Modification is in force, the GBCS Use Case, ECS35f “Read ALCS Event Log” is replaced with an equivalent Use Case that adds Electricity Distributor access.

## Business Requirements relating to Electricity Distributor access to SR7.7 – Read Auxiliary Load Switch Data

- The User Role of Electricity Distributor (ED) is to be provided access to SR 7.7 “Read Auxiliary Load Switch Data” in order to read ALCS Data. This is a DCC only change, and so will provide access to such data on all ESME, whether installed prior to this change or not.

## Business Requirements relating to creating a new DCC Alert for Electricity Distributors

- The DCC would create a new DCC Alert to notify the relevant Electricity Distributor (ED) whenever the DCC:

a. receives a successful Response from an ESME to either:

i. change the ALCS configurations, which may include an ALCS Calendar change (so a successful Response to a ‘ECS46c Set HC ALCS and ALCS configuration in ESME (excluding labels)’ Command), or

ii. change the ALCS labels (so a successful Response to a ‘ECS46a Set HC ALCS or ALCS Labels in ESME’ Command).

b. Receives an Alert from an ESME meaning a future dated change to the Auxiliary Load Control Switches Calendar has successfully been made (so an Alert with Alert Code of 0x8F66 (meaning ‘Future – date HAN Interface Command Successfully Actioned’ ) containing the COSEM-ATTRIBUTE-DESCRIPTOR of 0x232800005E2C801A06 (meaning AuxiliaryLoadControlSwitchesCalendar))

- The new Alert shall contain only the Device ID of the ESME on which the ALCS configurations had been changed.

- The new DCC Alert to notify the relevant Electricity Distributor (ED) whenever the DCC receives a successful Response from an ESME to change the ALCS configurations shall be generated equally for all ESMEs recorded in the DCC database, including those deployed before this change.

## Business Requirements relating to User Testing

- (Optional) DCC should act as the supplier party during User Testing of the new functionality. Electricity Network Users would undertake Testing of the new functionality.

Note that (6) is marked optional and the DCC are to provide split out costings for each of those elements in their Impact Assessment.

# Solution

This section details the required changes to SEC and wider documents that are proposed to implement the requirement for this SEC Modification. Please note that all numbering, messages codes, alert codes and so on are based on GBCS 2.0 Draft 4.

## GBCS changes

To create the new Use Case to replace ECS35f, in Table 20 of GBCS:

1) In the ‘Use Case Reference’ tab, copy the row that contains ‘ECS35f’ in column C (‘Use Case (DLMS/ASN.1)’) to the end of the tab.

2) In that copied row in the ‘Use Case Reference’ tab:

i) In column C (‘Use Case (DLMS/ASN.1)’) replace ‘ECS35f’ with ‘ECS35g’;

ii) In column D (‘Message Code’) replace ‘0x00BA’ with ‘0x00FD’;

iii) Set column M (‘Access: Network Operator (NC)’) to ‘A’.

3) In the ‘SMETS Required objects’ tab, in all rows where column AH (‘DLMS COSEM Read UC’) contains ‘ECS35f’:

a) copy the value in column Q (‘Access: Supplier (C)’) to column U (‘Access: network Operator (C)’)

b) copy the value in column R (‘Access: Supplier (NC)’) to column V (‘Access: network Operator (NC)’)

c) in column AH (‘DLMS COSEM Read UC’) replace ‘ECS35f’ with ‘ECS35g’

To mark the old ‘ECS35f’ Use Case as unused and guard against accidental future re-use of either the message code or the use case tag, in the ‘Use Case Reference’ tab:

1) Set column A (‘Use Case Name’) to ‘This row is not used’;

2) Set column B (‘DLMS/ASN.1 message Location (1= in html, TOC = in main GBCS, x = do not use Message Code or Use Case tag in columns D and C’) to ‘x’;

3) To column C (‘Use Case (DLMS/ASN.1)’) and to column D (‘Message Code’) add a comment ‘Unused since GBCS [DN: last version at which ECS35f was used, so the last GBCS version prior to this change being implemented]’;

4) Clear columns E to AS inclusive.

This results in a revised Table 20 and replacement Use Case / Message Template for ECS35f:

## DUIS and MMC changes

The solution requires changes to DUIS and MMC; these changes are detailed in the Legal Text for SECMP0025.