This document is classified as White in accordance with the Panel Information Policy. Information can be shared with the public, and any members may publish the information, subject to copyright.

SECMP0025 ‘Electricity Network Party Access to Load Switching Information’

Annex B

Legal text – version 2.0

About this document

This document contains the redlined changes to the SEC that would be required to deliver this Modification Proposal.

These changes have been drafted against the DCC User Interface Specification (DUIS) version 2.0 and GB Companion Specification (GBCS) versions 2.0, 2.1 and 3.1.

Schedule 8 ‘GB Companion Specification’ versions 2.0, 2.1 and 3.1

## Changes to Table 20 – replace Use Case ECS35f with ECS35g as follows:

### The addition of ‘ECS35g’ Use Case and ‘0x00FD’ Message Code

‘Use Case Reference’ Tab

- In column C (‘Use Case (DLMS/ASN.1)’) replace ‘ECS35f’ with ‘ECS35g’;

- In column D (‘Message Code’) replace ‘0x00BA’ with ‘0x00FD’;

- Set column M (‘Access: Network Operator (NC)’) to ‘A’.

‘SMETS Required Objects’ Tab

- copy the value in column Q (‘Access: Supplier (C)’) to column U (‘Access: network Operator (C)’)

- copy the value in column R (‘Access: Supplier (NC)’) to column V (‘Access: network Operator (NC)’)

- in column AH (‘DLMS COSEM Read UC’) replace ‘ECS35f’ with ‘ECS35g’

### The discontinuation of ‘ECS35f’ Use Case and ‘0x00BA’ Message Code

(To prevent future accidental use of either ‘ECS35f’ Use Case and ‘0x00BA’ Message Code)

‘Use Case Reference’ tab:

- Set column A (‘Use Case Name’) to ‘This row is not used’;

- Set column B (‘DLMS/ASN.1 message Location (1= in html, TOC = in main GBCS, x = do not use Message Code or Use Case tag in columns D and C’) to ‘x’;

- To column C (‘Use Case (DLMS/ASN.1)’) and to column D (‘Message Code’) add a comment ‘Unused since GBCS [DN: last version at which ECS35f was used, so the last GBCS version prior to this change being implemented]’;

- Clear columns E to AS inclusive.

Revised Table 20 with Replacement Use Case / Message Template for ECS35f:

Appendix AD ‘DCC User Interface Specification’ version 2.0

## Changes to Service Request 6.13 as follows:

The addition of ‘ECS35g’ Use Case and ‘0x00FD’ Message Code and the removal of ‘ECS35f’ Use Case and ‘0x00BA’ Message Code.

| Service Request Name | - ReadEventOrSecurityLog |  |
| --- | --- | --- |
| Service Reference | - 6.13 |  |
| Service Reference Variant | 6.13 |  |
| Eligible Users | Import Supplier (IS) Gas Supplier (GS) Electricity Distributor (ED) Gas Transporter (GT) Registered Supplier Agent (RSA) |  |
| Security Classification | Non Critical |  |
| BusinessTargetID - Device Type applicable to this request | Electricity Smart Meter (ESME) Gas Smart Meter (GSME) Gas Proxy Function (GPF) Communications Hub Function (CHF) |  |
| Can be future dated? | No |  |
| On Demand? | Yes |  |
| Capable of being DCC Scheduled? | No |  |
| Command Variants applicable to this Request - (Only one populated) | 1 - Send (Non-Critical) 2 - Return for local delivery (Non-Critical) 3 - Send and Return for local delivery (Non-Critical) |  |
| Common Header Data Items | See clause 3.4.1.1 |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause 3.5 for more details on processing patterns - Acknowledgement - Service Response from Device – GBCSPayload - Response to a Command for Local Delivery Request – LocalCommand Format Also see Response Section below for details specific to this request |  |
| Response Codes possible from this Service Request | See clause 3.5.1.0 for Common Response Codes |  |
| GBCS Cross Reference | Electricity | Gas |
| GBCS v1.0 MessageCode | 0x0048 (Device event log) 0x0049 (Device security log) 0x0093 (CHF event log) 0x0094 (CHF security log) 0x00B9 (ESME power event log) 0x00BA (ESME HAN ALCS event log ) | 0x0014 (Device event log) 0x00A1 (Device security log) |
| GBCS v1.0 Use Case | ECS35a (Device event log) ECS35b (Device security log) ECS35c (CHF event log) ECS35d (CHF security log) ECS35e (ESME power event log) ECS35f (ESME HAN ALCS event log) | CS10a (Device event log) CS10b (Device security log) |
| GBCS v2.0 MessageCode | 0x0048 (Device event log) 0x0049 (Device security log) 0x0093 (CHF event log) 0x0094 (CHF security log) 0x00B9 (ESME power event log) 0x00BA (ESME HAN ALCS event log) | 0x0014 (Device event log) 0x00A1 (Device security log) |
| GBCS v2.0 Use Case | ECS35a (Device event log) ECS35b (Device security log) ECS35c (CHF event log) ECS35d (CHF security log) ECS35e (ESME power event log) ECS35f (ESME HAN ALCS event log) | CS10a (Device event log) CS10b (Device security log) |
| GBCS vn.0 MessageCode | 0x0048 (Device event log) 0x0049 (Device security log) 0x0093 (CHF event log) 0x0094 (CHF security log) 0x00B9 (ESME power event log) 0x00FD (ESME HAN ALCS event log) | 0x0014 (Device event log) 0x00A1 (Device security log) |
| GBCS vn.0 Use Case | ECS35a (Device event log) ECS35b (Device security log) ECS35c (CHF event log) ECS35d (CHF security log) ECS35e (ESME power event log) ECS35g (ESME HAN ALCS event log) | CS10a (Device event log) CS10b (Device security log) |

## Changes to Service Request 7.7 as follows:

The addition of Electricity Distributor (ED) as an Eligible Users.

| Service Request Name | ReadAuxiliaryLoadSwitchData |  |
| --- | --- | --- |
| Service Reference | 7.7 |  |
| Service Reference Variant | 7.7 |  |
| Eligible Users | Import Supplier (IS) Electricity Distributor (ED) Other User (OU) |  |
| Security Classification | Non Critical |  |
| BusinessTargetID Device Type applicable to this request | Electricity Smart Meter (ESME) |  |
| Can be future dated? | DSP |  |
| On Demand? | Yes |  |
| Capable of being DCC Scheduled? | No |  |
| Command Variants applicable to this Request (Only one populated) | 1 – Send (Non-Critical) 2 – Return for local delivery (Non-Critical) 3 – Send and Return for local delivery (Non-Critical) |  |
| Common Header Data Items | See clause 3.4.1.1 |  |
| Data Items Specific to this Service Request | See Specific Data Items Below |  |
| Possible responses from this Service Request | These are the possible responses applicable to this Service Request. Please see clause 3.5 for more details on processing patterns Acknowledgement Service Response (from Device) – GBCSPayload Response to a Command for Local Delivery Request – LocalCommand Format Also see Response Section below for details specific to this request |  |
| Response Codes possible from this Service Request | See clause 3.5.1.0 for Common Response Codes |  |
| GBCS Cross Reference | Electricity (ALCS and HCALCS ) |  |
| GBCS MessageCode | 0x00BB |  |
| GBCS Use Case | ECS61a |  |

## Changes to Table 35 as follows:

The addition of a new DCC alert.

| DCC Alert Code | Alert Name | Event | Trigger | DCC Alert Recipient |
| --- | --- | --- | --- | --- |
| NXX | ALCS/HCALCS configuration change | ALCS/HCALCS configuration changed on ESME | Upon successful completion of Service Request 6.14.2 Update Device Configuration (Auxilliary Load Control Scheduler) OR Upon successful completion of Service Request 6.14.1 Update Device Configuration (Auxilliary Load Control Descriptions) OR Future Dated Execution Of Instruction Alert (DLMS COSEM) Alert (Alert Code 0x8F66 and Message Code 0x00CC) corresponding to AuxiliaryLoadControlSwitchesCalendar received by the DCC Data Systems | ED |

## Changes to Table 36 as follows

| The addition of a new Response Code Alert Code | Response Code |
| --- | --- |
| NXX | I0 |

## Changes to Table 256 as follows:

The addition of DCC Alert Data Items.

| DCC Alert Format / Data Item | Description / Allowable values | Type | Mandatory for Alert Codes | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ALCSHCALCSConfigurationChange | The trigger event indicates the ESME’s ALCS/HCALCS configuration has changed | sr:ALCSHCALCSConfigChange | Nxx | None | N/A |

## Changes to Section 3.9 as follows:

The addition of a new subsection and table of Data items.

3.9.zz  ALCSHCALCS Configuration Change

3.9.zz.1 Specific Data Items for this DCC Alert

ALCSHCALCS Data Items Definition

| DCC Alert Format / Data Item | Description / Allowable values | Type | Mandatory for Alert Codes | Default | Units |
| --- | --- | --- | --- | --- | --- |
| ESMEDeviceID | The Device ID of the ESME for which the ALCS / HCALCS configuration has changed | sr:EUI (see Section 3.10.1.3 EUI) | Yes | None | N/A |

# Appendix AF ‘Message Mapping Catalogue’

## Changes to Table 158 as follows:

The addition of ‘ECS35g’ and ‘0x00FD’ Electricity Responses and the removal of ‘ECS35f’ and ‘0x00BA’ Electricity Response.

| Data Item | GBCS V2.0 Electricity Response | GBCS vn.0 Electricity Response |
| --- | --- | --- |
| - GBCSHexadecimalMessageCode | 0x00BA | 0x00FD |
| - GBCS Use Case (for reference - not in header) | ECS35F | ECS35g |

## Changes to Table 207 as follows:

| Data Item | Electricity Response (HCALCS or ALCS) |
| --- | --- |
| - GBCSHexadecimalMessageCode | 0x00BB |
| - GBCS Use Case (for reference - not in header) | ECS61a |
| - SupplementaryRemotePartyID | - sr:EUI - (as set out in DUIS Section 3.10 Shared Data Types) - Where originator is Unknown Remote Party or Electricity Distributors (EDs) |
| - SupplementaryRemotePartyCounter | - xs:nonNegativeInteger - Where originator is Unknown Remote Party or Electricity Distributors (EDs) |