DNO investigates permanent solution to the situation. Options include upgrading cable, overlaying cable, replacing transformer, new smart solutions as they become available. Further flexibility tender.

Network solution implemented and Last Resort solution removed within 12 months.

Reporting as per Governance proposal initiated

Last resort solution implemented for a maximum of 12 months

New provider appointed. If no interest in providing services or cost uneconomic then last resort solution implemented

Customers with EV Charging equipment contacted and asked if they wish to participate in the last resort solution to mitigate network interruptions while a permanent solution is investigated and put in place

In parallel customers contacted and advised of the situation

Network Outage Occurs

Flexibility contract remains in place and load continuously monitored by substation equipment

If  load begins to increase flexibility contract renegotiated

Flexibility contract remains in place and load continuously monitored by substation equipment

Tender accepted and flexibility provider appointed

Load on feeders and substations automatically monitored-if load reaches a predetermined level (90%) then flexibility services tender issued

LV Monitoring installed in these substations

Reason for failure investigated-if found it is due to failure of flexibility contract another emergency tender initiated

Analytics and forecasting identify feeders and transformers susceptible to EV  uptake risk