Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 6 October 2017 |
| --- | --- |
| Issue Resolution Proposal | IRP545 |
| Issues Log ID | TS0779 |
| Issue Title | PCS02 Command Issue Date - Time Field |
| Source | IHDL |
| Date Raised | 19 September 2017 |
| Status | Draft v0_2 |
| Documentation References | GBCS V1.0 |

Description:

Does the Command Issue Date/Time field need to be set to 0 in PCS02?

Table 7.2.10d states 'These fields shall have the meaning specified in the ZSE / ZCL specifications' for which 0 means 2000/01/01 00:00:00, not 'now'.

Proposed Position:

Draft 0_2 changes as indicated by the yellow highlight text.

PCS02 uses the ZSE command 'Select Available Emergency Credit' which has a parameter called 'Command Issue Date/Time'.  ZSE states that this parameter is a 'UTCTime field to indicate the date and time at which the selection command was issued'.

Neither ZSE nor GBCS specifies any processing of the value in this parameter and so the GBCS Message Template sets its value to zero (which in ZigBee means midnight at the start of 1st January 2000).  However, the description in the Message Template is incorrect as it states 'Set to 0 to execute immediately' - the field has nothing to do with when the command is executed.

The Message Template for GCS06 (which is the other use of this ZSE command in GBCS) correctly states for this parameter 'Date and time command is issued, set = 0'

Thus, the proposed resolution is to correct the explanatory text in PCS01 to align to that in GCS06.

Related, Table 7.2.10d states that such fields within ZCL Payloads ‘shall have the meaning specified in the ZSE / ZCL specifications’.  ‘0’ is used in these fields in GBCS Messages, and this does not have the ZSE meaning 'UTCTime field to indicate the date and time at which the selection command was issued'’.  Thus, the text is to be amended to recognise that GBCS does override ZigBee meanings in certain cases.

Interoperability:

This change has no normative effect since it does not change the contents of any Message and does not change Message processing requirements.  Thus, it does not affect interoperability and it does not require any system or Device changes.

Required Changes to Documentation:

- In GBCS V2.0 Draft 5, Table 20, ZigBee Commands tab:

- change cell I1187 from 'Set to 0 to execute immediately'  to 'Date and time command is issued, set = 0'.  As shown in the modified Table 20 embedded below.

This results in a changed PCS02 Message Template as below.

- In the last cell of the last row of Table 7.2.10.d:

- add the underlined text as shown by the mark-up text below:

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |