Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 26 October 2017 |
| --- | --- |
| Issue Resolution Proposal | IRP547 |
| Issues Log ID | TS0781 |
| Issue Title | Default Response in GCS21k message 'profile/cluster' flag |
| Source | DCC |
| Date Raised | 20 September 2017 |
| Status | Draft v0_1 |
| Documentation References | GBCS V2.0 |

Description:

It seems that the Default Response in the new GCS21k message has the wrong 'profile/cluster' flag. It's set to being cluster specific but default responses are all profile-wide.

Proposed Position:

As stated in the issue, Default Responses are profile wide and so need to be specified as such.

Interoperability:

This correction affects a new Use Case introduced at GBCS 2.0. Thus, it does not affect interoperability and it does not require any system or Device changes for systems or Devices in live operation today.  However, GSME and GPF being built to comply with GBCS2.0 or later will need to include this change in their solutions.

Required Changes to Documentation:

In GBCS V2.0, Table 20, ZigBee Commands tab, set cells U1632 and U1633 to ‘1’ as shown in the modified Table 20 embedded below.

This results in a changed GCS21k Message Template as below.

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |