Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 10 November 2017 |
| --- | --- |
| Issue Resolution Proposal | IRP549 |
| Issues Log ID | TS0787 |
| Issue Title | ChannelChangePending Clarification |
| Source | Telefonica |
| Date Raised | 27 September 2017 |
| Status | Draft v0_1 |
| Documentation References | GBCS V2.0 Draft 6 |

Description:

Looking at the wording in section 10.2.2.2, you seem to be saying that the GSME shall only act on certain flags in the FunctionalNotificationFlags (line 2378 in 2.0 draft 5)? ChannelChangePending isn't one of them.

Proposed Position:

Additional FunctionalNotificationFlags (Energy Scan Pending and Channel Change Pending) were added to ZigBee and GBCS to support the Sub-GHz functionality.  The requirements on GSME behaviour in relation to these flags are in GBCS sections 10.6.2.8 and 10.6.4 which are to be added in GBCS 2.0.

However, these additional flags should also have been added Table 10.2.2.2b to allow their usage (since that table lists the flags that a GSME is allowed to respond to).

The proposed change is to include these new GBCS 2.0 flags in Table 10.2.2.2b in order to correct the discrepancy, and so allow compliant GSME behaviour.

Interoperability:

There are currently no live Devices or systems which implement GBCS 2.0.  Therefore, the proposed change does not affect interoperability, nor does it affect any live system or Device.

Required Changes to Documentation:

In GBCS Section 10.2.2.2, in Table 10.2.2.2b, add the two rows as shown in the following mark-up:

<…run on from existing text>

| Bit Number | Waiting Command |
| --- | --- |
| 0 | New OTA Firmware |
| 1 | CBKE Update Request |
| 4 | Stay Awake Request HAN |
| 5 | Stay Awake Request WAN |
| 6-8 | Push Historical Metering Data Attribute Set |
| 9-11 | Push Historical Prepayment Data Attribute Set |
| 12 | Push All Static Data - Basic Cluster |
| 13 | Push All Static Data - Metering Cluster |
| 14 | Push All Static Data - Prepayment Cluster |
| 15 | NetworkKeyActive |
| 21 | Tunnel Message Pending |
| 22 | GetSnapshot |
| 23 | GetSampledData |
| 25 | Energy Scan Pending |
| 26 | Channel Change Pending |

Table 10.2.2.2b:  flags in FunctionalNotificationFlags to be actioned by the GSME

<run on as is….>

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | None |