Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 10 November 2017 |
| --- | --- |
| Issue Resolution Proposal | IRP550 |
| Issues Log ID | TS0771 |
| Issue Title | GCS20r - Response when an error occurs |
| Source | Critical Software |
| Date Raised | 30 August 2017 |
| Status | Draft v0_1 |
| Documentation References | GBCS V2.0 Draft 6 |

Description:

What ZigBee command should be sent when an error occurs while executing GCS20r (Read non-critical event and alert behaviours - GSME- Supplier)?

The GBZ template for this use case only includes the component Report Event Configuration which does not support a way of signalling a failed execution. Neither the GBZ template in GBCS not the SEP specific sections for GetEventConfiguration and Report Event Configuration specify that a Default Response should be sent in case of error (although it's possible to argue that the fall-back behaviour in ZCL and therefore in SEP is to send a Default Response in case of error).

Proposed Position:

Errors cannot be signalled via the ZSE Report Event Configuration command which is currently required as the only option in the GCS20r Response. However, errors could occur in Device processing and so the GCS20r Response needs to allow for such situations.

In common with other Use Cases where this situation applies, the proposed change is to allow the GCS20r Response to  contain a ZigBee Default Response (as an Alternate Response) to signal any failures.

Further, a Drafting Note was left in the description of the Event Configuration Payload field, which is to be removed as part of this IRP.

Interoperability:

There are currently no live Devices or systems which implement GBCS 2.0. Therefore, the proposed change does not affect interoperability, nor does it affect any live system or Device.

Required Changes to Documentation:

In GBCS Mapping Table 20, in the ZigBee Commands tab, make the following changes:

- To remove the Drafting Note:

  - Remove the text “[DN: to raise with ZigBee alliance]” from cell I1583.

- To add the default response:

  - Insert the two rows from the embedded spreadsheet at the end of the ‘ZigBee Commands’ sheet in Table 20:

The result is shown in the modified GBCS Table 20 and the revised GCS20r Message Template embedded below.

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) |  |