Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 23 November 2017 |
| --- | --- |
| Issue Resolution Proposal | IRP551 |
| Issues Log ID | TS0807 |
| Issue Title | Confirmation of alerts for DBCH with associated triggers |
| Source | DCC |
| Date Raised | 8 November 2017 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS 2.0 Draft 7 |

Description:

In section 10.6.2.4 we have:

Event / Alert Code Meaning Requirements

Sub GHz Channel not changed after scan

This event occurs when a Sub GHz Channel scan has been undertaken but the CH determines not to change the Sub GHz Channel

However, I can’t seem to find a matching alert, the closest is 0x8F2B (Sub GHz Channel not changed due to Frequency Agility Parameters), can it be confirmed that this is correct?

Proposed Position:

GBCS Table 10.6.2.4 has an event / alert named ‘Sub GHz Channel not changed after scan’ whereas the corresponding Table 16.2 event is named ‘Sub GHz Channel not changed due to Frequency Agility Parameters’.

The name in GBCS Table 10.6.2.4 is to be changed from ‘Sub GHz Channel not changed after scan’ to ‘Sub GHz Channel not changed due to Frequency Agility Parameters’ in line with DUIS 2.0 and Table 16.2.

In aligning to DUIS, this change is to align GBCS drafting to the DCC (DSP) solution being built. Note that this name is unknown to Devices and so has no impact upon Devices.

Interoperability and / or Compatibility:

This IRP is to align GBCS documentation to the DCC solution being built for Release 2.0, and so has no impact on solutions or interoperability.

Required Changes to Documentation:

In GBCS 2.0 Table 10.6.2.4, replace:

‘Sub GHz Channel not changed after scan’ with ‘Sub GHz Channel not changed due to Frequency Agility Parameters’.  As shown in the following markup:

| Event / Alert Code Meaning | Requirements |
| --- | --- |
|  | <…run on from existing text> |
| Sub GHz Channel not changed due to Frequency Agility Parameters | This event occurs when a Sub GHz Channel scan has been undertaken but the CH determines not to change the Sub GHz Channel |
|  | <run on as is…> |

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |