Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 29 March 2018 |
| --- | --- |
| Issue Resolution Proposal | IRP563 |
| Issues Log ID | TS0720 |
| Issue Title | Clarification on IRP477 |
| Source | DCC |
| Date Raised | 21 April 2017 |
| Status | Draft v0_1 |
| Documentation References | GBCS v2.0 |

Description:

Issue in section 13.3.5.10:

1. Specifies the billing data log snapshot cause as 'with a snapshot cause of 0x00020000 (Change of Supplier)' it should be 0x00002000 (as it’s bit 13 for the CoS), NB. this is also incorrect in GBCS 0.8.2

Proposed Position:

GBCS Table 7.4 requires that GSME and GPF support the ZSE PublishSnapshot command. GBCS 10.4.2.4 requires that:

#### Billing Data Log – Tariff TOU Register Matrix, the Consumption Register and Tariff Block Counter Matrix

The GSME shall capture this snapshot at the following trigger points:

…

as specified in Section 13.3.5.10 (snapshot cause ‘Change of Supplier’).

When it next turns on its HAN Interface, the GSME shall push this snapshot to the GPF using the PublishSnapshot Command with a SnapshotPayloadType and a SnapshotSub-Payload populated in line with the requirements of Use Case ‘GCS15b Read GSME Billing Data Log (change of mode / tariff triggered)’.

ZSE table D-52 requires that bit 13 is set for a snapshot cause of ‘Change of Supplier’.  This requirement is reflected in Use Case GCS15b where the only bits that are allowed to be set are bits 14, 13 and 3 reflected in the value 0x00006008 ( Change of Meter Mode | Change of Supplier | Tariff Information).  The P&C software validates, in GCS15b Responses, that the only bits set are 14, 13 or 3.  Therefore, GSME and GPF only use these bits (otherwise they would produce GCS15b Responses that fail P&C validation).

Thus, both ZSE requirements over the HAN and GCS15b requirement mean that GSME must use the value 0x00002000 to represent Change of Supplier.  GPF store the snapshots pushed by the GSME unmodified and so, therefore, use the cause reported by the GSME.

Therefore, the text in section 13.3.5.10 is to be changed to align with wider GBCS and ZSE requirements.

Interoperability:

As previously, wider GBCS and ZSE requirements, along with existing P&C validation, mean that the correct bit 13 is in use by GSME and so GPF.  There will therefore be no interoperability impact of this change.

Required Changes to Documentation:

In GBCS Section 13.3.5.10, change ‘0x00020000’ to ‘0x00002000’.  As shown in the markup below:

…run on from existing text

run on as is…

| Proposed specification version for incorporation | TBC |
| --- | --- |
| Impact on previous RPs | None |
| Attachment(s) | None |

The Issue TS0720 was in two parts, Part 2 will require a separate resolution and is now covered by TS0871.