Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 7 October 2021 |
| --- | --- |
| Issue Resolution Proposal | IRP638 |
| Issues Log ID | TS1395 |
| Issue Title | Clarifications following TS1358 |
| Source | BEIS |
| Date Raised | 23 October 2020 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.0 |

Description:

As explained in TS1358, GBCS section 13.3.5.10 includes a superfluous requirement in relation to the {root, keyCertSign, management} Trust Anchor Cell, and the implications of changing this cell.  This IRP removes this misleading reference.

Proposed Position:

As explained in TS1358, the GBCS 13.3.5.10 requirement in relation to Device processing of CS02b Commands has the condition:

‘the targetTrustAnchorCell is … {root, keyCertSign, management}; and the Entity Identifier in the targetTrustAnchorCell is changed by the replacement’

This condition can never be met, because there is no Entity Identifier in the Trust Anchor Cell (since root has no Entity Identifier).

As such, requirements for actions to take when this condition is met can never occur and are potentially misleading.  This IRP removes the redundant and potentially misleading text.

By way of explanation:

SEC Appendix B, section 6.3.2, requires that the root certificate has a Validity Period of 50 years, and so the root certificate is not ever expected to be changed, to a different root, due to expiry.  However, at the time the current text in GBCS was drafted, that life span had not been agreed, and so provisions were included for root certificate expiry and so roll over, specifically that text that is the subject of this IRP.

Given the very long life, SEC Appendix B, therefore, does not require an Entity Identifier to distinguish one root from another.

Specifically, the certificate profiles in SEC Appendix B, Organisation Certificate Policy, require that SMKI Certificates issued to organisations include the entity's Entity Identifier in the certificate subject field as a uniqueIdentifier object.

Such a requirement is not present for Root and ICA certificates (see the table headed "Root OCA Certificate Profile", and compare this to the table headed "Organisation Certificate Profile", both in Annex B of SEC Appendix B).

GBCS section 12.3 further clarifies this, stating that, as the root certificate is not required to include a Subject Unique ID, Devices must always use the Entity Identifier of 00-00-00-00-00-00-00-00 to refer to the root certificate.

Interoperability and / or Compatibility

As the entity identifier for the root certificate is never permitted to change, the behaviour in this section can never be triggered. As such, there can be no interoperability impact from this change.

Required Changes to Documentation:

In GBCS, at Section 13.3.5.10, amend to text as shown in the mark-up below:

<…run on from, existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | None, (this is a documentation-only change leading to no change in behaviour). |

Requirements applicable to Organisation Certificates only
All Organisation Certificates that are issued by the OCA shall:

- within the subject field, in addition to other attributes, contain an AttributeTypeAndValue structure whose type shall be id-at-uniqueIdentifier {joint-iso-itu-t(2) ds(5) attributeType(4) uniqueIdentifier(45)} and whose value shall be the 8 octet Entity Identifier of the subject of the Certificate;

Requirements applicable to the Root OCA and Issuing OCA

All OCA Certificates issued by the OCA shall:

- have globally unique subject field contents;

- contain a single public key except for the Root-CA where there shall be two public keys. The second public key shall be referred to as the Contingency Key and shall be present in the WrappedApexContingencyKey extension with the meaning of IETF RFC5934. The Contingency Key shall be encrypted as per the requirements of the GBCS;

Note that Certificates where RemotePartyRole is root do not have to contain a Subject Unique ID. Thus, the Device will not have a value provided to it for this field in the {root, keyCertSign, management} Trust Anchor Cell. For consistency, Devices shall record a value of 0x0000000000000000 for Subject Unique ID in the {root, keyCertSign,

management} Trust Anchor Cell