Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 23 February 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP643 |
| Issues Log ID | TS1434 |
| Issue Title | Snapshots for multiple GSME actions |
| Source | BEIS |
| Date Raised | 22 February 2021 |
| Status | Draft v0_2 |
| Documentation Reference | GBCS v4.1 |

IRP643 Draft v0_1 was published in error on 16 December 2021.  This version is the correct version.

Description:

The Technical Specifications require that GSME undertake a number of actions at the same time, e.g. at midnight.

If these actions include the taking of a snapshot and also steps that may update a value in that snapshot, then the order the GSME carries them out is important, in terms of the accuracy of data on the GPF, and so the accuracy of data provided to Consumers / DCC Users.

For example, and as described in the explanation to TS1397, the following two actions are required of the GSME at midnight:

1.  making any time based debt recovery from each Debt Register, so updating that register and requiring that the corresponding ZigBee attribute be pushed to the GPF; and

2.  taking the Prepayment Daily Read Log snapshot (which contains the current Debt Register values) and pushing that to the GPF.

If the snapshot is taken first, its value will not reflect the debt recovery made.  This, as explained in TS1404, may cause the GPF to record either the (inaccurate) value from the snapshot or the (accurate) value of the attribute – the GPF cannot reliably tell which is the accurate one.

Proposed Position:

The proposed position is to add text to GBCS to recommend that GSME undertakes actions that might affect snapshot or attribute values before pushing those attributes / snapshots to the GPF, unless there is an explicit requirement to the contrary.

Interoperability and / or Compatibility

There is no interoperability impact from this change since it relates to internal GSME behaviour.  Note that this change is a recommendation so does not require that existing GSME change their behaviour.

Required Changes to Documentation:

Add the following new Section to GBCS:

<…run on from existing text>

<run on as is…>

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) | None |
| Affected Parties | Suppliers may wish to be aware of the potential differences in GSME behaviour, which may mean that the GPF and GSME may periodically have different values for Operational Data. |