Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 16 December 2021 |
| --- | --- |
| Issue Resolution Proposal | IRP645 |
| Issues Log ID | TS1438 |
| Issue Title | Clarity on Event/Alert Code information |
| Source | Secure |
| Date Raised | 2 March 2021 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.0 |

Description:

With reference to Section 16.2 <Event and alerts excel sheet> attached in SEC-Schedule-8-GB-Companion-Specification-v4.0.docx there is observation that for below Event/Alert Codes the value mentioned in “ESME/GSME Alert Always ON” column seems to be incorrect and its correct value should be “Y”

(Note: “Y” means alerts/events cannot be configured and “N” means can be configured) .

| Event / Alert Code | Event / Alert Code Meaning | Alert WAN (Alert Type) | Alert HAN | Security Log | Event Log (Boost Function Event Log or Sub GHz Channel Log where specified) | Power Event Log | Auxiliary Controller Event Log | Alert Critical (C) / non-critical (NC) | Alert Message Category | Alert contains Sensitive Data | ESME / GSME Alert Always On |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0x8F70 | Update Security Credentials | N | N | Y | N | N | N | N/A | N/A | N | N |  |
| 0x8F82 | Remaining Battery Capacity reset | Y (1) | N | Y | N | N | N | NC | SME.A.NC | N | N |  |
| 0x8F85 | ECS100 Command not supported by Device | Y | N | N | N | N | N | See GBCS 7.2.9.1 | See GBCS 7.2.9.1 | N | N |  |
| 0x8F86 | ECS101 Limit APC [n] Level Command processed | Y | N | N | N | N | N | See GBCS 7.2.9.1 | See GBCS 7.2.9.1 | N | N |  |
| 0x8F87 | ECS102 Limit APC [n] Level ended or cancelled | Y | N | N | N | N | N | See GBCS 7.2.9.1 | See GBCS 7.2.9.1 | N | N |  |
| 0x8F88 | ECS200 Operational Update | Y | N | N | N | N | N | See GBCS 7.2.9.1 | See GBCS 7.2.9.1 | N | N |  |

Reason behind raising this point is that it is contradicting the statement mentioned in SEC-Schedule-8-GB-Companion-Specification-v4.0.docx (Snapshot attached).

Proposed Position:

We agree with the issue raised in that the Alerts identified should not be able to be configured.  Upon investigation we believe there are more instances where the indication is incorrect.  The proposal is, therefore, to remove the contents of the entire column in the table, leaving the title and amending the excel note in the title to say, ‘not used’.  This has no affect other than to remove a degree of confusion, the intended purpose and any value from the column has since been lost to the past.

Interoperability and / or Compatibility

This is a clarification change and as such has no interoperability impact between Systems and Devices and does not affect the format or content of any Messages.

Required Changes to Documentation:

In the GBCS, Table 16.2:

- clear the contents of the column ‘O’ from cell ‘O3’ to the bottom on the sheet;

- at cell ‘O1’ replace the Excel cell comment with ‘Not used’.

The resulting Table 16.2, as modified in embedded at the table below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | Device manufacturers and suppliers to note |

Note: column referred to is GBCS, Table 16.2, ‘Column O’. This has been shaded for this IRP to highlight that column.