Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 6 April 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP652 |
| Issues Log ID | TS1523 |
| Issue Title | Aligning Alerts Table Configuration Responsibility |
| Source | BEIS |
| Date Raised | 1 April 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.1 |

Description:

This IRP corrects anomalies in the GBCS Alerts Table 16.2 relating to the configuration responsibility.

The anomalies have arisen as a result of incomplete addition of new Alerts through SEC Modification MP090.

Proposed Position:

The query raised at TS1507 posed the question:

‘Could BEIS advice how a DCC User knows from GBCS table 16.2, which alert is configurable by them? Originally DCC User could look at column O (IRP645 proposes to remove column O), and anything marked as N is configurable by them (as they are supported by DUIS schema).’

The following explanation was provided:

‘The DCC User may use the spreadsheet filters associated with column 'Q' of GBCS, Table 16.2.  In so doing, it is possible to establish whether a given Alert / Event is configurable by: Supplier; Network Operator; or whether it is 'Not possible' to configure the Alert / Event.

The IRP652 will address an existing anomaly in relation to Alert Codes: 0x81C7, 0x81C8, 0x81C9, 0x81CA, 0x81CB, 0x81CC, 0x81CD, 0x81CE, 0x81CF, 0x81D0 and 0x81D1.  Each of those should be marked 'Not possible' in Column 'Q' as they are currently not able to be configured..’

This IRP652 will now address the anomaly for those cells in GBCS, Table 16.2.

Further, we also propose to take the opportunity of improving the excel ‘notes’ text in the heading for columns ‘F’, ‘Q’ and ‘AB’ at Table 16.2.

For column ‘F’ (Alert WAN (Alert Type)), the note will say:

‘Y means an Alert is to be sent via the WAN to a Remote Party

the type in brackets is as defined in GBCS Section 16.1.1.

Also, values for Codes beginning 0x80 or 0x81 relate to those

configured at or prior to installation, see GBCS Section 16.4’

For column ‘Q’ (ESME / GSME Event/Alert Configuration Responsibility), the note will say:

‘Configuration of an Event / Alert marked as “Not Possible” may

be for the following reasons: (1) it is a critical, or (2) DCC does

not support configuration (some non-critical/non-mandated

Alerts fall into this category)’

For column ‘AB’ (SAPC Support) the note will say:

‘“For detail see GBCS Section 16.1.1”

Interoperability and / or Compatibility

There is no interoperability nor compatibility impact from this IRP.

Required Changes to Documentation:

At GBCS, Table 16.2:

- for Cells Q341:Q351 set the value to be ‘Not possible’;

- at Cell F1, replace the Excel ‘note’ with the following:

Y means an Alert is to be sent via the WAN to a Remote Party the type in brackets is as defined in GBCS Section 16.1.1.

Also, values for Codes beginning 0x80 or 0x81 relate to those at configured prior to installation see GBCS Section 16.4

- at Cell Q1, replace the Excel ‘note’ with the following:

Configuration of an Event / Alert marked as “Not Possible” may be for the following reasons: (1) it is a critical, or (2) DCC does not support configuration (some non-critical/non-mandated Alerts fall into this category)

- at Cell AB1, add the following text as an Excel ‘note’:

‘“For detail see GBCS Section 16.1.1”.

This results in the revised GBCS, Table 16.2 embedded in the table below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | DCC Service Users for info only |

https://smartenergycodecompany.co.uk/modifications/incorporation-of-non-gbcs-non-mandated-alerts-into-the-sec/