Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 6 April 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP653 |
| Issues Log ID | TS1509 |
| Issue Title | Clarify CCS01 UC text to more clearly state intent |
| Source | BEIS |
| Date Raised | 20 January 2022 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.1 |

Description:

As a result of analysis associated with TS1497, we identified a need to clarify the CCS01 Use Case text.

By way of some background.  The intent of the Proposed Position section text in the body of ‘IRP510 CH behaviour on 2nd CCS01’ had not been clearly represented in the Use Case text.

All three CHs have since been found to behave in the same way, if presented with two CCS01 / SRV8.11 joins consecutively, and without there being a CCS02 / SRV8.11 leave between.  We agreed to propose a zero-cost / document only change, so that the GBCS CCS01 Use Case text reflects the actual CH behaviour, which also clarifies the original intent.

Proposed Position:

As per the Issue Description the GBCS CCS01 Use Case text is to be modified to add the following clarifying text in the situation where a CH receives a CCS01 Command for a Device whose Device ID is already in its CHF Device Log:

‘The CHF shall allow SMHAN joining based on the TC Link Key already held by the CH for that Device ID. For clarity, the TC Link Key already held by the CH for that Device ID will depend on the progress of any prior CCS01 command and may be a historic TC Link Key in the CHF Historic Device Log, a pre-configured TC Link Key based on the install code from the previous CCS01 Command or an established TC Link Key due to a previous successful join.’; and

additional text to clarify the situation if a further CCS01 Command is received, as follows:

‘If a CH receives a CCS01 Command for a Device whose Device ID is not in its CHF Device Log,’.

Interoperability and / or Compatibility

There is no interoperability impact from this IRP, it is purely a clarificatory text change.

Required Changes to Documentation:

In GBCS Table 20, Use Case reference tab, at cell AE3 make the changes as shown in the mark-up below:

<…run on from existing text>

<run on as is…>

This results in the revised GBCS Table 20 and GBCS CCS01 Use Case, as embedded in the table below.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | CH manufactures for info only |