Smart Metering Implementation Programme

Technical Specification Issue Resolution Proposal

This note provides BEIS’s interim position on an issue identified with the current version of the Technical Specifications or their associated content.  Please note this position should be considered as Draft until ratified through TBDG.

| Date | 6 April 2022 |
| --- | --- |
| Issue Resolution Proposal | IRP654 |
| Issues Log ID | TS1488 |
| Issue Title | Query Related to attributes of Twin element meter |
| Source | SECURE |
| Date Raised | 17 August 2021 |
| Status | Draft v0_1 |
| Documentation Reference | GBCS v4.1 |

Description:

As identified in the explanations provided to the questions raised at TS1488 in the ‘TS1488 - Query Related to attributes of Twin element meter.docx’ document embedded in this IRP, we propose to make corrections to GBCS Table 7.4.

Proposed Position:

GBCS Table 7.4 to be modified so as to be as proposed by red text in the table representation below:

Table 7.4 needs to appear as follows:

| Row Index | SMETS Device : ZigBee Smart Energy Device (Label) | Attribute(s) | Notes - informative | GBCS/SMETS/CHTS ref - informative (SMETS reference if not explicitly stated) |
| --- | --- | --- | --- | --- |
| 186 | ESME : Energy Services Interface (Twin ESME aggregate ESI Endpoint) | InstantaneousDemand | Active Power Import (SMETS) | 5.13.2.1 |
| 194 | ESME : Energy Services Interface (Twin ESME aggregate ESI Endpoint) | CurrentSummationDelivered | Sum of Active Import Register and Secondary Active Import Register (SMETS) | [INFO] |
| 270 | ESME : Energy Services Interface (Twin ESME primary/secondary ESI Endpoint) | CurrentSummationDelivered | Active Import Register (SMETS) | 5.7.5.3 |
| 271 | ESME : Energy Services Interface (Twin ESME primary/secondary ESI Endpoint) | CurrentSummationDelivered | Secondary Active Import Register (SMETS) | 5.13.2.11 |

Interoperability and / or Compatibility

There is no interoperability nor compatibility impact from this clarification IRP.

Required Changes to Documentation:

At GBCS, Table 7.4:

- set Cells: I186 to ‘5.13.2.1’, I194 to ‘[INFO]’, I270 to ‘5.7.5.3’ and I271 to ‘5.13.2.11;

- set the text at Cell H194 to be ‘Sum of Active Import Register and Secondary Active Import Register (SMETS)’;

- set the text at Cell H270 to be ‘Active Import Register (SMETS)’; and

- set the text at Cell H271 to be ‘Secondary Active Import Register (SMETS)’.

This results in the revised GBCS, Table 7.4 embedded in the table below as ‘IRP654 Table 7.4.xlsx’.

| TS version for incorporation | TBC |
| --- | --- |
| Impact on previous IRPs | None |
| Attachment(s) |  |
| Affected Parties | Twin Element meter manufactures for info |